3GPP TSG-RAN WG4 Meeting # 110bis	R4-2405396

Changsha, China, April 15th – 19th, 2024

Agenda Item:	6.9.2

Source: 		Qualcomm Incorporated

Title: 	Discussion on Rel-18 TRP and TRS requirements

Document for:	Approval

# Introduction

In RAN4#110bis meeting, the WF for Rel-18 TRP and TRS was approved in [1] and the following agreements were made for TRP and TRS measurement campaign and performance requirements.

Issue 2-3-2: Confirm TRP TRS measurement campaign Agreement: - Confirm the updated performance measurement campaign arrangements. - Browsing mode measurement campaign and requirements: measurement campaign started after RAN4#110 meeting. - Talk mode measurement campaign and requirements: measurement campaign started after RAN4#110 meeting. - Collecting measurement results from aligned test labs in RAN4#110bis meeting, start the discussion of potential requirements. - Conclude final TRP TRS requirements in RAN4#111 meeting, close performance WI. - Further contribution of measurement results to RAN4 data pool after RAN4#110bis is allowed.

Therefore, RAN4 will start the discussion of potential requirements from RAN4#110bis and conclude the performance requirements in RAN4#111 meeting. In this paper, we provide our view on Rel-18 TRP and TRS requirements.

# 2. Discussion

As captured in working procedure for Rel-18 measurement campaign to define requirements [2], the TRP and TRS requirements is specified based on the following guidelines:

- Specify TRP TRS requirements: - Only the results from aligned labs will be considered for specifying requirements - Requirements will not be specified if the following thresholds [further check in RAN4#109] are not satisfied by the devices pool: - Minimum number of devices for each band, each device size, each power class: [40] - Minimum number of device models: [25~30] - Minimum number of devices' vendors: 5 - Percentage of devices from [second-half 2021 to 2024]: TBD%: - Percentage of the devices that are certified by PTCRB/GCF/NAL-CTA/CE/FCC: TBD% - Percentage of devices that are commercially available: [100] % - Performance part of the work will proceed in a contribution-driven manner. Start with one type of device width requirement which is most efficient to collect enough results in Rel-18. - Method of limits derivation: per-band Data driven approach - The value at [TBD] percentile of the CDF curve could be selected as the starting point for minimum requirement discussion - For a band supporting both PC2 and PC3, specify PC3 requirement based on finalized PC2 requirements, e.g.,with [X]dB offset

Similar as Rel-17 TRP and TRS requirements discussion, the main two issues of requirements derivation are 1) the threshold percentile at CDF curve which is obtained by measurement results; 2) offset between selected value from CDF curve and final requirements in the spec.

Recalling the process of Rel-17 TRP and TRS requirement derivation, the measurement results were collected in [3]. And the summary of analysis is shown in Table 1.

Table 1: Summary of analysis for TRP TRS performance test campaign [3]

| CDF analysis （dBm） |  |  |  |  |
| --- | --- | --- | --- | --- |
| Percentile | n41 TRP | n78 TRP | n41 TRS | n78 TRS |
| 80%-tile | 12.94 | 12.92 | -82.18 | -83.46 |
| 85%-tile | 12.54 | 12.70 | -81.83 | -83.26 |
| 90%-tile | 12.42 | 12.54 | -81.20 | -82.65 |
| 95%-tile | 11.13 | 11.89 | -80.52 | -81.71 |
| Num of samples | 69 | 50 | 69 | 50 |

The final requirements for Rel-17 TRP and TRS for following bands are captured in TS 38.161

- n41 TRP/TRS:12.5 dBm/-80dBm;

- n78 TRP/TRS:13 dBm/-81.2dBm;

Therefore, the offset between measurement results and final requirements according to percentile can be summarized in Table 2.

Table 1: Summary of offset for Rel-17 TRP TRS performance test campaign and requirement [3]

| Offset (dB) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Percentile | n41 TRP | n78 TRP | n41 TRS | n78 TRS |
| 80%-tile | -0.44 | -0.08 | -2.18 | -2.26 |
| 85%-tile | -0.04 | -0.3 | -1.83 | -2.06 |
| 90%-tile | 0.08 | -0.46 | -1.2 | -1.45 |
| 95%-tile | 1.37 | -1.11 | -0.52 | -0.51 |

Even though there was no agreement on the threshold percentile in Rel-17 TRP and TRS deviation, it could be observed:

- For TRP, the offset to 80% percentile value is the smallest, offset is less than 0.3dB.

- For TRS, the offset to 95% percentile value is the smallest, offset is less than 0.52dB.

With that, RAN4 could select 90% as the threshold percentile and +/-0.5dB as offset to derive Rel-18 TRP and TRS requirement as the starting point.

Proposal: RAN4 to select 90% as the threshold percentile and +/-0.5dB as offset as the starting point to derive Rel-18 TRP and TRS requirement.

# 3.	Conclusion

In this paper, we provide our views on Rel-18 TRP and TRS requirements:

Proposal: RAN4 to select 90% as the threshold percentile and +/-0.5dB as offset as the starting point to derive Rel-18 TRP and TRS requirement.

# 4.	References

- R4-2402875, “WF for [110][331] NR_FR1_TRP_TRS_enh”, vivo

- R4-2316995, “Working procedure for Rel-18 lab alignment and harmonization campaign”, vivo

- R4-2212818, “Analysis of TRP TRS Performance Test Campaign and Proposals for requirements”, vivo