3GPP TSG-RAN WG4 Meeting # 110bis 	R4-2405425

Changsha, China, April 15 – 19, 2024

Source: 	Huawei, HiSilicon

Title: 	LS reply for range 4400 to 4800 MHz

Agenda Item:	9.3.2

Document for:	Approval

1 Introduction

Working Party 5D is seeking information on IMT parameters for the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz [1].

In the last plenary (RAN#103) the Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved [2].

In this contribution we provide IMT parameters and antenna characteristics for frequency band 4400-4800 MHz.

2 Discussion

In the WRC-23 study cycle, 3GPP RAN4 did reply WP5D on IMT parameters in consolidated LS [3] and [4].

[3] R4-2103104, LS to ITU-R on IMT parameters for sharing and compatibility studies in preparation for WRC-23 (6.425 to 10.5 GHz)

[4] R4-2108080, LS to ITU-R and CEPT on extension of IMT array antenna model to support sub-array structures

For the range 4400-4800 MHz, which is part of 3GPP FR1 band n79 and the RF parameters are specified in TS 38.104 for BS and TS 38.101-1 for UE, the IMT parameters are covered in previous LS [3] and [4], i.e. IMT-2020 technology-related and deployment-related parameters and antenna characteristics for small cell in [3], and antenna characteristics for Macro in [4].

Observation 1: for the frequency bands 4400-4800 MHz, we should reuse the IMT parameters agreed in last WRC-23 cycle, no further work is expected.

Compared to [3] and [4], the suggested changes are highlighted as below, with following major points.

- The table for extended AAS model is updated to make it applicable to both sub-array structures and element structures

- The AAS parameters for Small cell outdoor/Micro urban are added using [4] as the basis.

- Some Notes are updated

ANNEX 1

IMT-2020 technology-related and deployment-related parameters for bands between 470 and 4990 MHz

TABLE 1A

IMT-2020 specification related parameters in 470 - 4 990 MHz

|  |  | IMT |  |  |
| --- | --- | --- | --- | --- |
| No. | Parameter | Base station (non-AAS) | Base station (AAS) | Mobile station |
| 1 | Duplex Method | FDD / TDD See [1], § 5.2. (Note X) |  | FDD / TDD See [2], § 5.2. (Note X) |
| 2 | Channel bandwidth (MHz) | See [1], §5.3.5 (Note X) |  | See [2], §5.3.5 (Note X) |
| 3 | Signal bandwidth | Derived from Channel Bandwidth, see [1], § 5.3.2. Signal bandwidth = NRB × SCS × 12. |  | Derived from Channel Bandwidth, see [2], § 5.3.2. Signal bandwidth = NRB × SCS × 12. |
| 4 | Transmitter characteristics |  |  |  |
| 4.1 | Power dynamic range (dB) | Depends on channel bandwidth, See [1], § 6.3.3, Table 6.3.3.2-1. |  | See [2], § 6.2.1 (UE max output power) and § 6.3.1 (UE min output power, depends on channel bandwidth,). |
| 4.2 | Spectral mask (dB) | Category A: See [1], § 6.6.4.2.1 (Wide Area BS), §6.6.4.2.3 (Medium Range BS), §6.6.4.2.4 (Local Area BS). Category B: See [1], § 6.6.4.2.2 (Wide Area BS), §6.6.4.2.3 (Medium Range BS), §6.6.4.2.4 (Local Area BS). | Category A and B: See [1], § 9.7.4.2. (With reference to §6.6.4.2, where the same basic limits apply as for non-AAS BS.) | See [2], § 6.5.2.2, Table 6.5.2.2-1. |
| 4.3 | ACLR | See [1], § 6.6.3.2, Table 6.6.3.2-1. |  | See [2], § 6.5.2.4.1. |
| 4.4 | Spurious emissions | Category A: See [1], § 6.6.5, Table 6.6.5.2.1-1. Category B: See [1], § 6.6.5, Table 6.6.5.2.1-2. |  | See [2], § 6.5.3.1. |
| 4.5 | Maximum/typical output power | See [1], § 6.2, Table 6.2.1-1. (Note X) | See Item No. 1.9 in Table 2 for typical values | See [2], § 6.2.1, Table 6.2.1-1. (Note X) |
| 5 | Receiver characteristics |  |  |  |
| 5.1 | Noise figure (dB) | 5 dB (Macro cell scenario) 10 dB (Micro cell scenario) 13 dB (Indoor small cell scenario) (Note Y) |  | 9 dB |
| 5.2 | Sensitivity (dBm) | Depends on channel bandwidth and BS class, see [1], § 7.2.2. | Depends on channel bandwidth and BS class, see [1], § 10.3.2. | Depends on operating band, see [2], § 7.3.2, Table 7.3.2-1. |
| 5.3 | Blocking response | See [1], § 7.5.2, Table 7.5.2-1 and § 7.4.2, Tables 7.4.2.2-1, 7.4.2.2-2 and 7.4.2.2-3. | See [1], § 10.6.2, Table 10.6.2.1-1 and § 10.5.2, Tables Table 10.5.2.2-1, 10.5.2.2-2 and 10.5.2.2-3. | Depends on operating band, see [2], § 7.6, Tables 7.6.2-2 and 7.6.2-4, 7.6.3-2 and 7.6.3-4 for blocking levels and § 7.7, Table 7.7-2 for spurious response. |
| 5.4 | ACS | See [1], § 7.4.1.2. | See [1], § 10.5.1.2. | See [2], § 7.5, Table 7.5-1 and 7.5-2. |
| 5.5 | SINR operating range (dB) | See below “SINR operating range and mapping function” |  |  |
| References used in the Table: [1] 3GPP TS 38.104 v.18.4.0, “NR; Base Station (BS) radio transmission and reception”. [2] 3GPP TS 38.101-1 v.18.4.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”. Note X: Typical values of duplex method, channel bandwidth and max output power for both non-AAS and AAS IMT stations in different frequency bands are provided in Section 3.1.2.1. Note Y: Wide Area Base Stations are characterised by requirements derived from Macro Cell scenarios, Medium Range Base Stations are characterised by requirements derived from Micro Cell scenarios and Local Area Base Stations are characterised by requirements derived from Pico Cell scenario, see [1], § 4.4. |  |  |  |  |

SINR operating range and mapping function

The following equations approximate the throughput over a channel with a given SINR, when using link adaptation:

Throughput SINR, bps/Hz =0                                         for SINR<SINRMIN                         ∝∙SSINR                     for SINRMIN≤SINR<SINRMAX ∝∙SSINRMAX               for SINR ≥ SINRMAX

Where:

S(SINR)	Shannon bound, S(SINR) =log2(1+SINR) [bps/Hz]
		Attenuation factor, representing implementation losses
SINRMIN  	Minimum SINR of the code set, dB
SINRMAX 	Maximum SINR of the code set, dB

The parameters α, SINRMIN and SINRMAX can be chosen to represent different modem implementations and link conditions. The parameters proposed in table 2 represent a baseline case, which assumes:

- 1:1 antenna configurations

- AWGN channel model

- Link Adaptation (see table 2 for details of the highest and lowest rate codes)

- No HARQ

Table 1B: Parameters describing baseline Link Level performance for 5G NR

| Parameter | DL | UL | Notes |
| --- | --- | --- | --- |
| α | 0.6 | 0.4 | Represents implementation losses |
| SINRMIN, dB | -10 | -10 | Based on QPSK, 1/8 rate (DL) & 1/5 rate (UL) |
| SINRMAX, dB | 30 | 22 | Based on 256QAM 0.93(DL) & 64QAM 0.93 (UL) |

ANNEX 2

Antenna characteristics for IMT-2020 AAS base stations 
for bands between 1710 and 4990 MHz

Table 1-1: Extended AAS model

| Description | Equation |
| --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ |
| Sub-array excitation (Note 1) | wm=1Msubexpj2πm-1dv,subλsinθsubtilt |
| Sub-array radiation pattern (Note 1) | Asubθ,φ=AEθ,φ+10log10m=1Msubwmvm2 , where vm=expj2πm-1dv,subλcosθ |
| Array excitation | wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan Where M and N is corresponding to (Row × Column) in Table 1-2, row 1.6. |
| Composite array radiation pattern | For sub-array structures: AAθ,φ=Asubθ,φ+10log10m=1Mn=1Nwm,nvm,n2 For element structures: AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ Where M and N is corresponding to (Row × Column) in Table 1-2, row 1.6. |
| Note 1: it is only applicable for sub-array structures |  |

Table 1-2: Beamforming antenna characteristics supporting subarray structures for IMT in 1710 – 4990 MHz

|  |  | Macro Rural | Macro suburban | Macro urban | Small cell outdoor/ Micro urban |
| --- | --- | --- | --- | --- | --- |
| 1 | Base station Antenna Characteristics |  |  |  |  |
| 1.1 | Antenna pattern | Refer to Table 1-1 |  |  |  |
| 1.2 | Element gain (dBi) (Note 2) | 6.4 | 6.4 | 6.4 | 6.4 |
| 1.3 | Horizontal/vertical 3 dB beam width of single element (degree) | 90º for H 65º for V | 90º for H 65º for V | 90º for H 65º for V | 90º for H 65º for V |
| 1.4 | Horizontal/vertical front-to-back ratio (dB) | 30 for both H/V | 30 for both H/V | 30 for both H/V | 30 for both H/V |
| 1.5 | Antenna polarization | Linear ±45º | Linear ±45º | Linear ±45º | Linear ±45º |
| 1.6 | Antenna array/sub-array configuration (Row × Column) (Note 4) | 4 × 8 sub-arrays | 4 × 8 sub-arrays | 4 × 8 sub-arrays | 8 × 8 elements |
| 1.7 | Horizontal/Vertical radiating array/sub-array spacing | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 2.1 of wavelength for V | 0.5 of wavelength for H, 0.7 of wavelength for V |
| 1.7a | Number of element rows in sub-array (Note 6) | 3 | 3 | 3 | N/A |
| 1.7b | Vertical element separation in sub-array (dv,sub) (Note 6) | 0.7 of wavelength of V | 0.7 of wavelength of V | 0.7 of wavelength of V | N/A |
| 1.7c | Pre-set sub-array down-tilt (degrees) (Note 6) | 3 | 3 | 3 | N/A |
| 1.8 | Array Ohmic loss (dB) (Note 2) | 2 | 2 | 2 | 2 |
| 1.9 | Conducted power (before Ohmic loss) per sub-array (dBm) (Note 3) | 28 | 28 | 28 | 16 |
| 1.10 | Base station horizontal coverage range (degrees) | +/-60 | +/-60 | +/-60 | +/-60 |
| 1.11 | Base station vertical coverage range (degrees) (Note 1) | 90-100 | 90-100 | 90-100 | 90-120 |
| 1.12 | Mechanical down-tilt (degrees) | 3 | 6 | 6 | 10 |

Note 1:	The vertical coverage range is given for the elevation angle θ, defined between 0° and 180° as 
in ITU-R M.2101.

Note 2:	The element gain in row 1.2 includes the loss given in row 1.8 and is per polarization.

Note 3:	The conducted power is per sub-array assumes 4x8x2 sub-arrays (i.e. power per H/V polarized element).

Note 4:	4 × 8 sub-arrays means there are 4 vertical and 8 horizontal radiating sub-arrays. 8 × 8 elements means there are 8 vertical and 8 horizontal radiating elements.

Note 5: For the case of 3 elements per sub array, dv will be 2.1 wavelengths.

Note 6: 1.7a, 1.7b and 1.7c are only applicable for sub-array structures

Proposal 1: It is proposed to reply WP5D with IMT parameters and antenna characteristics for frequency band 4400-4800 MHz as above.

3	Conclusion

In this contribution, we provide text proposal for reply LS for frequency band 4400-4800 MHz.

Observation 1: for the frequency bands 4400-4800 MHz, we should reuse the IMT parameters agreed in last WRC-23 cycle, no further work is expected.

Proposal 1: It is proposed to reply WP5D with IMT parameters and antenna characteristics for frequency band 4400-4800 MHz as listed in clause 2.

References

[1] R4-2400333, “Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz” ITU-R WP 5D

[2] RP-240787,	New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz, 	Ericsson

[3] R4-2103104, LS to ITU-R on IMT parameters for sharing and compatibility studies in preparation for WRC-23 (6.425 to 10.5 GHz)

[4] R4-2108080, LS to ITU-R and CEPT on extension of IMT array antenna model to support sub-array structures

[5] R4-220731, CR to TR 38.803: Addition of array antenna model extension in sub-clause 5.2.3