3GPP TSG-RAN WG4 Meeting #110bis 	R4-2405788

Changsha, China, April 15 – 19, 2024

Source:	Nokia

Title:	Simulation Results on HST FR2 Enhanced with Multi-Rx Chain DL Reception

Agenda item:	6.7.3.3

Document for:	Information

Introduction

In RAN4#110, we have provided our simulation results in [1]. Nonetheless, after discussions with the other companies, it was noticed that there was a different assumption used in the simulation. In [1], the power difference was considered in the simulation, which was not the case with the other companies’ simulation.

In RAN4#108bis and RAN4#109, in fact, we have provided simulation results without power difference as documented in [2] and [3], respectively. Hence, during RAN4#110 meeting, we first updated our results via email by reusing previous contribution in RAN4#108bis [2] and RAN4#109 [3]. As in [2] and [3], there were only simulation results that can be considered for MCS pair of (19,13), we run another simulation for MCS pair (19,11), which results were then also shared by email on the fourth day of RAN4#110 meeting.

The updated simulation results themselves have been included in defining the SNR requirements for HST FR2 multi-Rx together with the simulation results from the other companies. The final SNR requirements have been captured in the draft big CR [4].

In this paper, for completeness of information and for references, the updated simulation results which were only shared via email in RAN4#110, are now provided and documented in a formal document.

Simulation Results

The updated simulation results are presented in Table 1 and Table 2 below for the case of with and without impairments, respectively:

Table 1. Simulation Results HST FR2 multi-Rx – Ideal

| Bandwidth (MHz) | Subcarrier Spacing (KHz) | Carrier Frequency (GHz) | MCS Pair (MCS1, MCS2) | Rank per TRP | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Throughput (%) | SNR Pair (SNR1, SNR2) (dB) |
| 200 | 120 | 30 | (19,11) | 2 | 2x2 Low | 70 | (12.6, 5.5) |
| 200 | 120 | 30 | (19,13) | 2 | 2x2 Low | 70 | (12.6, 7.5) |

Table 2. Simulation Results HST FR2 Multi-Rx – Impairments

| Bandwidth (MHz) | Subcarrier Spacing (KHz) | Carrier Frequency (GHz) | MCS Pair (MCS1, MCS2) | Rank per TRP | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Throughput (%) | SNR Pair (SNR1, SNR2) (dB) |
| 200 | 120 | 30 | (19,11) | 2 | 2x2 Low | 70 | (15.1, 8) |
| 200 | 120 | 30 | (19,13) | 2 | 2x2 Low | 70 | (15.1, 10) |

Summary

In this paper, we provide the updated simulation results previously shared via email during RAN4#110 meeting. The simulation results themselves, together with the results from the other companies, have been used to define the SNR requirements of Rel-18 HST FR2 with multi-Rx demodulation performance as already captured in the draft big CR.

- References

- R4-2402718, Simulation Results on HST FR2 Enhanced with Multi-Rx Chain DL Reception, Nokia, Nokia Shanghai Bell, RAN4 #110, Athens, Greece, February 26 – March 01, 2024.

- R4-2316636, Simulation Results on HST FR2 Enhanced with Multi-Rx Chain DL Reception, Nokia, Nokia Shanghai Bell, RAN4#108bis, Xiamen, China, October 09 – 13, 2023

- R4-2320582, Simulation Results on HST FR2 Enhanced with Multi-Rx Chain DL Reception, Nokia, Nokia Shanghai Bell, RAN4#109, Chicago, USA, November 13 – 17, 2023

- R4-2401572, Draft big CR for TS 38.101-4 on Rel-18 FR2 HST demodulation, Samsung, RAN4 #110, Athens, Greece, February 26 – March 01, 2024