3GPP TSG-RAN WG4 Meeting #110bis              	R4- 2405868

Changsha, China, 14th -19th April, 2024

Agenda item:	6.19.4.2

Source:	Samsung

Title:	Simulation results on BS demodulation requirements for Rel-18 FeMIMO

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the test setup of BS demodulation performance part for FeMIMO was under discussion. The related agreement was captured in the WF [1] as following

- Scope of BS demodulation performance requirements - Introduce applicability rule for UL Rel-18 DMRS port, but the details - clarify the details of applicability rule for Rel-18 DMRS ports - Unless otherwise stated, PUSCH requirements with enhanced DM-RS port configuration shall apply only for a BS declaring support of enhanced DM-RS port type (see D.xxx in table 4.6-1) - [a BS that passes tests with enhanced DM-RS port can consider corresponding legacy PUSCH tests as passed. Definition of “corresponding” needs to be further specified]. FFS on specific wording - clarify if BS demodulation requirements are needed for FR2 STxMP - Do not define FR2 STxMP demodulation requirements in Rel-18, postpone the discussion on BS performance introduction with UE FR2 STxMP to future release. - Test setup and simulation assumptions for Rel-18 DMRS - DMRS ports - Rank 1 for 1Tx: {8} - Rank 2 for 2Tx: {8, 9} - CBW - 15KHz SCS, 5MHz - 30KHz SCS, 10MHz - MCS - MCS 16

In this contribution, the initial simulation results for Rel-18 DMRS port enhancement are provided for alignment purpose.

# 2	Simulation Results

In this section, the initial simulation results are provided based on agreed simulation assumption as following

Table 2-1: simulation assumption for PUSCH with Rel-18 DMRS port

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {8}, {8,9} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
|  | DM-RS type | enhanced-dmrs-Type_r18 |
| Time domain | PUSCH mapping type | A and B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| TPMI index for 4Tx four-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

Table 2-2:  initial simulation results for PUSCH with Rel-18 DMRS port

| Test Cases | SCS&BW | RB | MCS | Channel | Mapping Type | Length of symbol | Numb of Tx and Rx | Number of DMRS | SNR @70% with Rel-18 DMRS ports |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15KHz 5MHz | 25 | 16 | TDLC300-100 | A | 14 | 1Tx2Rx | 1+1 (2,11) | 8.7 |
| 2 | 30KHz 10MHz | 24 | 16 | TDLC300-100 | A | 14 | 1Tx2Rx | 1+1 (2,11) | 8.7 |
| 3 | 15KHz 5MHz | 25 | 16 | TDLC300-100 | A | 14 | 2Tx2Rx | 1+1 (2,11) | 16.8 |
| 4 | 30KHz 10MHz | 24 | 16 | TDLC300-100 | A | 14 | 2Tx2Rx | 1+1 (2,11) | 18.7 |
| 5 | 15KHz 5MHz | 25 | 16 | TDLC300-100 | B | 14 | 1Tx2Rx | 1+1 (2,11) | 8.7 |
| 6 | 30KHz 10MHz | 24 | 16 | TDLC300-100 | B | 14 | 1Tx2Rx | 1+1 (2,11) | 8.8 |
| 7 | 15KHz 5MHz | 25 | 16 | TDLC300-100 | B | 14 | 2Tx2Rx | 1+1 (2,11) | 16.8 |
| 8 | 30KHz 10MHz | 24 | 16 | TDLC300-100 | B | 14 | 2Tx2Rx | 1+1 (2,11) | 18.7 |

# 3	Conclusion

In this contribution, the initial simulation results are provided for alignment purpose.

# References

R4-2403061, WF on NR_MIMO_evo_DL_UL_demod