3GPP TSG RAN WG4 Meeting #110bis	R4-2405877

Changsha, China, 15 Apr. 2024  – 19 Apr. 2024

Agenda Item:	6.14.2.1

Source: 	Ericsson

Title:	Discussion on LTM test cases

Document for:	Discussion

# Introduction

In this contribution we provide our views on LTM test cases.​

# Discussion

In last meeting Ran4 agreed to define test cases for

- L1-RSRP accuracy requirements

- Cell switch requirements

- PDCCH-order RACH on neighbour cell

Tentative list of test cases is copied in Annex for reference. In these test cases, RAN4 agreed to consdier early TCI state activation in some of the test cases. For the test cases where early TCI state activation is assumed, we think RAN4 should test the requirements for known and unknown TCI states. We have provided our analysis on why RAN4 should define the unknown TCI state activation requirements in core part maintenance paper. Since defining different test case for unknown TCI state seperately may increase the total number of test cases, we think this can be defined as a sub test in the relevant test cases.

- RAN4 should define test cases for unknown TCI state activation for the test cases involving early TCI state activation.

# Summary and Conclusion

In this contribution we have provided our views on performance part of LTM. The following proposals are made:

- RAN4 should define test cases for unknown TCI state activation for the test cases involving early TCI state activation.

# References

- R4-2317329 “WF for RRM performance requirements of R18 Further NR mobility enhancement”, Apple

# Annex

| Accuracy requirements | Detail | Company |
| --- | --- | --- |
| Intra-frequency L1-RSRP accuracy requirements | 10.1.19X Intra-frequency L1-RSRP accuracy requirements for neighbour cell in FR1 - Absolute Accuracy - Relative Accuracy | CATT |
|  | 10.1.20X Intra-frequency L1-RSRP accuracy requirements for neighbour cell in FR2 - Absolute Accuracy - Relative Accuracy | ZTE |
| Inter-frequency L1-RSRP accuracy requirements | 10.1.19Y Inter-frequency L1-RSRP accuracy requirements in FR1 - Absolute Accuracy - Relative Accuracy | CMCC |
|  | 10.1.20Y Inter-frequency L1-RSRP accuracy requirements in FR2 - Absolute Accuracy - Relative Accuracy | Huawei |

| Core requirements defined | Detail | Company |
| --- | --- | --- |
| PCell Cell switch requirements | A.6.3.x.1 - RACH based Cell switch from FR1 to FR1 - Intra-frequency cell switch - Without early TCI state activation | CATT |
|  | A.6.3.x.2 - RACH based Cell switch from FR1 to FR1 - Intra-frequency cell switch - With early TCI state activation | MTK, Ericsson |
|  | A.6.3.x.3 - RACH-less Cell switch from FR1 to FR1 - Intra-frequency cell switch - Without early TCI state activation | ZTE |
|  | A.6.3.x.4 - RACH-less Cell switch from FR1 to FR1 - Intra-frequency cell switch - With early TCI state activation | vivo |
|  | A.6.3.x.5 - RACH based Cell switch from FR1 to FR1 - Inter-frequency cell switch - Without early TCI state activation | vivo |
|  | A.6.3.x.6 - RACH based Cell switch from FR1 to FR1 - Inter-frequency cell switch - With early TCI state activation | Nokia |
|  | A.7.3.x.1 - RACH based Cell switch from FR2 to FR2 - Intra-frequency cell switch - Without early TCI state activation | Huawei |
|  | A.7.3.x.2 - RACH-less Cell switch from FR2 to FR2 - Intra-frequency cell switch - Without early TCI state activation | Nokia |
|  | A.7.3.x.3 - RACH based Cell switch from FR2 to FR2 - Inter-frequency cell switch - Without early TCI state activation |  |
| PSCell cell switch | A.6.3.y.1 - RACH based Cell switch from FR1 to FR1 - Intra-frequency cell switch - Without early TCI state activation | CATT |
|  | A.6.3.y.2 - RACH based Cell switch from FR1 to FR1 - Intra-frequency cell switch - With early TCI state activation | Nokia |
|  | A.7.3.y.1 - RACH based Cell switch from FR2 to FR2 - Intra-frequency cell switch - Without early TCI state activation |  |
| PDCCH-order RACH on neighbour cell | A.6.5.x.1 - intra-frequency target cell in FR1 - RACH BW is within active BWP | MTK, Ericsson |
|  | A.6.5.x.2 - inter-frequency target cell in FR1 - RACH BW is outside any configured BWP | vivo |
|  | A.7.5.x.1 - intra-frequency target cell in FR2 - RACH BW is within active BWP |  |
|  | A.7.5.x.2 - inter-frequency target cell in FR2 - RACH BW is outside any configured BWP |  |
| Intra-f L1-RSRP measurement for LTM | A.6.6.x.1 - Intra-f L1-RSRP measurement in FR1 - Single frequency layer - 1 serving cell, 2 neighbor cells | MTK, Ericsson |
|  | A.7.6.x.1 - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, none of neighbor cells’ TCI state activated | CATT |
|  | A.7.6.x.2 - Intra-f L1-RSRP measurement in FR2 - Single frequency layer - 1 serving cell, 2 neighbor cells, one of neighbor cells’ TCI state activated | ZTE |
| Inter-f L1-RSRP measurement with MG | A.6.6.y.1 - Inter-f L1-RSRP measurement with MG in FR1 - 2 neighbor cells | Huawei |
|  | A.7.6.y.1 - Inter-f L1-RSRP measurement with MG in FR2 - 2 neighbor cells | Apple |
| Inter-f L1-RSRP measurement without gap for LTM | A.6.6.z.1 - Inter-f L1-RSRP measurement without gap in FR1 - Single frequency layer - 2 neighbor cells | CMCC |
|  | A.7.6.z.1 - Inter-f L1-RSRP measurement without gap in FR2 - Single frequency layer - 2 neighbor cells, none of neighbor cells’ TCI state activated | Apple |

| Test cases | Company |
| --- | --- |
| Inter-frequency L1-RSRP accuracy requirements for neighbour cell in FR1 | Huawei |
| Inter-frequency L1-RSRP accuracy requirements for neighbour cell in FR2 |  |