3GPP TSG-RAN WG4 Meeting #110bis	R4-2406011

# Changsha, China, 15 - 19 Apr, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | Draft | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on Minimum requirements and FRC definition for sDCI SDM (TS38.101-4, Rel-18) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_multiRX_DL-Perf |  |  |  |  |  | Date: |  |  | 2024-04-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Move clause 7.2.2.2.7 to the right place. Remove square brackets for FR2 multi-Rx minimum requirements for sDCI SDM in the new clause 7.2.2.2.7. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Delete clause 7.2.2.2.7 under clause 7.1.1.3 and add new clause 7.2.2.2.7 under clause 7.2.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be inconsist between specification and RAN4 agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.2.2.7 under 7.1.1.3 (Delete), 7.2.2.2.7 under 7.2.2.2 (New clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Delete: 7.2.2.2.7 under 7.1.1.3 New clause: 7.2.2.2.7 under 7.2.2.2 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <START OF THE CHANGE 1>

#### 7.1.1.3	Applicability of requirements for optional UE features

<Unchanged part skipped>

# <END OF THE CHANGE 1>

# <START OF THE CHANGE 2>

#### 7.2.2.2	TDD

<Unchanged part skipped>

##### 7.2.2.2.7	Minimum requirements for PDSCH Single-DCI based SDM scheme

The performance requirements are specified in Table 7.2.2.2.7-3, with the addition of test parameters in Table 7.2.2.2.7-2 and the downlink physical channel setup according to Annex C.5.1.

The test purposes are specified in Table 7.2.2.2.7-1.

Table 7.2.2.2.7-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH performance with Single-DCI based SDM scheme for multi-Rx simultaneous DL reception. | 1-1, 1-2 |

Table 7.2.2.2.7-2: Test Parameters

| Parameter |  |  |  | Unit | Value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| SSB |  |  |  |  | SSB #0 | SSB #1 |
| PDCCH configuration |  | TCI state |  |  | TCI State #2 | N/A |
|  |  | CORESETPoolIndex |  |  | 0 | N/A |
| CSI-RS for tracking |  | First subcarrier index in the PRB used for CSI-RS |  |  | k0=0 for CSI-RS resources 1,2,3,4 | k0=1 for CSI-RS resources 5,6,7,8 |
|  |  | First OFDM symbol in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resources 1 and 3 l0 = 10 for CSI-RS resources 2 and 4 | l0 = 6 for CSI-RS resources 5 and 7 l0 = 10 for CSI-RS resources 6 and 8 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 | 1 for CSI-RS resource 5,6,7,8 |
|  |  | CDM Type |  |  | ‘No CDM’ for CSI-RS resource 1,2,3,4,5,6,7,8 |  |
|  |  | Density |  |  | 3 |  |
|  |  | CSI-RS periodicity |  | Slots | 160 |  |
|  |  | CSI-RS offset |  | Slots | 80 for CSI-RS resources 1 and 2 81 for CSI-RS resources 3 and 4 | 80 for CSI-RS resources 5 and 6 81 for CSI-RS resources 7 and 8 |
|  |  | QCL info |  |  | TCI state #0 | TCI state #1 |
| CSI-RS for beam refinement |  | First subcarrier index in the PRB used for CSI-RS (k0) |  |  | k0=0 for CSI-RS resource 1,2 | k0=1 for CSI-RS resource 3,4 |
|  |  | First OFDM symbol in the PRB used for CSI-RS (l0) |  |  | l0 = 8 for CSI-RS resource 1 l0 = 9 for CSI-RS resource 2 | l0 = 8 for CSI-RS resource 3 l0 = 9 for CSI-RS resource 4 |
|  |  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2 | 1 for CSI-RS resource 3,4 |
|  |  | CDM Type |  |  | 'No CDM' for CSI-RS resource 1,2 | 'No CDM' for CSI-RS resource 3,4 |
|  |  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2 | 3 for CSI-RS resource 3,4 |
|  |  | CSI-RS periodicity |  |  | 160 for CSI-RS resource 1,2 | 160 for CSI-RS resource 3,4 |
|  |  | CSI-RS offset |  |  | 0 for CSI-RS resource 1,2 | 0 for CSI-RS resource 3,4 |
|  |  | Repetition |  |  | ON | ON |
|  |  | QCL info |  |  | TCI state #2 | TCI state #3 |
| Duplex mode |  |  |  |  | TDD |  |
| Active DL BWP index |  |  |  |  | 1 |  |
| PDSCH configuration | Mapping type |  |  |  | Type A |  |
|  | k0 |  |  |  | 0 |  |
|  | Starting symbol (S) |  |  |  | 1 |  |
|  | Length (L) |  |  |  | Specific to each Reference channel as defined in A.3.2.2 |  |
|  | PRB bundling type |  |  |  | Static |  |
|  | PRB bundling size |  |  |  | 2 |  |
|  | Resource allocation type |  |  |  | Type 1 |  |
|  | RBG size |  |  |  | Config2 |  |
|  | VRB-to-PRB mapping type |  |  |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  |  |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  |  |  | 1000 | 1002 |
|  | TCI state |  |  |  | TCI State #2 | TCI State #3 |
|  | DMRS Type |  |  |  | Type 1 |  |
|  | Number of additional DMRS |  |  |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  |  | 1 |  |
| TCI State #0 | Type 1 QCL information |  | SSB index |  | SSB #0 | N/A |
|  |  |  | QCL Type |  | Type C | N/A |
|  | Type 2 QCL information |  | SSB index |  | SSB #0 | N/A |
|  |  |  | QCL Type |  | Type D | N/A |
| TCI State #1 | Type 1 QCL information |  | SSB index |  | N/A | SSB #1 |
|  |  |  | QCL Type |  | N/A | Type C |
|  | Type 2 QCL information |  | SSB index |  | N/A | SSB #1 |
|  |  |  | QCL Type |  | N/A | Type D |
| TCI State #2 | Type 1 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type A | N/A |
|  | Type 2 QCL information |  | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking’ configuration | N/A |
|  |  |  | QCL Type |  | Type D | N/A |
| TCI State #3 | Type 1 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type A |
|  | Type 2 QCL information |  | CSI-RS resource |  | N/A | CSI-RS resource 5 from 'CSI-RS for tracking’ configuration |
|  |  |  | QCL Type |  | N/A | Type D |
| PTRS configuration | Frequency density (KPT-RS) |  |  |  | 2 | 2 for Test 1-2, Disabled for Test 1-1 |
|  | Time density (LPT-RS) |  |  |  | 1 | 1 for Test 1-2, Disabled for Test 1-1 |
|  | Resource Element Offset |  |  |  | 2 | 3 for Test 1-2, N/A for Test 1-1 |
| Resource allocation |  |  |  |  | Full-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  |  |  | us | -0.0625 |  |
| Frequency offset of the second TRxP from the first TRxP |  |  |  | Hz | 600 |  |
| Number of HARQ Processes |  |  |  |  | 8 |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |  |
| Precoding configuration |  |  |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) |  |  |  |  |  |  |

Table 7.2.2.2.7-3: Minimum performance

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.5-16.1 TDD | 100 / 120 | 16QAM, 0.48 | FR1.120-1 | TDLA30-35 | 4x4, FR2- mTRxP-mRX ρ = -12dB | 70 | 15.9 |
| 1-2 | R.PDSCH.5-16.2 TDD | 100 / 120 | 16QAM, 0.48 | FR1.120-1 | TDLA30-35 | 4x4, FR2- mTRxP-mRX ρ = -12dB | 70 | 15.6 |
| Note 1: The propagation conditions apply to each of TRxP #1 and TRxP #2 and are statistically independent Note 2: Correlation matrix according to the RFR2-mTxRP-mRX in B.2.3.3. TRxP#1 uses TX antenna indices (1,2) and TRxP#2 uses TX antenna indices (3,4) corresponding to the respective antenna configuration matrix rows. Note 3: SNR is defined per UE Rx chain. SNR of Rx chain i (i=1,2) is derived based on Es from TRxP#i, as defined in 4.5.2. |  |  |  |  |  |  |  |  |

# <END OF THE CHANGE 2>