3GPP TSG-RAN WG4 Meeting#110bis                              R4-2406609

Changsha, China, 15th – 19th April, 2024

Agenda item:			6.16.5

Source:	ZTE, Samsung

Title:	WF on Rel-18 NTN UE RF requirements

Document for:	Approval

Online agreement

Issue 1-1: Maximum TRP

Online Agreement:

- For type 1 and type 4 with mechanical steering antenna (e.g. parabolic antenna)

  - 35dBm

- For type 2,3, 5 with electronic steering antenna (e.g. phase antenna array)

  - 35dBm, 40dBm or 45dBm for further decision in this meeting.

Offline agreement:

For maximum TRP:

- For type 2,3, 5 with electronic steering antenna (e.g. phase antenna array)

  - [43dBm] for further ACLR evaluation; if there is coexistence issue identified for 43dBm next meeting, then set 40dBm as maximum TRP power;

  - The maximum EIRP 76.2 is not changed, the antenna gain for other TRP values could further adjusted;

Offline agreement:

For the minimum peak EIRP requirement:

- To state in RAN4 spec to declare the supported elevation angles to comply the minimum peak EIRP requirement and leave the test for RAN5 discussion.

Issue 1-2: Minimum output power

WF:

Not to define the transmitter Minimum controlled output power requirement;

Issue 1-3: Transmit ON-OFF power

- Recommended for further discussion:

  - Option 1: reusing FR2 BS OFF power requirement  -36dBm/MHz

Online Agreement:

- Agree on Option 1.

Issue 1-4: Transmit ON-OFF time mask

Online Agreement:

- Transient period for FR2-NTN is 5us.

Issue 1-5:  Transmit freq error

- Recommended Agreement:

  - Follow the agreement reached in R4-232197 with fixed doppler shift.

  - Time varying doppler shift will be discussed in Rel-19 RAN4 NTN WID.

Issue 1-6:  Transmit modulation quality

- Recommended Agreement:

  - Follow the agreement reached in R4-2317648, R4-2321974

Issue 1-7:  SEM

Online Agreement:

- Agree on proposal 4.

  - FFS on definition of P_(rated,UE)

Offline agreement for  NS indication for regional requirements

Step 1: to make the regulatory requirement clear and applicable to which bands;

Step 2: to further discuss how to arrange the regulatory requirement in the specification next meeting;

Issue 1-8:  Power control requirement

Online Agreement:

- No requirement for power control for Ka band for NR-NTN UE

Issue 1-9:  MPR requirement for type 2 and type 5 to meet OFF-axis EIRP requirement

Offline agreement:

For type 2 an type 5,  the power reduction for OFF-axis EIRP requirement might be needed and the exact value is left for the implementation;

Issue 2-1  Minimum EIS requirement

Online Agreement:

- for type 3 UE, to specify minimum EIS as -115.6dBm for 50MHz, for the other channel bandwidth, the corresponding EIS requirement could be scaled with PRB based compared with 50MHz;

- for type 1/2/4/5 UE, to specify minimum EIS as -122dBm for 50MHz, for the other channel bandwidth, the corresponding EIS requirement could be scaled with PRB based compared with 50MHz

Issue 2-2: Maximum input power

Online Agreement:

- For type 3,

  - [-101]dBm as maximum input power with 64QAM

- For type 1/2/4/5:

  - -109.6dBm with 16QAM or [64QAM].

  - FFS for QPSK

- The exact MCS or coding rate for FRC of maximum input power need further discussion and confirmation.

Issue 2-5: Polarization characteristics

Online Agreement:

- Either RHCP or LHCP is supported by Ka band VSAT in Rel-18

Open for further discussion

Issue 1-11: Antenna performance

- Proposal 1: Specify Antenna performance requirements for band n511 and n510 based on FCC 47 CFR 25.209. [Ericsson, R4-2404939]

- Proposal 2: Antenna point accuracy should not be introduced into the spec until RAN4 fully discuss such requirements in detail.  [Huawei, R4-2405339]
Recommended WF:

  - Further discuss the proposal 1

  - To clarify anything is not clear for antenna point accuracy.

Issue 1-7:  SEM

Online Agreement:

- Agree on proposal 4.

  - FFS on definition of P_(rated,UE)

  - FFS for the boundary for SEM and spurious emission requirements.

Issue 1-7:  SEM

Online Agreement:

- Agree on proposal 4.

  - FFS on definition of P_(rated,UE)

  - FFS for the boundary for SEM and spurious emission requirements.

Issue 2-2: Maximum input power

- FFS on Whether the maximum input power should be aligned with ACS interfering signal power level;

Issue 2-3: ACS

- FFS on the test parameters for ACS requirement

Issue 2-4: Antenna off-axis performance

- Proposal 1: Specify Rx antenna off-axis performance requirements for band n512 based on ETSI EN 301 360, EN 301 459, EN 303 699, EN 303 978. [Ericsson, Thales, R4-2404940]

- Recommended WF:

  - Need further discussions online.