3GPP TSG-RAN WG4 Meeting #110bis	R4-2406610

Changsha, China, April 15th – 19th, 2024

Title: 	WF on NR_NTN_Ph3_UERF

Agenda Item:	9.15.4

Source: 	Qualcomm Inc.

Document for:	Approval

Numbering of sub-topics in this WF is aligned with issue numbers in moderator summary R4-2405291.

# Topic #1: Work plan and RedCap features in scope

## Sub-topic 1-1 & 1-2: UE RF impact from DL coverage enhancement and UL capacity enhancements

Agreement: Wait for RAN1 progress before concluding whether there is RAN4 spec impact.

## Sub-topic 1-3: HD-FDD (e)RedCap

Agreement: Start the discussions on the RF requirements for NTN RedCap with HD-FDD in the next meeting (RAN4#111).

## Sub-topic 1-4: 1Rx vs. 2Rx for (e)RedCap

Agreement: Consider both 1Rx and 2Rx for requirements.

## Sub-topic 1-5: eRedCap BW reduction

Agreement: Specify both eRedCap variants, i.e. with and without bandwidth reduction.

# Topic 2: NTN (e)RedCap UE RF requirements

## Sub-topic 2-1: Target power class

Agreement: Target power class is PC3.

## Sub-topic 2-2: Channel bandwidths and SCS

Agreement: Adopt the same bandwidth as well as SCS as TN RedCap and TN eRedCap UE.

## Sub-topic 2-3: Re-use of TN requirements and specification structure

Agreement: Add RedCap and eRedCap NTN requirements under the same new suffix into TS 38.101-5. Consider re-use of TN specification structure, i.e. which clauses are under suffix I in TS 38.101-1, as starting point.

## Sub-topic 2-4: Operating bands

Agreement: All the NR-NTN FR1-NTN bands will be considered.

## Sub-topic 2-5: 2 Rx FD-FDD Refsens

Agreement: Re-use non-RedCap NTN UE refsens for 2Rx FD-FDD NTN (e)RedCap UE

## Sub-topic 2-6: 1 Rx FD-FDD Refsens

Agreement: Re-use TN (e)RedCap UE 1 Rx refsens allowance for NTN (e)RedCap UE as follows

| Operating band | Channel bandwidth (MHz) | ΔR1R (dB) |
| --- | --- | --- |
| FDD band | 5 | 2.5 |
| FDD band | 10, 15, 20 | 3.0 |