3GPP TSG-RAN WG4 Meeting#111	R4-2407720

# Fukuoka, Japan, 20 – 24 May 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | 0082 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_channel_raster_enh-Core) Amendment of the definition of the enhanced channel raster |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_channel_raster_enh-Core |  |  |  |  |  | Date: |  |  | 2024-05-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Make clear the objective of the enhanced raster and why this must be supported. The enhanced channel raster with 10 kHz granularity for both the UE and BS was specified to meet the work item objective of “specify[ing] necessary changes to the UE channel raster such that configuring a narrower UE channel BW inside a wider gNB channel BW is always possible”. The concrete problem is handling of UE implementations only capable of locating their channel bandwidth (when configured by RRC) on the 100 kHz channel raster. That the enhanced channel raster and the associated capability lift this restriction is not clear from 38.101-1: a UE indicating the capability enhancedChannelRaster-r18 in BandNR shall - be capable of locating its UE-specific channel bandwith in accordance with ServingCellConfig with PRB granularity within a carrier bandwidth - meet the minimum requirements (scope of 38.101-1) for all possible locations each of which is on the Enhanced channel raster but not necessarily on the 100 kHz raster. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 5.4.2.3: specify that indication of enhancedChannelRaster-r18 for a band implies that the UE is compliant with minimum requirements for all possible UE specific channel bandwidth and locations configurable by ServingCellConfig for the DL and UL in the band. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The objective of the enhanced raster and why this must be supported are unclear. Non-supporting UEs require specific configurations by the network. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | . |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

#### 5.4.2.3	Channel raster entries for each operating band

The RF channel positions on the channel raster in each NTN satellite operating band are given through the applicable NR-ARFCN in Table 5.4.2.3-1 and Table 5.4.2.3-2, using the channel raster to resource element mapping in clause 5.4.2.2.

For NTN satellite operating bands with 100 kHz channel raster, ΔFRaster = 20 × ΔFGlobal. In this case every 20th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-1 is given as <20> for FR1-NTN and Table 5.4.2.3-3 for FR2-NTN.

Table 5.4.2.3-1: Applicable NR-ARFCN per operating band in FR1-NTN

| NTN satellite operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 100 | 396000 – <20> – 402000 | 434000 – <20> – 440000 |
| n255 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |
| n254 | 100 | 322000 – <20> – 325300 | 496700 – <20> – 500000 |
| NOTE : The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. |  |  |  |

For NTN operating bands with 100 kHz channel raster, Enhanced channel raster is defined with ΔFRaster = 2 × ΔFGlobal. In this case every 2th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.2.3-2 is given as <2>.

A UE indicating enhancedChannelRaster-r18 for a band supports the Enhanced channel raster and is compliant with minimum requirements for all possible UE specific channel bandwidth and locations configurable by ServingCellConfig [8] for the DL and UL in the band.

Table 5.4.2.3-2: Applicable NR-ARFCN per operating band in FR1-NTN

< end of changes >