3GPP TSG-RAN WG4 Meeting #111                                                     R4-2407796

# Fokuoka, Japan, May 20 –24, 2024

Source:		Nokia

Title:	RF testing methodology for FR2 non-handheld UE that can transmit simultaneously with multi-panel

Agenda item:		10.5.2

Document for:	Discussion

# Introduction

In this contribution we discuss RF testing methodology for FR2 non-handheld UE that can transmit simultaneously with multi-panel based on the WF from last RAN4 meeting in Changsha China [1]. In this paper, we’d like to share our views about on Test method for configured transmitted power as listed below:

Issue 2-6-1: AoA separation and UE orientation for EIRPmax - Proposals - Proposal 1: The EIRPmax is verified with AoA offset = 30° at the peak direction of all tested AoA pairs. - Proposal 2: FFS on the selection of proper AoA pair(s) for aggregated EIRP and TRP measurement in configured transmitted power requirement. - WF - FFS on AoA separation and proper UE orientation for EIRPmax verification. Issue 2-6-2: AoA separation and UE orientation for TRPmax - Proposals - Proposal 1: FFS on how to test TRP when 2 beams are activated simultaneously. - Proposal 2: FFS on the selection of proper AoA pair(s) for aggregated EIRP and TRP measurement in configured transmitted power requirement. - WF - FFS on AoA separation and proper UE orientation for TRPmax verification. Issue 2-6-3: Alternative test method for EIRPmax and TRPmax - Proposals - Proposal 1: If the peak EIRP and TRP of single carrier is smaller than EIRPmax – 3dB or TRPmax – 3dB, the corresponding EIRPmax or TRPmax verification can be skipped. - WF - FFS on alternative test method for EIRPmax and TRPmax verification.

# Discussion

  - Views on separation and UE orientation for EIRPmax

In RAN4 109 meeting, RAN4 has defined the Configured transmitted power for simultaneous transmission to multiple directions for STxMP and the changes has been implemented in 38.101-2 [2]. This requirement basically provides a maximum power output limitation of the STxMP in term of EIRP, and this is for both overlapping and non-overlapping cases of the two main beams without indicating the transmission angles of the two TX beams.

Considering that the side lobe from one Tx panel can add on to the main beam from the other Tx panels, and this is another type of overlapping case which the sum of them may violate the EIRPmax limitation even the EIPR of each of Tx beams complies with the EIPRmax limitation. It is hard to predict in which AoA offset, this type of overlapped EIRP will be higher than EIRPmax. Therefore, the AoA offset from 0-180 degrees should be in the scope of OTA test. Considering the test time issue, we suggest selecting a few AoA offsets in the OTA test for EIRPmax verification.

Observation 1: There are two types of overlapping cases in STxMP: i.e., (1) overlapping of the two main beams from two TXs, and (2) overlapping of the man beam from one Tx and the side lobe from the other Tx.

Observation 2: The AoA offset from 0-180 degrees should be in the scope of OTA test due to the risk of that a sum of EIRP of the main beam of one Tx and the sidelobe of the other Tx may higher than EIRPmax in any AoA offsets.

Proposal 1:  Define a few AoA separation/offsets in the OTA test for EIRPmax verification.

In 38.101-2, J.2 Test conditions and angle definitions, the orientation of the UE are well defined. Suggest reusing the current orientation for STxMP OTA test.

Proposal 2: reuse the orientation definitions in current standards.

  - Views on separation and UE orientation for TRPmax

TRP is the integration of all the EIRP in all the 3D directions. There is no different of overlapping and non-overlapping in terms of TRP OTA test. However, during the OTA test of EIRP, the data are available, the calculation of TRP based on tested EIRP data is achievable.

Observation 3: TRP can be calculated based on EIRP OTA measurement data.

Proposal 3: No need to redefine TRP OTA tests, the TRPmax verification can be done together with EIRPmax verification.

  - Views on Alternative test method for EIRPmax and TRPmax

For a full test of each UE with STxMP capability, the UE need to pass single Tx OTA tests for each panel, as well as pass the STxMP OTA test for using the two Tx. The STxMP cannot be avoid in any case, because one of the reasons is that the antenna radiation pattern will be slightly different between only activating one panels and activating two panels at the same time.

The antenna radiation pattern is impacted by the surroundings of the UE, as well as the currents distribution around the PCB and antenna feed/ground points. When UE actives one panel only, the current flowing on the PCB and antenna feed/ground points will be different from those when UE actives two panels at the same time. Therefore, the radiation pattern of one antenna will be impacted by the changes of the current distribution when the other Tx panels are working at the same time.

If the antenna radiation pattern changes, it means the gains of antenna in different directions change. Therefore, the peak EIRP in all directions for single Tx OTA will not be valid for STxMP verification, and the OTA test for STxMP when two Tx panels are working at the same time need to be verified.

Observation 4: the radiation pattern (antenna pattern) will be impacted by current distribution around PCB and antenna feed/ground points. The current distribution is different between only one panel is working and both panels are working at the same time.

Observation 5: the antenna gains in all direction maybe slightly different between when using single Tx panels and when using two Tx panels at the same time. The OTA test data when only active one Tx panel cannot be used for STxMP OTA verification.

Proposal 4: not suggest avoiding the STxMP OTA test in any situation due to the antenna radiation pattern will be slightly different between using single Tx only and using two Tx at the same time. The data from single Tx panel OTA test cannot be used to verify STxMP cases.

# Conclusions

In this contribution, we make the following observations and conclusions:

Observation 1: There are two types of overlapping cases in STxMP: i.e., (1) overlapping of the two main beams from two TXs, and (2) overlapping of the man beam from one Tx and the side lobe from the other Tx.

Observation 2: The AoA offset from 0-180 degrees should be in the scope of OTA test due to the risk of that a sum of EIRP of the main beam of one Tx and the sidelobe of the other Tx may higher than EIRPmax in any AoA offsets.

Proposal 1:  Define a few AoA separation/offsets in the OTA test for EIRPmax verification.

Proposal 2: reuse the orientation definitions in current standards.

Observation 3: TRP can be calculated based on EIRP OTA measurement data.

Proposal 3: No need to redefine TRP OTA tests, the TRPmax verification can be done together with EIRPmax verification.

Observation 4: the radiation pattern (antenna pattern) will be impacted by current distribution around PCB and antenna feed/ground points. The current distribution is different between only one panel is working and both panels are working at the same time.

Observation 5: the antenna gains in all direction maybe slightly different between when using single Tx panels and when using two Tx panels at the same time. The OTA test data when only active one Tx panel cannot be used for STxMP OTA verification.

Proposal 4: not suggest avoiding the STxMP OTA test in any situation due to the antenna radiation pattern will be slightly different between using single Tx only and using two Tx at the same time. The data from single Tx panel OTA test cannot be used to verify STxMP cases.

# References

[1]	R4-2406087, Way Forward for [110bis][339] NR_FR2_OTA

[2]          R4-2321727, CR to introduce configured transmitted power for STxMP