3GPP TSG-RAN WG4 Meeting # 111	R4-2408563

# Fukuoka City, Fukuoka , Japan, 20th – 24th May, 2024

Agenda item:	7.4.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on enhancement for FR2 SCell activation delay reduction

Document for:	Discussion

# Introduction

The Rel-18 WI of even further RRM enhancement was approved in [1] and further revised in [2]. The remaining issues about core requirements maintenance are captured in WF [3]. In this paper, we further provide our views on the remaining issues.

# Discussion

Regarding the applicable scenarios and requirements multiple SCells activation, the remaining issues are summarized as follows:

Issue 1-1: Applicability of multiple SCell activation with L3 reporting on FR1 and FR2 band - Proposal (Nokia): - The conditions shall be defined for FR1 and FR2 band separately, assuming “all to-be-activated SCells are on the same band”. - To adopt the following conditions for multiple SCell activation delay requirement with L3 reporting: - All to-be-activated SCells are unknown on the same FR2 band, and there is no active serving cells on the same band, or - All to-be-activated SCells are on the same FR1 band, and there is at least one unknown to-be-activated SCell and there is no active serving cell or known to-be-activated SCell contiguous to the SCell on the same band. - The above conditions shall be captured in application paragraphs. - Recommended WF: - Discuss in the CR. Issue 1-2: N1 in multiple SCell activation requirement with L3 reporting - Proposal (Nokia): - For multiple non-contiguous unknown SCells on same FR1 band, N1 needs to be counted for the cell detection on the unknown SCells which were not reported and non-contiguous to any of the reported unknown SCells. - Recommended WF: - Discuss in the CR

During the discussion in last meeting, we also had a CR trying to clarify the issue in the spec but was finally postponed since the scenario is not clear enough in last meeting. Thus, we further provide our views in this contribution.

The first question needs to be answer is what scenario is considered in the new clause 8.3.18 compared with legacy multiple SCell activation requirements in clause 8.3.7. Compared the legacy requirements, the new UE feature (i.e. L3 reporting triggered by SCell activation command) may speed up the SCell activation following cases:

- FR1 target SCell:

  - Case 1-1: The UE has L3-RSRP after SCell activation command.

  - Case 1-2: The UE does not have L3-RSRP report but it is contiguous to a Cell with L3-RSRP report after SCell activation command.

  - Case 1-3: The UE does not have L3-RSRP report and it is not contiguous to such a Cell, but there is at least one such Cell is counted in N1.

- FR2 target SCell:

  - Case 2-1: UE have L3-RSRP for at least one SCell in the same FR2 band.

As listed above, for case 1-1 and case 2-1, compared with legacy pure unknown case, L1/L3 procedures can be save the activation delay can be greatly reduced. For case 1-2, Cell search can be saved for this Cell. For case 1-3, for the activation procedure for this particular Cell, all procedures are still there (e.g. AGC/Search/time tracking), but the overall activation delay can still be reduced since the number of Cell counted in N1 could be reduced.

Observation 1: The new UE feature (i.e. L3 reporting triggered by SCell activation command) can speed up the multiple SCell activation for following cases:

- FR1 target SCell:

  - Case 1-1: The UE has L3-RSRP after SCell activation command.

  - Case 1-2: The UE does not have L3-RSRP report but it is contiguous to a Cell with L3-RSRP report after SCell activation command.

  - Case 1-3: The UE does not have L3-RSRP report and it is not contiguous to such a Cell, but there is at least one such Cell is counted in N1.

- FR2 target SCell:

  - Case 2-1: UE have L3-RSRP for at least one SCell in the same FR2 band.

Based on the analysis above, the reasonable split between the new clause and legacy clause is that:

- Capture the requirements for case 1-1 and case 2-1 in 8.3.18

- Case 1-2 and Case 1-3 to be referred to legacy clause with necessary clarification about N1 definition.

Proposal 1: The split between new clause 8.3.17 and the legacy clause 8.3.7 are suggested as follows:

- Capture the requirements for case 1-1 and case 2-1 in 8.3.18

- Case 1-2 and Case 1-3 to be referred to legacy clause with necessary clarification about N1 definition.

The following question is how to modify the equation as now it is exactly the same as single SCell activation requirements. As mentioned in previous meeting, it should be divided into FR1 and FR2 separately. As defined in existing spec, we only consider follows cases:

- Any to-be-activated SCell is unknown and in the same band, and - No any active serving cell(s) or known to-be-activated SCell(s) exists on the same band

For FR2, the situation is much clearer that the applicable case can be interpreted as:

- All to-be-activated SCell are unknown without known to-be-activated SCell or active serving Cell;

- One or more to-be-activated SCells has valid L3 report;

Based on above, the requirements defined for single CC in clause 8.3.17 can apply to each target SCell with some modification as:

For FR2 target SCell, Tactivation_time_multiple_scells is equal to Tactivation_time which is the SCell activation delay in millisecond as specified in Clause 8.3.17 except the definition of Tuncertainty_MAC and Tuncertainty_RRC are replaced with:

-	Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable), relative to

-	First valid L3-RSRP reporting of a to-be-activated SCell within the same band for unknown case, when UE reports valid L3-RSRP.

-	Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to

-	First valid L3-RSRP reporting of a to-be-activated SCell within the same band for unknown case, when UE reports valid L3-RSRP.

Proposal 2: For FR2 target SCell, the requirements can be updated as follows:

For FR2 target SCell, Tactivation_time_multiple_scells is equal to Tactivation_time which is the SCell activation delay in millisecond as specified in Clause 8.3.17 except the definition of Tuncertainty_MAC and Tuncertainty_RRC are replaced with:

-	Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable), relative to

-	First valid L3-RSRP reporting of a to-be-activated SCell within the same band for unknown case, when UE reports valid L3-RSRP.

-	Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to

-	First valid L3-RSRP reporting of a to-be-activated SCell within the same band for unknown case, when UE reports valid L3-RSRP.

For FR1, the situation is much more complicated. Similar as the analysis for FR2, the scenario could be summarized as follows:

- All to-be-activated SCell are unknown without known to-be-activated SCell or active serving Cell;

- The target FR1 SCell has valid L3 reprot;

However, for FR1 multiple SCells activation, based on the principles in legacy requirements, UE may still need to perform AGC using SMTC_Max even the target FR1 SCell is known. If looking at the legacy requirements, the exact number of samples for AGC are further differentiated considering following aspects:

- Measure period for the target Cell

- Whether there is unknown to-be-activated SCell

- The measurement period for the parallel to-be-activated SCell

Following the requirements for single SCell activation in 8.3.17, there is no further differentiation with respect to the measurement period for the target Cell. Thus, follow the same principle, we only need to consider whether there is pure unknown to-be-activated SCell. The cases can be summarized as follows:

- One the same band UE also has at least one parallel to-be-activated SCell which is FR1 unknown SCell without valid L3-RSRP report after SCell activation.

- All parallel to-be-activated SCells are unknown with valid L3 report.

Based on the analysis above, for the second case, single CC requirements can apply directly for each target SCell. For the first case, the SMTC_MAX shall be considered in the equation, which makes the procedure more complicated. We use following fig to elaborate the case.

Fig.1 FR1 multiple SCell activation with L3 report.

The requirements are proposed as follows:

For FR1 target SCell, Tactivation_time_multiple_scells is:

-	3ms + max (4ms + [TL3 report] + Tuncertainty_SP + 3ms+ THARQ, max(TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells, 4ms + [TL3 report] + Tuncertainty_MAC + 3ms + THARQ) + TFineTiming + 2ms ), if the semi-persistent CSI-RS is used for CSI reporting

-	3ms + max (4ms + [TL3 report] + Tuncertainty_RRC + TRRC_delay, max(TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells, 4ms + [TL3 report] + Tuncertainty_MAC + THARQ) + TFineTiming + 2ms), if the periodic CSI-RS is used for CSI reporting

Max(AGC, L3 report + PDCCH TCI)

Max(L3 report + P/SP CSI-RS, Max() + fine timing )

Based on the analysis above, the requirements for FR1 are proposed to be modified as follows:

For FR1 target SCell, Tactivation_time_multiple_scells is:

-	3ms + max (4ms + [TL3 report] + Tuncertainty_SP + 3ms+ THARQ, max(TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells, 4ms + [TL3 report] + Tuncertainty_MAC + 3ms + THARQ) + TFineTiming + 2ms ), if the semi-persistent CSI-RS is used for CSI reporting

-	3ms + max (4ms + [TL3 report] + Tuncertainty_RRC + TRRC_delay, max(TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells, 4ms + [TL3 report] + Tuncertainty_MAC + THARQ) + TFineTiming + 2ms), if the periodic CSI-RS is used for CSI reporting

if on the same band UE also has at least one parallel to-be-activated SCell which is FR1 unknown SCell without valid L3-RSRP report after SCell activation. TFirstSSB_MAX_multiple_scells, TSMTC_MAX_multiple_scells is defined in 8.3.7; if on the same band, UE does not have any parallel to-be-activated SCell which is FR1 unknown SCell without valid L3-RSRP report after SCell activation, requirements in 8.3.17 apply.

Proposal 3: For FR1 target SCell, the requirements can be updated as follows:

For FR1 target SCell, Tactivation_time_multiple_scells is:

-	3ms + max (4ms + [TL3 report] + Tuncertainty_SP + 3ms+ THARQ, max(TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells, 4ms + [TL3 report] + Tuncertainty_MAC + 3ms + THARQ) + TFineTiming + 2ms ), if the semi-persistent CSI-RS is used for CSI reporting

-	3ms + max (4ms + [TL3 report] + Tuncertainty_RRC + TRRC_delay, max(TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells, 4ms + [TL3 report] + Tuncertainty_MAC + THARQ) + TFineTiming + 2ms), if the periodic CSI-RS is used for CSI reporting

if on the same band UE also has at least one parallel to-be-activated SCell which is FR1 unknown SCell without valid L3-RSRP report after SCell activation. TFirstSSB_MAX_multiple_scells, TSMTC_MAX_multiple_scells is defined in 8.3.7; if on the same band, UE does not have any parallel to-be-activated SCell which is FR1 unknown SCell without valid L3-RSRP report after SCell activation, requirements in 8.3.17 apply.

The above proposed changes can also be found in our companied CR in this meeting.

# Conclusions

Observation 1: The new UE feature (i.e. L3 reporting triggered by SCell activation command) can speed up the multiple SCell activation for following cases:

- FR1 target SCell:

  - Case 1-1: The UE has L3-RSRP after SCell activation command.

  - Case 1-2: The UE does not have L3-RSRP report but it is contiguous to a Cell with L3-RSRP report after SCell activation command.

  - Case 1-3: The UE does not have L3-RSRP report and it is not contiguous to such a Cell, but there is at least one such Cell is counted in N1.

- FR2 target SCell:

  - Case 2-1: UE have L3-RSRP for at least one SCell in the same FR2 band.

Proposal 1: The split between new clause 8.3.17 and the legacy clause 8.3.7 are suggested as follows:

- Capture the requirements for case 1-1 and case 2-1 in 8.3.18

- Case 1-2 and Case 1-3 to be referred to legacy clause with necessary clarification about N1 definition.

Proposal 2: For FR2 target SCell, the requirements can be updated as follows:

For FR2 target SCell, Tactivation_time_multiple_scells is equal to Tactivation_time which is the SCell activation delay in millisecond as specified in Clause 8.3.17 except the definition of Tuncertainty_MAC and Tuncertainty_RRC are replaced with:

-	Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable), relative to

-	First valid L3-RSRP reporting of a to-be-activated SCell within the same band for unknown case, when UE reports valid L3-RSRP.

-	Tuncertainty_RRC is the time period between reception of the RRC configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) relative to

-	First valid L3-RSRP reporting of a to-be-activated SCell within the same band for unknown case, when UE reports valid L3-RSRP.

Proposal 3: For FR1 target SCell, the requirements can be updated as follows:

For FR1 target SCell, Tactivation_time_multiple_scells is:

-	3ms + max (4ms + [TL3 report] + Tuncertainty_SP + 3ms+ THARQ, max(TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells, 4ms + [TL3 report] + Tuncertainty_MAC + 3ms + THARQ) + TFineTiming + 2ms ), if the semi-persistent CSI-RS is used for CSI reporting

-	3ms + max (4ms + [TL3 report] + Tuncertainty_RRC + TRRC_delay, max(TFirstSSB_MAX_multiple_scells + TSMTC_MAX_multiple_scells, 4ms + [TL3 report] + Tuncertainty_MAC + THARQ) + TFineTiming + 2ms), if the periodic CSI-RS is used for CSI reporting

if on the same band UE also has at least one parallel to-be-activated SCell which is FR1 unknown SCell without valid L3-RSRP report after SCell activation. TFirstSSB_MAX_multiple_scells, TSMTC_MAX_multiple_scells is defined in 8.3.7; if on the same band, UE does not have any parallel to-be-activated SCell which is FR1 unknown SCell without valid L3-RSRP report after SCell activation, requirements in 8.3.17 apply.

# References

[1] RP-220977 New WID: Even Further RRM enhancement for NR and MR-DC

[2] RP-221696 Revised WID: Even Further RRM enhancement for NR and MR-DC

[3] R4-2406339 WF for [110bis][207] NR_RRM_enh3_part1