3GPP TSG-RAN WG4 Meeting #111	R4-2408671

Fukuoka, Japan, May 20 – May 24, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DRAFT | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for eEMR test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | R4 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | Nokia |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_Enh2-Perf |  |  |  |  |  | Date: |  |  | 2024-05-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Test cases for fast SCell setup requirements are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This draft CR provides a high level test case proposals for Rel-18 SCell setup delay improvement test cases. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-18 requirements are not tested. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New A.7.6.X1, A.7.6.X2, A.7.6.X3, A.7.6.X4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  |  |  |  |
| affected: |  |  |  | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of change 1>

### A.7.6.X1	Measurement reporting for fast CA/DC setup

#### A.7.6.X1.1	Early reporting of inter-frequency SA cell re-selection measurement for FR2 without measurement results

##### A.7.6.X1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly reports valid cell re/selection measurements for the configured inter-frequency carrier in MeasReselectionCarrierListNR-r18 during connection setup. The test verifies the requirements specified in section 4.7 when measReselectionValidityDuration-r18 is configured for the case when there are no measurement results to report at RRC connection setup.

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.  The test parameters are given in Tables A.7.6.X1.1.1-1, A.7.6.X1.1.1-2, A.7.6.X1.1.1-3 and A.7.6.X1.1.1-4.

The test consists of four successive time periods, with time duration of T1, T2, T3, and T4, respectively.

During T1, the UE is connected to cell 1 only and shall not have any timing information of cell 2. UE is configured with cell re-selection measurements for cell 2 in MeasReselectionCarrierListNR-r18, and with measReselectionValidityDuration-r18. The connection is released at the end of T1.

T2 starts when the connection is released. During the time period T2 and T3 UE is in idle mode and configured to perform cell re-selection measurements on cell 2. Cell 2 signal is turned off during T3. The duration of T3 equals to measReselectionValidityDuration-r18 seconds.

At the beginning of T4, the UE is paged for connection setup and requested by the network to report valid idle mode measurements for cell 2.

Table A.7.6.X1.1.1-1: supported test configuration

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.6.X1.1.1-2: General test parameters for SA cell re-selection measurement for FR2

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | Two FR2 NR carrier frequencies is used. |
| Active cell |  | NR cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | NR cell 2 | NR cell 2 is on NR RF channel number 2. |
| SMTC-SSB parameters |  | SSB.1 FR2 | As specified in clause A.3.10.2 |
| Hysteresis | dB | 0 |  |
| PRACH configuration index |  | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| CP length |  | Normal |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX in connected mode |  | OFF | DRX is not used |
| DRX in idle mode | s | 0.32 | The value shall be used for all cells in the test. |
| Time offset between serving and neighbour cells |  | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
| measReselectionValidityDuration-r18 | s | [100] |  |
| T1 | s | 10 |  |
| T2 | s | [154] | Time for cell 2 detection and measurement in idle mode in FR2 |
| T3 | s | [20] | The duration of T3 equals to the configured value measReselectionValidityDuration-r18 |
| T4 | s | 10 |  |

Table A.7.6.X1.1.1-3: Cell specific test parameters for connected mode for FR2

| Parameter |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  |  | TDD |  |  |  |  |  |  |  |
| TDD configuration |  |  | TDDConf.3.1 |  |  |  |  |  |  |  |
| BWchannel |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |  |
| BWP BW |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |  |  | NA |  |  |  |
|  | Initial UL BWP |  | DLBWP.1.1 |  |  |  | NA |  |  |  |
|  | Dedicated DL BWP |  | ULBWP.0.1 |  |  |  | NA |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  | NA |  |  |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |  |  | NA |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | OP.1 |  |  |  | OP.1 |  |  |  |
| PDSCH Reference measurement channel |  |  | SR3.1 TDD |  |  |  | SR3.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | CR3.1 TDD |  |  |  | CR3.1 TDD |  |  |  |
| SSB parameters |  |  | SSB. 3 FR2 |  |  |  | SSB. 3 FR2 |  |  |  |
| SMTC configuration defined in A.3.11 |  |  | SMTC.1 |  |  |  | SMTC.1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | 0 |  |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 |  |  |  | -98 |  |  |  |
| Note2 |  | dBm/SCS | -98 |  |  |  | -98 |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | [-81] | [-81] | [-81] | [-81] | -inf | -inf | -inf | -inf |
|  |  | dB | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
|  |  | dB | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
| IoNote3 |  | dBm/9.36MHz | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
| Propagation Condition |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |

Table A.7.6.X1.1.1-4: Cell specific test parameters for idle mode for FR2

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  | 1 |  |  |  | 2 |  |  |  |
| TDD configuration |  | N/A |  |  |  | N/A |  |  |  |
| PDSCH RMC |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |  |
| RMSI CORESET |  | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |  |
| Dedicated CORESET |  | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  |  |
| OCNG Pattern |  | OP.1 defined in A.3.2.1 |  |  |  | OP.1 defined in A.3.2.1 |  |  |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  |  |  | DLBWP.0.1 |  |  |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  |  |  | ULBWP.0.1 |  |  |  |
| RLM-RS |  | SSB |  |  |  | SSB |  |  |  |
| Qrxlevmin | dBm/SCS | -140 |  |  |  | -140 |  |  |  |
| Pcompensation | dB | 0 |  |  |  | 0 |  |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | SS-RSRP |  |  |  | SS-RSRP |  |  |  |
|  | dB | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
| Note2 | dBm/SCS | -98 |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |  |  |  |  |  |
|  | dB | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
| ÊsIot | dB | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
| SS-RSRP | dBm/SCS | -81 | -81 | -81 | -81 | -inf | -inf | -inf | -inf |
| Io | dBm/9.36 MHz | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
| Treselection | s | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 |
| SnonintrasearchP | dB | Not sent |  |  |  | Not sent |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

##### A.7.6.X1.1.2	Test Requirements

The UE behaviour during time durations T2, T3 and T4 shall be as follows:

During the time period T2 and T3, the UE is in idle mode.

At the beginning of T2, the signal level of cell 2 changes. The UE shall perform cell reselection measurements according to Section 4.2.2.4. The UE shall not perform reselection.

At the beginning of T3, signal level of cell 2 is turned off.

At the beginning of T4, the UE is paged for connection setup. During the connection setup the UE is requested to report valid idle mode measurements for cell 2. The UE shall not send early measurement report to PCell in this test.

<End of change 1>

<Start of change 2>

### A.7.6.X1	Measurement reporting for fast CA/DC setup

#### A.7.6.X2.1	Early reporting of inter-frequency SA cell re-selection measurement for FR2 with measurement results

##### A.7.6.X2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly reports valid cell re/selection measurements for the configured inter-frequency carrier in MeasReselectionCarrierListNR-r18 during connection setup. The test verifies the requirements specified in section 4.7 when measReselectionValidityDuration-r18 is configured.

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.  The test parameters are given in Tables A.7.6.X2.1.1-1, A.7.6.X2.1.1-2, A.7.6.X2.1.1-3 and A.7.6.X2.1.1-4.

The test consists of four successive time periods, with time duration of T1, T2, T3, and T4, respectively.

During T1, the UE is connected to cell 1 only and shall not have any timing information of cell 2. UE is configured with cell re-selection measurements for cell 2 in MeasReselectionCarrierListNR-r18, and with measReselectionValidityDuration-r18. The connection is released at the end of T1.

T2 starts when the connection is released. During the time period T2 and T3 UE is in idle mode and configured to perform cell re-selection measurements on cell 2. SS-RSRP of cell 2 during T2 is different than SS-RSRP during T3. The duration of T3 equals to measReselectionValidityDuration-r18 seconds.

At the beginning of T4, the UE is paged for connection setup and requested by the network to report valid idle mode measurements for cell 2.

Table A.7.6.X2.1.1-1: supported test configuration

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.6.X2.1.1-2: General test parameters for SA cell re-selection measurement for FR2

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | Two FR2 NR carrier frequencies is used. |
| Active cell |  | NR cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | NR cell 2 | NR cell 2 is on NR RF channel number 2. |
| SMTC-SSB parameters |  | SSB.1 FR2 | As specified in clause A.3.10.2 |
| Hysteresis | dB | 0 |  |
| PRACH configuration index |  | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| CP length |  | Normal |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX in connected mode |  | OFF | DRX is not used |
| DRX in idle mode | s | 0.32 | The value shall be used for all cells in the test. |
| Time offset between serving and neighbour cells |  | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
| measReselectionValidityDuration-r18 | s | [100] |  |
| T1 | s | 10 |  |
| T2 | s | [154] | Time for cell 2 detection and measurement in idle mode in FR2 |
| T3 | s | [20] | The duration of T3 equals to the configured value measReselectionValidityDuration-r18 |
| T4 | s | 10 |  |

Table A.7.6.X2.1.1-3: Cell specific test parameters for connected mode for FR2

| Parameter |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  |  | TDD |  |  |  |  |  |  |  |
| TDD configuration |  |  | TDDConf.3.1 |  |  |  |  |  |  |  |
| BWchannel |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |  |
| BWP BW |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |  |  | NA |  |  |  |
|  | Initial UL BWP |  | DLBWP.1.1 |  |  |  | NA |  |  |  |
|  | Dedicated DL BWP |  | ULBWP.0.1 |  |  |  | NA |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  | NA |  |  |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |  |  | NA |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | OP.1 |  |  |  | OP.1 |  |  |  |
| PDSCH Reference measurement channel |  |  | SR3.1 TDD |  |  |  | SR3.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | CR3.1 TDD |  |  |  | CR3.1 TDD |  |  |  |
| SSB parameters |  |  | SSB. 3 FR2 |  |  |  | SSB. 3 FR2 |  |  |  |
| SMTC configuration defined in A.3.11 |  |  | SMTC.1 |  |  |  | SMTC.1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | 0 |  |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 |  |  |  | -98 |  |  |  |
| Note2 |  | dBm/SCS | -98 |  |  |  | -98 |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | [-81] | [-81] | [-81] | [-81] | -inf | [-87] | [-102] | [-102] |
|  |  | dB | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
|  |  | dB | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
| IoNote3 |  | dBm/9.36MHz | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
| Propagation Condition |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |

Table A.7.6.X2.1.1-4: Cell specific test parameters for idle mode for FR2

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  | 1 |  |  |  | 2 |  |  |  |
| TDD configuration |  | N/A |  |  |  | N/A |  |  |  |
| PDSCH RMC |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |  |
| RMSI CORESET |  | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |  |
| Dedicated CORESET |  | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  |  |
| OCNG Pattern |  | OP.1 defined in A.3.2.1 |  |  |  | OP.1 defined in A.3.2.1 |  |  |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  |  |  | DLBWP.0.1 |  |  |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  |  |  | ULBWP.0.1 |  |  |  |
| RLM-RS |  | SSB |  |  |  | SSB |  |  |  |
| Qrxlevmin | dBm/SCS | -140 |  |  |  | -140 |  |  |  |
| Pcompensation | dB | 0 |  |  |  | 0 |  |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | SS-RSRP |  |  |  | SS-RSRP |  |  |  |
|  | dB | [TBD] | [TBD] | [TBD] | [TBD] | -infinity | [TBD] | [TBD] | [TBD] |
| Note2 | dBm/SCS | -98 |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |  |  |  |  |  |
|  | dB | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
| ÊsIot | dB | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
| SS-RSRP | dBm/SCS | -81 | -81 | -81 | -81 | -inf | -87 | -102 | -102 |
| Io | dBm/9.36 MHz | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
| Treselection | s | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 |
| SnonintrasearchP | dB | Not sent |  |  |  | Not sent |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

##### A.7.6.X2.1.2	Test Requirements

The UE behaviour during time durations T2, T3 and T4 shall be as follows:

During the time period T2 and T3, the UE is in idle mode.

At the beginning of T2, the signal level of cell 2 changes. The UE shall perform cell reselection measurements according to Section 4.2.2.4. The UE shall not perform reselection.

At the beginning of T3, signal level of cell 2 changes again. The UE shall continue to perform cell reselection measurements according to Section 4.2.2.4. The UE shall not perform reselection.

At the beginning of T4, the UE is paged for connection setup. During the connection setup the UE is requested to report valid idle mode measurements for cell 2. The UE shall send early measurement report to PCell with valid measurement results, where the reported measurements are considered valid if they pass the validity check as defined in 4.7.3.

After receiving the early measurement report, the test equipment verifies the accuracy of measurements reported for cell 2 meets the requirements [in Section 10.1.5B for SS-RSRP and in Section 10.1.10B for SS-RSRQ], and the test ends. In the test case, the reported measurements are considered valid if they fulfil measurement accuracy requirements according to cell 2 signal level during T3.

The rate of correct events observed during repeated tests shall be at least 90%.

<End of change 2>

<Start of change 3>

### A.7.6.X1	Measurement reporting for fast CA/DC setup

#### A.7.6.X3.1	Early reporting of inter-frequency SA cell EMR measurements for FR2 without measurement results

##### A.7.6.X3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly reports valid cell re/selection measurements for the configured inter-frequency carrier in MeasIdleCarrierListNR-r18 during connection setup. The test verifies the requirements specified in section 4.7 when MeasIdleValidityDuration-r18 is configured for the case when there are no measurement results to report at RRC connection setup.

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.  The test parameters are given in Tables A.7.6.X3.1.1-1, A.7.6.X3.1.1-2, A.7.6.X3.1.1-3 and A.7.6.X3.1.1-4.

The test consists of four successive time periods, with time duration of T1, T2, T3, and T4, respectively.

During T1, the UE is connected to cell 1 only and shall not have any timing information of cell 2. UE is configured with cell re-selection measurements for cell 2 in MeasIdleCarrierListNR-r18, and with MeasIdleValidityDuration-r18. The connection is released at the end of T1.

T2 starts when the connection is released. During the time period T2 and T3 UE is in idle mode and configured to perform cell re-selection measurements on cell 2. Cell 2 signal is turned off during T3. The duration of T3 equals to MeasIdleValidityDuration-r18 seconds.

At the beginning of T4, the UE is paged for connection setup and requested by the network to report valid idle mode measurements for cell 2.

Table A.7.6.X3.1.1-1: supported test configuration

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.6.X3.1.1-2: General test parameters for SA cell re-selection measurement for FR2

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | Two FR2 NR carrier frequencies is used. |
| Active cell |  | NR cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | NR cell 2 | NR cell 2 is on NR RF channel number 2. |
| SMTC-SSB parameters |  | SSB.1 FR2 | As specified in clause A.3.10.2 |
| Hysteresis | dB | 0 |  |
| PRACH configuration index |  | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| CP length |  | Normal |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX in connected mode |  | OFF | DRX is not used |
| DRX in idle mode | s | 0.32 | The value shall be used for all cells in the test. |
| Time offset between serving and neighbour cells |  | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
| MeasIdleValidityDuration-r18 | s | [100] |  |
| T331 | s | 300 |  |
| T1 | s | 10 |  |
| T2 | s | [154] | Time for cell 2 detection and measurement in idle mode in FR2 |
| T3 | s | [20] | The duration of T3 equals to the configured value MeasIdleValidityDuration-r18 |
| T4 | s | 10 |  |

Table A.7.6.X3.1.1-3: Cell specific test parameters for connected mode for FR2

| Parameter |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  |  | TDD |  |  |  |  |  |  |  |
| TDD configuration |  |  | TDDConf.3.1 |  |  |  |  |  |  |  |
| BWchannel |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |  |
| BWP BW |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |  |  | NA |  |  |  |
|  | Initial UL BWP |  | DLBWP.1.1 |  |  |  | NA |  |  |  |
|  | Dedicated DL BWP |  | ULBWP.0.1 |  |  |  | NA |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  | NA |  |  |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |  |  | NA |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | OP.1 |  |  |  | OP.1 |  |  |  |
| PDSCH Reference measurement channel |  |  | SR3.1 TDD |  |  |  | SR3.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | CR3.1 TDD |  |  |  | CR3.1 TDD |  |  |  |
| SSB parameters |  |  | SSB. 3 FR2 |  |  |  | SSB. 3 FR2 |  |  |  |
| SMTC configuration defined in A.3.11 |  |  | SMTC.1 |  |  |  | SMTC.1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | 0 |  |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 |  |  |  | -98 |  |  |  |
| Note2 |  | dBm/SCS | -98 |  |  |  | -98 |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | [-81] | [-81] | [-81] | [-81] | -inf | -inf | -inf | -inf |
|  |  | dB | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
|  |  | dB | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
| IoNote3 |  | dBm/9.36MHz | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
| Propagation Condition |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |

Table A.7.6.X3.1.1-4: Cell specific test parameters for idle mode for FR2

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  | 1 |  |  |  | 2 |  |  |  |
| TDD configuration |  | N/A |  |  |  | N/A |  |  |  |
| PDSCH RMC |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |  |
| RMSI CORESET |  | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |  |
| Dedicated CORESET |  | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  |  |
| OCNG Pattern |  | OP.1 defined in A.3.2.1 |  |  |  | OP.1 defined in A.3.2.1 |  |  |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  |  |  | DLBWP.0.1 |  |  |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  |  |  | ULBWP.0.1 |  |  |  |
| RLM-RS |  | SSB |  |  |  | SSB |  |  |  |
| Qrxlevmin | dBm/SCS | -140 |  |  |  | -140 |  |  |  |
| Pcompensation | dB | 0 |  |  |  | 0 |  |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | SS-RSRP |  |  |  | SS-RSRP |  |  |  |
|  | dB | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
| Note2 | dBm/SCS | -98 |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |  |  |  |  |  |
|  | dB | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
| ÊsIot | dB | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
| SS-RSRP | dBm/SCS | -81 | -81 | -81 | -81 | -inf | -inf | -inf | -inf |
| Io | dBm/9.36 MHz | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
| Treselection | s | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 |
| SnonintrasearchP | dB | Not sent |  |  |  | Not sent |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

##### A.7.6.X3.1.2	Test Requirements

The UE behaviour during time durations T2, T3 and T4 shall be as follows:

During the time period T2 and T3, the UE is in idle mode.

At the beginning of T2, the signal level of cell 2 changes. The UE shall perform cell reselection measurements according to Section 4.2.2.4. The UE shall not perform reselection.

At the beginning of T3, signal level of cell 2 is turned off.

At the beginning of T4, the UE is paged for connection setup. During the connection setup the UE is requested to report valid idle mode measurements for cell 2. The UE shall not send early measurement report to PCell in this test.

<End of change 3>

<Start of change 4>

### A.7.6.X1	Measurement reporting for fast CA/DC setup

#### A.7.6.X4.1	Early reporting of inter-frequency SA cell EMR measurement for FR2 with measurement results

##### A.7.6.X4.1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE correctly reports valid cell re/selection measurements for the configured inter-frequency carrier in MeasIdleCarrierListNR-r18 during connection setup. The test verifies the requirements specified in section 4.7 when MeasIdleValidityDuration-r18 is configured.

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.  The test parameters are given in Tables A.7.6.X4.1.1-1, A.7.6.X4.1.1-2, A.7.6.X4.1.1-3 and A.7.6.X4.1.1-4.

The test consists of four successive time periods, with time duration of T1, T2, T3, and T4, respectively.

During T1, the UE is connected to cell 1 only and shall not have any timing information of cell 2. UE is configured with cell re-selection measurements for cell 2 in MeasIdleCarrierListNR-r18, and with MeasIdleValidityDuration-r18. The connection is released at the end of T1.

T2 starts when the connection is released. During the time period T2 and T3 UE is in idle mode and configured to perform cell re-selection measurements on cell 2. SS-RSRP of cell 2 during T2 is different than SS-RSRP during T3. The duration of T3 equals to MeasIdleValidityDuration-r18 seconds.

At the beginning of T4, the UE is paged for connection setup and requested by the network to report valid idle mode measurements for cell 2.

Table A.7.6.X4.1.1-1: supported test configuration

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.6.X4.1.1-2: General test parameters for SA cell re-selection measurement for FR2

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | Two FR2 NR carrier frequencies is used. |
| Active cell |  | NR cell 1 (PCell) | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | NR cell 2 | NR cell 2 is on NR RF channel number 2. |
| SMTC-SSB parameters |  | SSB.1 FR2 | As specified in clause A.3.10.2 |
| Hysteresis | dB | 0 |  |
| PRACH configuration index |  | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| CP length |  | Normal |  |
| TimeToTrigger | s | 0 |  |
| Filter coefficient |  | 0 | L3 filtering is not used |
| DRX in connected mode |  | OFF | DRX is not used |
| DRX in idle mode | s | 0.32 | The value shall be used for all cells in the test. |
| Time offset between serving and neighbour cells |  | 3ms | Asynchronous cells. The timing of Cell 2 is 3ms later than the timing of Cell 1. |
| MeasIdleValidityDuration-r18 | s | [20] |  |
| T331 | s | 300 |  |
| T1 | s | 10 |  |
| T2 | s | [154] | Time for cell 2 detection and measurement in idle mode in FR2 |
| T3 | s | [20] | The duration of T3 equals to the configured value MeasIdleValidityDuration-r18 |
| T4 | s | 10 |  |

Table A.7.6.X4.1.1-3: Cell specific test parameters for connected mode for FR2

| Parameter |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  |  | 1 |  |  |  | 2 |  |  |  |
|  |  |  | TDD |  |  |  |  |  |  |  |
| TDD configuration |  |  | TDDConf.3.1 |  |  |  |  |  |  |  |
| BWchannel |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |  |
| BWP BW |  | MHz | 100: NRB,c = 66 |  |  |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |  |  | NA |  |  |  |
|  | Initial UL BWP |  | DLBWP.1.1 |  |  |  | NA |  |  |  |
|  | Dedicated DL BWP |  | ULBWP.0.1 |  |  |  | NA |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  | NA |  |  |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |  |  | NA |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | OP.1 |  |  |  | OP.1 |  |  |  |
| PDSCH Reference measurement channel |  |  | SR3.1 TDD |  |  |  | SR3.1 TDD |  |  |  |
| CORESET Reference Channel |  |  | CR3.1 TDD |  |  |  | CR3.1 TDD |  |  |  |
| SSB parameters |  |  | SSB. 3 FR2 |  |  |  | SSB. 3 FR2 |  |  |  |
| SMTC configuration defined in A.3.11 |  |  | SMTC.1 |  |  |  | SMTC.1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 |  |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | 0 |  |  |  | 0 |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  | dBm/15kHz | -98 |  |  |  | -98 |  |  |  |
| Note2 |  | dBm/SCS | -98 |  |  |  | -98 |  |  |  |
| SS-RSRP Note 3 |  | dBm/SCS | [-81] | [-81] | [-81] | [-81] | -inf | [-87] | [-102] | [-102] |
|  |  | dB | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
|  |  | dB | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
| IoNote3 |  | dBm/9.36MHz | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
| Propagation Condition |  |  | AWGN |  |  |  | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |  |  |

Table A.7.6.X4.1.1-4: Cell specific test parameters for idle mode for FR2

| Parameter | Unit | Cell 1 |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T1 | T2 | T3 | T4 |
| NR RF Channel Number |  | 1 |  |  |  | 2 |  |  |  |
| TDD configuration |  | N/A |  |  |  | N/A |  |  |  |
| PDSCH RMC |  | SR.1.1 TDD |  |  |  | SR.1.1 TDD |  |  |  |
| RMSI CORESET |  | CR.1.1 TDD |  |  |  | CR.1.1 TDD |  |  |  |
| Dedicated CORESET |  | CCR.1.1 TDD |  |  |  | CCR.1.1 TDD |  |  |  |
| OCNG Pattern |  | OP.1 defined in A.3.2.1 |  |  |  | OP.1 defined in A.3.2.1 |  |  |  |
| Initial DL BWP configuration |  | DLBWP.0.1 |  |  |  | DLBWP.0.1 |  |  |  |
| Initial UL BWP configuration |  | ULBWP.0.1 |  |  |  | ULBWP.0.1 |  |  |  |
| RLM-RS |  | SSB |  |  |  | SSB |  |  |  |
| Qrxlevmin | dBm/SCS | -140 |  |  |  | -140 |  |  |  |
| Pcompensation | dB | 0 |  |  |  | 0 |  |  |  |
| Cell_selection_and_ reselection_quality_measurement |  | SS-RSRP |  |  |  | SS-RSRP |  |  |  |
|  | dB | [TBD] | [TBD] | [TBD] | [TBD] | -infinity | [TBD] | [TBD] | [TBD] |
| Note2 | dBm/SCS | -98 |  |  |  |  |  |  |  |
| Note2 | dBm/15 kHz | -98 |  |  |  |  |  |  |  |
|  | dB | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
| ÊsIot | dB | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
| SS-RSRP | dBm/SCS | -81 | -81 | -81 | -81 | -inf | -87 | -102 | -102 |
| Io | dBm/9.36 MHz | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] | [TBD] |
| Treselection | s | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 |
| SnonintrasearchP | dB | Not sent |  |  |  | Not sent |  |  |  |
| Propagation Condition |  | AWGN |  |  |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

##### A.7.6.X4.1.2	Test Requirements

The UE behaviour during time durations T2, T3 and T4 shall be as follows:

During the time period T2 and T3, the UE is in idle mode.

At the beginning of T2, the signal level of cell 2 changes. The UE shall perform cell reselection measurements according to Section 4.2.2.4. The UE shall not perform reselection.

At the beginning of T3, signal level of cell 2 changes again. The UE shall continue to perform cell reselection measurements according to Section 4.2.2.4. The UE shall not perform reselection.

At the beginning of T4, the UE is paged for connection setup. During the connection setup the UE is requested to report valid idle mode measurements for cell 2. The UE shall send early measurement report to PCell with valid measurement results, where the reported measurements are considered valid if they pass the validity check as defined in 4.7.3.

After receiving the early measurement report, the test equipment verifies the accuracy of measurements reported for cell 2 meets the requirements [in Section 10.1.5B for SS-RSRP and in Section 10.1.10B for SS-RSRQ], and the test ends. In the test case, the reported measurements are considered valid if they fulfil measurement accuracy requirements according to cell 2 signal level during T3.

The rate of correct events observed during repeated tests shall be at least 90%.

<End of change 4>