# 3GPP TSG- RAN WG4 Meeting # 112	R4-2411859

# Maastricht, Netherlands, 19th–23rd Aug, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on system parameters for NR NTN Ku band

# Agenda item:	8.9.3

# Document for:	Approval

# 1 Introduction

In RAN#104 meeting, a new Work Item [1] was approved on introduction of Ku Band for NR NTN for NR NTN with the following RAN4 objective. In this contribution, we want to share some initial views about NR NTN Ku band from system parameters perspective.

The following assumptions are taken as the baseline for this work: - GSO and NGSO - NTN capable UE type 1 to 5 (38.101-5 Table 9.2.1.0-1) for above 10 GHz (no handheld devices) - FDD Mode - GNSS Receiver capability The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 excluding US: - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 excluding US - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate TX/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services . - Specify RF requirements for satellite access node and NTN VSAT types in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Potential approaches for achieving this shall be studied, from which one shall be selected including but not limited to: - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing - RAN4 will select which one of the above approaches for DL and UL will be pursued by RAN #106. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A to support satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths. - Specify RRM requirements Note: Support for NTN CA is not in the scope of this WID

# 2 Discussion

According to the objective of the WID, there are two options for us to consider to define Ku band.

- Option 1: Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing.

- Option 2: Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing.

Figure 2.1-1. Normative work for Ku band #1a for all ITU regions 1 and 3

Figure 2.1-2. Normative work for Ku band #1b for all ITU regions 2

With regard to dividing Ku band to FR1 or FR2, we can refer to TR 38.820, a study item on 7.125–24.250 GHz frequency range, covering the regulatory framework study, general RF aspects, as well as BS- and UE-specific aspects. The following table is the split of frequency range 7–24 GHz. And in TR 38.820, it indicates that the frequency range 7.125-[10-13] GHz would have “FR1 like” requirements.

Table 1: Frequency sub-ranges and examples frequencies selection in 7 – 24 GHz range

| Frequency sub-range | Frequency (GHz) | Example frequency (GHz) |
| --- | --- | --- |
| 1 | 7.125 - fboundary, low | 10 |
| 2 | fboundary, low - fboundary, high | 15 |
| 3 | fboundary, high - 24.250 | 20 |
| Note 1: fboundary, low is within the frequency range 10 - 13 GHz. Note 2: fboundary, high is within the frequency range 16 - 18 GHz. |  |  |

Regarding the frequency range of Ku band, it is closer to FR1. However, channel bandwidths specified in current NTN specification only support 5, 10, 15, 20 MHz, and 30 MHz channel bandwidth only be introduced without defining related RF requirements. If we divide Ku band into FR1-NTN, we need to define 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths and related RF requirements.

Observation 1: The frequency range of Ku band is closer to FR1. However, channel bandwidths specified in current NTN specification only support 5, 10, 15, 20 MHz, and 30 MHz channel bandwidth only be introduced without defining related RF requirements.

# 3 Conclusion

In this contribution, we shared some views on system parameters for NTN Ku band and the proposal is made as follows:

Observation 1: The frequency range of Ku band is closer to FR1. However, channel bandwidths specified in current NTN specification only support 5, 10, 15, 20 MHz, and 30 MHz channel bandwidth only be introduced without defining related RF requirements.

# 4 Reference

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat et al.