3GPP TSG-RAN WG4 Meeting #112bis	    R4- 2416085

Hefei, China, 14th – 18th October 2024

Agenda item:	6.4.3

Source:	Intel Corporation

Title:	Discussion on the RRM requirements for Rel-19 less than 5MHz UE

Document for:	Discussion

# Introduction

The Rel-19 NR channel BW less than 5MHz for FR1 Phase 2 work item was updated in June 2024 in [1]. The work item Core part objectives are provided below.

- Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101 - Define RRM requirements for inter-band CA and DC for combinations introduced in RF part Note: other band combinations than example band combinations can be specified in basket WIs after the above generic requirements are specified.

The idea of the objectives is to first define generic RF requirements for inter-band NR-CA and NR-DC n100-n101 band combinations considering less than 5MHz bandwidth in FDD band n100 and 5 MHz or 10 MHz in TDD band n101, and specify general RF requirements/applicability for inter-band CA/DC. RRM work started from August WG meeting 2024.

In this paper we provide our analysis on the RRM requirements for the work item.

# RRM requirements

## Carrier aggregation with 12 PRB SSB bandwidth SCell

Regarding supporting carrier aggregation when one of the bands in the combination is configured with less than 5MHz channel bandwidth, we need to specify new UE requirements for SCell operations targeting an SCell with smaller SSB bandwidth than those in the legacy specifications.

In Rel-18, consensus was that the measurement performance is not impacted when the target SSB has smaller BW than 5MHz, but only the PBCH bandwidth was reduced. However, the detection delay of a cell with 12 PRB or 15 PRB bandwidth was relaxed, when SSB index detection is necessary in the process.

Time period requirements for time index detection on deactivated SCell with 12PRB SSB bandwidth

In the Rel-18 specification, the SSB index detection delay was relaxed in the below manner – comparison is shown in the below box between T_SSB_time_index_intra and T_SSB_time_index_intra_less_than_5Mhz.

Table 9.2.6.2-3: Time period for time index detection (Frequency range FR1) DRX cycle; TSSB_time_index_intra. No DRX; max(120ms, ceil(3 x Kgap ) x max(MGRP, SMTC period)) x CSSFintra. DRX cycle≤ 320ms; max(120ms, ceil(M2Note 1x 3 x Kgap) x max(MGRP, SMTC period,DRX cycle) x CSSFintra). DRX cycle>320ms; Ceil(3 x Kgap )x max(MGRP, DRX cycle) x CSSFintra. NOTE 1: When highSpeedMeasFlag-r16 is not configured, M2 = 1.5; When highSpeedMeasFlag-r16 is configured, M2 = 1.5 if SMTC periodicity > 40 ms, otherwise M2=1. NOTE 2: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16 on measurements of the primary component carrier and do not apply to measurements of a secondary component carrier with active SCell. NOTE 3: For a UE supporting concurrent GAPs, if multiple concurrent GAPs are configured, the MGRP is the periodicity of the activated Pre-MG or the MG pattern associated to the intra-frequency layer. NOTE 4: When highSpeedMeasCA-Scell-r17 is configured, the requirements apply to UE on measurements of secondary component carrier with active SCell.. Table 9.2.6.1-12: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1) (Frequency range FR1) DRX cycle; TSSB_time_index_intra_less_than_5Mhz. No DRX; max(120ms, 7 x max(MGRP, SMTC period)) x CSSFintra. DRX cycle≤ 320ms; max(120ms, ceil(1.5 x 7) x max(MGRP, SMTC period,DRX cycle) x CSSFintra). DRX cycle>320ms; 7 x max(MGRP, DRX cycle) x CSSFintra. Note 1: Void NOTE 2: FFS When highSpeedMeasInterFreq-r17.

Similarly, we need to specify the time period requirements for time index detection for UE operating on a target cell with less than 5MHz SSB for deactivated SCell measurements in FR1.

Note that for the corresponding requirements for intra-frequency measurement with NCSG, since there were no requirements specified in Rel-18 for target SSB with 12PRB bandwidth, we propose not to introduce the Rel-19 requirements for these scenarios. Regarding measurements with CCA and/or for a ATG UE, same situation applies.

Proposal 1: Specify time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated SCell in FR1 as in the following table.

Table 9.2.5.1-x: Time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated SCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(7 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(7 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(7 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

A company draftCR reflecting the changes is submitted in this meeting in [2].

Fine time tracking time on 12PRB SSB BW when UE carries out SCell activation

In the SCell activation delay requirements, fine time tracking after search of the target cell is considered and when the target SSB BW is smaller than 5MHz the legacy requirement does not apply. This is to say that we need to specify new requirements for fine time tracking time, which is included in the SCell activation delay requirements.

To help understand, fine time tracking time for less than 5MHz target HO requirements is copied in the below box.

When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Where: - If the target cell is an unknown inter-frequency cell and Tsearch = 3* Trs ms if the target cell Es/Iot≥-2 dB. T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW. T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

In the last WG meeting, we had the agreement to start from single SCell activation delay requirements.

Proposal 2: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the SCell activation delay requirements in FR1 in Rel-19.

Tactivation_time is the SCell activation delay in millisecond. If the SCell is known and belongs to FR1, Tactivation_time is: - TFirstSSB_MAX + 3 * Trs + 5ms, if the measurement period of the SCell being activated is larger than 2400ms and the UE supporting [12PRB SSB bandwidth UE feature] is configured with a target SCell that is with 12PRB SSB bandwidth.

If the SCell is unknown and belongs to FR1, and if one of the following conditions is met - ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or - ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation, - TFirstSSB_MAX + TSMTC_MAX + 4*Trs + 5ms, if the UE supporting [12PRB SSB bandwidth UE feature] is configured with a target SCell that is with 12PRB SSB bandwidth.

- Otherwise - 6ms + TFirstSSB_MAX + TSMTC_MAX + 3 * Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting, and the UE supporting [12PRB SSB bandwidth UE feature] is configured with a target SCell that is with 12PRB SSB bandwidth, - 3ms + TFirstSSB_MAX + TSMTC_MAX + 3 * Trs + TL1-RSRP, measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting, and the UE supporting [12PRB SSB bandwidth UE feature] is configured with a target SCell that is with 12PRB SSB bandwidth.

A company draftCR reflecting the changes is submitted in this meeting in [2].

## NR-DC with 12PRB SSB bandwidth PSCell

Time period requirements for time index detection on deactivated PSCell with less than 5MHz SSB bandwidth

The similar requirements as in the NR CA discussed in the above section are expected to be also introduced for NR-DC with a PSCell that has 12PRB SSB bandwidth.

Proposal 3: Specify time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated PSCell in FR1 as in the following table.

Table 9.2.5.1-y: Time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated PSCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(7 x Kp) x measCyclePSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(7 x Kp) x max(measCyclePSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(7 x Kp) x max(measCyclePSCell, DRX cycle) x CSSFintra |

PSCell addition and change delay where PSCell SSB is 12PRB bandwidth

In the scope of NR-DC when PSCell is configured with less than 5MHz channel bandwidth, RAN4 needs to specify new PSCell addition delay requirements. Similar to handover and SCell activation, the fine time tracking delay needs to be relaxed.

Proposal 4: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the PSCell addition delay requirements.

### 8.9.2 PSCell Addition Delay Requirement Upon receiving PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards PSCell in FR1 or FR2 no later than in slot n+Tconfig_PSCellNR slot length: where: T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms for a known or unknown PSCell. T∆ = 3*Trs ms for either known or unknown target PSCell operating with 12 PRB SSB BW.

Automatically, the PSCell change delay requirements follow the PSCell addition delay requirements.

Proposal 5: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the PSCell change delay requirements.

## 8.11 PSCell Change This clause defines requirements for the delay within which the UE shall be able to change PSCell to other cell in EN-DC or NR-DC. The requirements in this clause are applicable to EN-DC and NR-DC. In the case of NR-DC PSCell change in FR1, when the UE supporting [12PRB SSB bandwidth UE feature] is configured with a target PSCell that is with 12PRB SSB bandwidth, the UE shall transmit PRACH preamble towards the target PSCell no later than the delay requirements specified in clause 8.9.2.

A company draftCR reflecting the changes is submitted in this meeting in [2].

# SSB bandwidth information indications

## Allowing 12PRB SSB bandwidth off sync raster

In Rel-18 there were discussions whether to introduce network indication about the 12PRB SSB bandwidth for a target cell for measurements. Due to introduction of a new 12 PRB SSB design, for an unknown target cell case the UE has no clear information about the target SSB bandwidth. The conclusion in Rel-18 was that there is no need for an indication because 12PRB SSB is limited to PCell and the UE can derive the SSB bandwidth based on the configured frequency position of the SSB (i.e., derive based on GSCN). For PCell the SSB is always configured on the sync raster, which is clearly defined in the RF specifications. As long as the configured SSB frequency position is aligned with the sync raster for 12PRB SSB, the UE uses 12PRB bandwidth to receive the SSB thus measurement performance is guaranteed, and requirements can be fulfilled.

In Rel-19 we aim to allow 12PRB SSB bandwidth configurations for SCells and PSCells. The situation changes since there is no guarantee that the SSB should align with the sync raster. On the condition that the 12PRB SSB is placed off the sync raster, there is no way for the UE to identify the correct SSB bandwidth according to the current specifications in 3GPP RAN.

Proposal 6: RAN4 allows that 12 PRB SSB can be transmitted off sync raster.

UE needs clear information on SSB bandwidth for different procedures including UE measurements, handover and SCell addition for an unknown target cell.

Measurements on a 12PRB SSB according to SSB bandwidth indication in the Measurement Object

According to RAN2 specifications, Measurement Object configuration is the correct place to indicate from the network side to the UE about the target SSB bandwidth for RRM measurements.

In the container of MeasObjectNR, there is an IE of referenceSignalConfig which contains the SSB-ConfigMobility property. Below we propose to add the SSB bandwidth information in SSB-ConfigMobility property item (either option serves the purpose, and it can be eventually decided by RAN2).

SSB-ConfigMobility::= SEQUENCE { ssb-ToMeasure SetupRelease { SSB-ToMeasure } OPTIONAL, -- Need M deriveSSB-IndexFromCell BOOLEAN, ss-RSSI-Measurement SS-RSSI-Measurement OPTIONAL, -- Need M ..., [[ ssb-PositionQCL-Common-r16 SSB-PositionQCL-Relation-r16 OPTIONAL, -- Cond SharedSpectrum ssb-PositionQCL-CellsToAddModList-r16 SSB-PositionQCL-CellsToAddModList-r16 OPTIONAL, -- Need N ssb-PositionQCL-CellsToRemoveList-r16 PCI-List OPTIONAL -- Need N ]], [[ deriveSSB-IndexFromCellInter-r17 ServCellIndex OPTIONAL, -- Need R ssb-PositionQCL-Common-r17 SSB-PositionQCL-Relation-r17 OPTIONAL, -- Cond SharedSpectrum2 ssb-PositionQCL-Cells-r17 SetupRelease {SSB-PositionQCL-CellList-r17} OPTIONAL -- Need M ]], [[ cca-CellsToAddModList-r17 PCI-List OPTIONAL, -- Need N cca-CellsToRemoveList-r17 PCI-List OPTIONAL -- Need N ]], [[ ssb-ToMeasureAltitudeBasedList-r18 SetupRelease { SSB-ToMeasureAltitudeBasedList-r18 } OPTIONAL -- Need M ]] SSB-12PRB BOOLEAN, SSBbandwidthinPRB ENUMERATED {12, 20} OPTIONAL, -- Need R }

Proposal 7: Introduce network signalling in the MO to facilitate UE knowledge about the unknown target SSB bandwidth.

With the SSB bandwidth indication introduced, the UE measurement performance is guaranteed for RRC connected mode RRM measurements targeting any cell that sends 12 PRB bandwidth SSB. For a UE that supports Rel-19 12 PRB SSB bandwidth measurements in RRC connected mode, the UE follows the new requirements introduced in Rel-19, when the UE is configured with 12 PRB bandwidth SSB in the measurement object configurations.

In the performance part of this item, we can introduce a new test case for measurements on target cell with 12 PRB SSB bandwidth off sync raster to test the UE ability to use the message conducted in the MO configurations.

Handover and SCell addition for a cell that has 12PRB SSB bandwidth

Regarding handover and SCell addition procedures, there is no obvious need to introduce SSB bandwidth indication.

For handover command reconfigurationwithsync, there is indication in servingcellcommon about the target carrier bandwidth in IE carrierBandwidth. For SCell addition command RRCreconfiguration, there is also indication in servingcellcommon about the target carrier bandwidth.

As long as the UE knows the carrier bandwidth of the target cell, the UE is able to derive the SSB bandwidth by the following approach:

- Carrier bandwidth smaller than 20 PRB corresponds to 12 PRB SSB

- Others carrier bandwidth correspond to 20 PRB SSB

With this information provided to UE, the UE is able to correctly follow the requirements we specified in Rel-18 and Rel-19.

Proposal 8: No need to introduce new indication of SSB bandwidth in HO command or in SCell addition command.

Idle mode measurements

Neighbour cell information for idle mode measurements is configured in SIB4 InterFreqCarrierFreqInfo. In Rel-18 dl-CarrierFreq-r18 was introduced for the Rel-18 12PRB SSB capable UE so that when the capable UE reads CarrierFreq-r18 in the InterFreqCarrierFreqInfo-v1800 container in SIB4, the UE is aware whether the target SSB is with 12PRB bandwidth by whether the SSB is on the 12PRB SSB sync raster. Meanwhile the counterpart IE in the original container for Rel-15 has to be set to a unique value to make sure that the legacy UE does not reselect to the 12PRB SSB cell.

dl-CarrierFreq This field indicates center frequency of the SS block of the neighbour cells, where the frequency corresponds to a GSCN value as specified in TS 38.101-1 [15] or TS 38.101-5 [75]. For a neighbouring carrier frequency when dl-CarrierFreq-r18 is included, the network sets the corresponding value of dl-CarrierFreq (without suffix) to 250, and the UE applies dl-CarrierFreq-r18 instead of dl-CarrierFreq (without suffix). In such case, if the UE does not support the GSCN value corresponding to the dl-CarrierFreq-r18, it ignores the corresponding neighbour cell.

In Rel-19 the 12PRB SSB can be off sync raster, but when the SSB is sent off sync raster the UE is not supposed to reselect itself to the cell according to that the IE only corresponds to an SSB that is transmitted on the sync raster. It is straightforward since the UE is not supposed to be reselected to a ‘SCell only’ cell.

Proposal 9: No need to introduce indications in SIB4 message for IDLE mode measurements.

## LS to RAN2

According to the discussions in the above section, we think it is necessary to send an LS to RAN2 about the introduction of SSB bandwidth indication in the MO message.

In the annex of this paper we propose the draft LS to RAN2 about introducing the indications.

Note that the core part of the work item is supposed to be closed in Dec. RANP meeting so sending LS in this meeting is crucial to the progress of this item.

Proposal 10: Send an LS to RAN2 about introducing SSB bandwidth indications in the MO configurations.

# Conclusion

In this paper we provide our analysis on the RRM requirements for the work item.

Proposal 1: Specify time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated SCell in FR1 as in the following table.

Proposal 2: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the SCell activation delay requirements in FR1 in Rel-19.

Proposal 3: Specify time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated PSCell in FR1 as in the following table.

Table 9.2.5.1-y: Time period for time index detection for a UE operating on a target cell with 12PRB SSB, deactivated PSCell (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(7 x Kp) x measCyclePSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(7 x Kp) x max(measCyclePSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(7 x Kp) x max(measCyclePSCell, DRX cycle) x CSSFintra |

Proposal 4: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the PSCell addition delay requirements.

Proposal 5: Specify fine time tracking time equals 3 SSB periods instead of 1 when the target cell is with 12PRB SSB bandwidth, as is included in the PSCell change delay requirements.

Proposal 6: RAN4 allows that 12 PRB SSB can be transmitted off sync raster.

Proposal 7: Introduce network signalling in the MO to facilitate UE knowledge about the unknown target SSB bandwidth.

Proposal 8: No need to introduce new indication of SSB bandwidth in HO command or in SCell addition command.

Proposal 9: No need to introduce indications in SIB4 message for IDLE mode measurements.

Proposal 10: Send an LS to RAN2 about introducing SSB bandwidth indications in the MO configurations.

# References

- RP-241622, Revised WID: NR channel BW less than 5MHz for FR1 Phase 2, Intel Corporation

- R4-2416086, Draft Big CR for less than 5MHz RRM requirements in Rel-19, Intel Corporation

# Annex

3GPP TSG RAN WG4 Meeting #112bis		TDoc

Hefei, China, 14th – 18th October 2024

Title:	LS on network indication of SSB bandwidth for Rel-19 less than 5MHz

Response to:

Release:	Rel-19

Work Item:	NR_FR1_lessthan_5MHz_BW_Ph2

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Zhang, Meng

meng.zhang@intel.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# Overall description

In Rel-19 NR_FR1_lessthan_5MHz_BW_Ph2 WI RAN4 agreed to allow 12 PRB SSB bandwidth configurations for SCells and PSCells. Furthermore, RAN4 reached an agreement to allow 12 PRB SSB transmissions off sync raster.

For a target cell which the UE is configured to measure on 12 PRB bandwidth SSB, the UE needs to be able to obtain clear information of the target SSB bandwidth, so that it can fulfil the correct set of requirements. It is RAN4 understanding that under the condition that the 12 PRB SSB is placed off the sync raster, that there is no way for the UE to get information on the SSB bandwidth according to the current specifications.

RAN4 kindly asks RAN2 to check RAN4 understanding, and, if needed, introduce signalling to SSB bandwidth indication for 12/20PRB SSB bandwidth as a part of Measurement Object (MO) configuration signaling.

# Actions

To RAN2

ACTION: 	RAN4 kindly asks RAN2 to check RAN4 understanding that UE is unaware on target cell SSB bandwidth for the when SSB is not placed on the frequency position corresponding to the synchronization raster, and, if needed, introduce signalling to SSB bandwidth indication for 12/20PRB SSB bandwidth as a part of Measurement Object (MO) configuration signaling.

# Dates of next TSG RAN WG4 meetings

3GPP RAN WG4 #113		18th – 22nd November 2024		Orlando, USA

3GPP RAN WG4 #114		17th – 21st February 2025			Athens, Greece