# 3GPP TSG-RAN WG4 Meeting #114	R4-2500271

Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.20.3

Source: 	OPPO

Title: 	Testing issues for CSI compression two-sided models

Document for:	Approval

- Discussion

For a two-sided AI/ML model, the information transmitted through air-interface (e.g. PMI) is generated by AI/ML models (e.g. CSI encoder), rather than codebooks predefined by protocol. Regarding the testability of two-sided model, it is necessary to consider bring in a test model to cooperate with the model under test.

In the previous RAN4 meetings, 4 options on test decoder have been proposed and discussed, i.e.,

- Option 1: test decoder is provided by the vendor of the encoder

- Option 2: test decoder is provided by the vendor of the decoder

- Option 3: The test decoder(s) are fully specified and captured in RAN4 spec.

- Option 4: The test decoder(s) are partially specified and captured in RAN4 spec.

In RAN4 #110 meeting, RAN4 agree to further discuss only options 3 and 4, i.e., fully or partially specify test decoder(s).

From our understanding, to define a test decoder in specification, following aspects should be taken into account.

-  Principles to define test decoder(s)

- To meet the minimum performance requirement

The test model is used to meet the minimum performance requirement in RAN4 tests, not directly to be used for commercial utilization and deployment.

- To be a simple design

As a test model to be specified in RAN4 spec, we may have a lot of proposals from different companies. How to do the merge and compromise among different designs and views from different companies would be a big challenge and a tough work.

From the beginning, we can try to design the test model(s) as simple as possible, to facilitate calibration and promote consistency among different companies.

Proposal 1: Principles to define test decoder(s)

- to meet the minimum performance requirement in RAN4 tests

- to be a simple design

-  CSI encoder and CSI decoder

CSI encoder and CSI decoder that proposed after RAN4 #111 email discussion are shown as below:

CNN based CSI encoder 								CNN based CSI decoder

Figure 1  CSI encoder and CSI decoder that proposed after RAN4 #111 email discussion

Observation 1: FLOPs, Number of parameters, model size for CNN based CSI encoder and CNN based CSI decoder are evaluated as below:

| CSI feedback bits | FLOPs |  | Num. of Trainable Parameters |  | Model Size |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Encoder | Decoder | Encoder | Decoder | Encoder | Decoder |
| 64 bits | 94.1M | 1481.6M | 0.13M | 1.80M | 638KB | 7.03MB |
| 112 bits | 94.2M | 1481.7M | 0.14M | 1.82M | 677KB | 7.11MB |

Observation 2: Based on the CNN based CSI encoder and CNN based CSI decoder proposed by RAN4#111, performance evaluation results are shown as below(train by UMa channel data, test by different channel model)

| CSI feedback bits | Training dataset | Test dataset (SGCS) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TDL-A30 | CDL-A30 | CDL-C30 | UMa |
| 64 bits | UMa | 0.759 | 0.705 | 0.603 | 0.725 |
| 112 bits | UMa | 0.784 | 0.768 | 0.645 | 0.782 |

Note：Baseline etype2 SGCS is 0.71

However, even after adopting the same model structure and the same simulation assumptions (based on RAN4 conclusion), different companies still obtained different simulation results(e.g., in SGCS), as summarized in RAN4#112 meeting.

Here, we first want to clarify an issue: for RAN4 test model alignment, what exactly needs to be aligned? Is it expected that the alignment of minimum performance understanding can be achieved through results from different companies? Or is it expected to strictly align the output results of each company's AI/ML models through simulation?

Proposal 2: For RAN4 test model alignment, the motivation should be further clarified

- Option1: It is expected that the alignment of minimum performance understanding can be achieved through results from different companies

- Option2: It is expected to strictly align the output results of each company's AI/ML models through simulation

For option2 (strict alignment of each company's model output), from our understanding, it is difficult to achieve by simply unifying the settings of some parameters (without strict calibration alignment of the simulation platform among different companies).

As for the first type of alignment, companies can relatively judge and reach a consensus on minimum performance understanding. On one hand, considering the impact of different datasets and model training schemes on test metrics, and on the other hand, serving as a reference basis for formulating RAN4 test performance metrics.

In RAN4#112 meeting, companies agreed to perform a second round of alignment and refined more simulation parameters as captured in R4-2414447. A second-round check could be done at the RAN4#112bis meeting. Simulation results from our side are as follows:

Observation 3: According to the refined simulation parameters in R4-2414447, corresponded second-round alignment results could be found as below:

| CSI feedback bits | Quantization of encoder output | SGCS (UMa channel) |
| --- | --- | --- |
| 64 bits | 2 bits quantization for CSI report | 0.717 |
| 32 floats | without quantization of the FC layer outputs for validation purposes only, i.e., float32 is sent in CSI report | 0.771 |

In RAN4#112bis meeting, RAN4 agreed that additional information relates to “CDF of SGCS per subband” could be further evaluated.

Observation 4: Simulation results on “CDF of SGCS per subband” are shown as follows:

| Subband index | SGCS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Average | 10% | 20% | 30% | 40% | 50% | 60% | 70% | 80% | 90% |
| 0 | 0.689 | 0.319 | 0.481 | 0.588 | 0.671 | 0.742 | 0.804 | 0.861 | 0.918 | 0.967 |
| 1 | 0.718 | 0.389 | 0.540 | 0.635 | 0.708 | 0.769 | 0.823 | 0.873 | 0.922 | 0.968 |
| 2 | 0.731 | 0.415 | 0.564 | 0.659 | 0.727 | 0.783 | 0.833 | 0.879 | 0.924 | 0.969 |
| 3 | 0.732 | 0.411 | 0.566 | 0.658 | 0.728 | 0.785 | 0.834 | 0.881 | 0.926 | 0.969 |
| 4 | 0.723 | 0.385 | 0.547 | 0.645 | 0.718 | 0.779 | 0.832 | 0.880 | 0.927 | 0.969 |
| 5 | 0.713 | 0.359 | 0.525 | 0.630 | 0.708 | 0.772 | 0.829 | 0.878 | 0.926 | 0.970 |
| 6 | 0.711 | 0.351 | 0.520 | 0.627 | 0.704 | 0.770 | 0.828 | 0.878 | 0.927 | 0.970 |
| 7 | 0.716 | 0.362 | 0.529 | 0.634 | 0.709 | 0.774 | 0.830 | 0.881 | 0.926 | 0.969 |
| 8 | 0.724 | 0.390 | 0.545 | 0.646 | 0.719 | 0.781 | 0.834 | 0.881 | 0.926 | 0.969 |
| 9 | 0.732 | 0.411 | 0.565 | 0.658 | 0.728 | 0.785 | 0.836 | 0.881 | 0.926 | 0.969 |
| 10 | 0.732 | 0.418 | 0.565 | 0.657 | 0.725 | 0.783 | 0.833 | 0.879 | 0.924 | 0.968 |
| 11 | 0.717 | 0.385 | 0.536 | 0.634 | 0.708 | 0.769 | 0.822 | 0.871 | 0.921 | 0.968 |
| 12 | 0.687 | 0.312 | 0.475 | 0.585 | 0.669 | 0.741 | 0.805 | 0.861 | 0.917 | 0.967 |

In RAN4 #113 meeting, following 2 tracks have been agreed to further evaluate option 3.

Track 1: Select 3 decoders at RAN4#113. Companies train own encoders (with own Eigenvector dataset) and check performance against decoders (with at least own test dataset)

Track 2: Create a mixed dataset at RAN4#113. By RAN4#114, companies train decoders based on the mixed dataset. At RAN4#114, one or more decoder selected and after RAN4#114, companies develop encoders and check encoder performance with own dataset checked against decoder(s).

For "Track 1", the selected companies are Samsung, CATT, and Huawei. We utilized the CSI decoder provided by Samsung (without quantization) as the test decoder and conducted encoder training using our own dataset. The results obtained are as follows:

| CSI Feedback | Encoder | Decoder | Test data | SGCS (UMa channel) |
| --- | --- | --- | --- | --- |
| 32 floats | OPPO (without quantization) | OPPO (without quantization) | OPPO | 0.771 |
| 32 floats | Samsung (without quantization) | Samsung (without quantization) | OPPO | 0.757 |
| 32 floats | OPPO retrain by OPPO dataset (without quantization) | Samsung (without quantization) | OPPO | 0.760 |

Observation 5: Simulation results for "Track 1" are shown as below:

| CSI Feedback | Encoder | Decoder | Test data | SGCS (UMa channel) |
| --- | --- | --- | --- | --- |
| 32 floats | OPPO (without quantization) | OPPO (without quantization) | OPPO | 0.771 |
| 32 floats | Samsung (without quantization) | Samsung (without quantization) | OPPO | 0.757 |
| 32 floats | OPPO - retrain by OPPO dataset (without quantization) | Samsung (without quantization) | OPPO | 0.760 |

For "Track 2", we trained both the encoder and decoder using a mixed dataset (with RAN4 agreed model structure). Additionally, we employed the decoder trained on the mixed dataset as the test decoder and retrained the encoder using OPPO own dataset. The results from these tests are presented below:

| CSI Feedback | Encoder | Decoder | Test data | SGCS (UMa channel) |
| --- | --- | --- | --- | --- |
| 32 floats | OPPO - train by mix dataset (without quantization) | OPPO - train by mix dataset (without quantization) | - by mix dataset - by OPPO dataset | 0.752(by mix dataset) 0.750(by OPPO dataset) |
| 32 floats | OPPO - retrain by OPPO dataset (without quantization) | OPPO - train by mix dataset (without quantization) | - by mix dataset - by OPPO dataset | 0.686(by mix dataset) 0.751(by OPPO dataset) |

Observation 6: Simulation results for "Track 2" are shown as below:

| CSI Feedback | Encoder | Decoder | Test data | SGCS (UMa channel) |
| --- | --- | --- | --- | --- |
| 32 floats | OPPO - train by mix dataset (without quantization) | OPPO - train by mix dataset (without quantization) | - by mix dataset - by OPPO dataset | 0.752(by mix dataset) 0.750(by OPPO dataset) |
| 32 floats | OPPO - retrain by OPPO dataset (without quantization) | OPPO - train by mix dataset (without quantization) | - by mix dataset - by OPPO dataset | 0.686(by mix dataset) 0.751(by OPPO dataset) |

-  Challenges of model conversion (ONNX to other format)

The ONNX (Open Neural Network Exchange) format has been designed to facilitate the interoperability of deep learning models across different frameworks. However, in practice(during track1/2 works), there are several challenges that can prevent a seamless conversion of ONNX models to other formats, e.g., TensorFlow or PyTorch. Some main issues are listed below:

- Lack of official conversation tools: The available ONNX to TensorFlow or PyTorch conversion tools, which are developed by the open-source community rather than officially released, may have significant limitations in their implementation and maintenance. For example, the onnx-tensorflow library, as one of the most commonly used and popular tools for converting models in ONNX format to TensorFlow format, has declared its code repository is not actively maintained and will be deprecated since two years ago.

- Version Compatibility Issues: Different versions of ONNX introduce new features and changes to operators. The conversion tools available for converting models in ONNX to TensorFlow or PyTorch format may not be fully updated to support the latest ONNX version. Similarly, TensorFlow and PyTorch themselves undergo continuous development and version updates, and changes in their internal architectures or APIs can impact how well they can import models which converted from ONNX format.

- Conversion for custom or unsupported layers: If a model in ONNX format contains custom layers that are designed specifically for the original framework in which it was developed, there may be no straightforward way to translate them to TensorFlow or PyTorch equivalents. These custom layers might encapsulate unique functionality or algorithms that are not natively available in the target frameworks, making it difficult to accurately represent the model during the conversion.

If we intend to use ONNX as the method for providing the test model, the aforementioned issues should be carefully considered and resolved. Relying on offline negotiation to address these problems has proven to be highly inefficient at present.

An alternative approach would be to reconsider the format in which the test model is delivered, such as providing it in PyTorch format, thereby relax the challenges associated with version and format conversion.

Observation 7: Several challenges that can prevent a seamless conversion of ONNX models to other formats, e.g., TensorFlow or PyTorch. Some main issues are listed below:

      - Lack of official conversation tools

      - Version Compatibility Issues

      - Conversion for custom or unsupported layers

Proposal 3: To reconsider the format in which the test model is delivered, such as providing it in PyTorch format, thereby relax the challenges associated with version and format conversion.

- Conclusions

In this contribution, we have discussed the testing issues for CSI compression two-sided models and got the following proposals:

Proposal 1: Principles to define test decoder(s)

- to meet the minimum performance requirement in RAN4 tests

- to be a simple design

Observation 1: FLOPs, Number of parameters, model size for CNN based CSI encoder and CNN based CSI decoder are evaluated as below:

| CSI feedback bits | FLOPs |  | Num. of Trainable Parameters |  | Model Size |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Encoder | Decoder | Encoder | Decoder | Encoder | Decoder |
| 64 bits | 94.1M | 1481.6M | 0.13M | 1.80M | 638KB | 7.03MB |
| 112 bits | 94.2M | 1481.7M | 0.14M | 1.82M | 677KB | 7.11MB |

Observation 2: Based on the CNN based CSI encoder and CNN based CSI decoder proposed by RAN4#111, performance evaluation results are shown as below(train by UMa channel data, test by different channel model)

| CSI feedback bits | Training dataset | Test dataset (SGCS) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TDL-A30 | CDL-A30 | CDL-C30 | UMa |
| 64 bits | UMa | 0.759 | 0.705 | 0.603 | 0.725 |
| 112 bits | UMa | 0.784 | 0.768 | 0.645 | 0.782 |

Note：Baseline etype2 SGCS is 0.71

Proposal 2: For RAN4 test model alignment, the motivation should be clarified

- Option1: It is expected that the alignment of minimum performance understanding can be achieved through results from different companies

- Option2: It is expected to strictly align the output results of each company's AI/ML models through simulation

Observation 3: According to the refined simulation parameters in R4-2414447, corresponded second-round alignment results could be found as below:

| CSI feedback bits | Quantization of encoder output | SGCS (UMa channel) |
| --- | --- | --- |
| 64 bits | 2 bits quantization for CSI report | 0.717 |
| 32 floats | without quantization of the FC layer outputs for validation purposes only, i.e., float32 is sent in CSI report | 0.771 |

Observation 4: Simulation results on “CDF of SGCS per subband” are shown as follows:

| Subband index | SGCS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Average | 10% | 20% | 30% | 40% | 50% | 60% | 70% | 80% | 90% |
| 0 | 0.689 | 0.319 | 0.481 | 0.588 | 0.671 | 0.742 | 0.804 | 0.861 | 0.918 | 0.967 |
| 1 | 0.718 | 0.389 | 0.540 | 0.635 | 0.708 | 0.769 | 0.823 | 0.873 | 0.922 | 0.968 |
| 2 | 0.731 | 0.415 | 0.564 | 0.659 | 0.727 | 0.783 | 0.833 | 0.879 | 0.924 | 0.969 |
| 3 | 0.732 | 0.411 | 0.566 | 0.658 | 0.728 | 0.785 | 0.834 | 0.881 | 0.926 | 0.969 |
| 4 | 0.723 | 0.385 | 0.547 | 0.645 | 0.718 | 0.779 | 0.832 | 0.880 | 0.927 | 0.969 |
| 5 | 0.713 | 0.359 | 0.525 | 0.630 | 0.708 | 0.772 | 0.829 | 0.878 | 0.926 | 0.970 |
| 6 | 0.711 | 0.351 | 0.520 | 0.627 | 0.704 | 0.770 | 0.828 | 0.878 | 0.927 | 0.970 |
| 7 | 0.716 | 0.362 | 0.529 | 0.634 | 0.709 | 0.774 | 0.830 | 0.881 | 0.926 | 0.969 |
| 8 | 0.724 | 0.390 | 0.545 | 0.646 | 0.719 | 0.781 | 0.834 | 0.881 | 0.926 | 0.969 |
| 9 | 0.732 | 0.411 | 0.565 | 0.658 | 0.728 | 0.785 | 0.836 | 0.881 | 0.926 | 0.969 |
| 10 | 0.732 | 0.418 | 0.565 | 0.657 | 0.725 | 0.783 | 0.833 | 0.879 | 0.924 | 0.968 |
| 11 | 0.717 | 0.385 | 0.536 | 0.634 | 0.708 | 0.769 | 0.822 | 0.871 | 0.921 | 0.968 |
| 12 | 0.687 | 0.312 | 0.475 | 0.585 | 0.669 | 0.741 | 0.805 | 0.861 | 0.917 | 0.967 |

Observation 5: Simulation results for "Track 1" are shown as below:

| CSI Feedback | Encoder | Decoder | Test data | SGCS (UMa channel) |
| --- | --- | --- | --- | --- |
| 32 floats | OPPO (without quantization) | OPPO (without quantization) | OPPO | 0.771 |
| 32 floats | Samsung (without quantization) | Samsung (without quantization) | OPPO | 0.757 |
| 32 floats | OPPO - retrain by OPPO dataset (without quantization) | Samsung (without quantization) | OPPO | 0.760 |

Observation 6: Simulation results for "Track 2" are shown as below:

| CSI Feedback | Encoder | Decoder | Test data | SGCS (UMa channel) |
| --- | --- | --- | --- | --- |
| 32 floats | OPPO - train by mix dataset (without quantization) | OPPO - train by mix dataset (without quantization) | - by mix dataset - by OPPO dataset | 0.752(by mix dataset) 0.750(by OPPO dataset) |
| 32 floats | OPPO - retrain by OPPO dataset (without quantization) | OPPO - train by mix dataset (without quantization) | - by mix dataset - by OPPO dataset | 0.686(by mix dataset) 0.751(by OPPO dataset) |

Observation 7: Several challenges that can prevent a seamless conversion of ONNX models to other formats, e.g., TensorFlow or PyTorch. Some main issues are listed below:

      - Lack of official conversation tools

      - Version Compatibility Issues

      - Conversion for custom or unsupported layers

Proposal 3: To reconsider the format in which the test model is delivered, such as providing it in PyTorch format, thereby relax the challenges associated with version and format conversion.

- References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface