3GPP TSG-RAN WG4 Meeting #114				                                            R4-2500636

Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.25.2.2

Title: 	Discussion on RRM requirements for adaptation of common signal/channel in NES

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In RANP #103 meeting, the WID [1,RP-240170] for R19 NES was agreed and one of the objectives below was discussed in the last meeting [4, R4-2416860].

- Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Study adaptation of PRACH in spatial domain, e.g. non-uniform PRACH resources per SSB, and specify if found beneficial - This study is to be done in 2Q’2024 only - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

In this meeting, we provide our further considers on open issues.

- Discussion

Firstly, the agreements made in RAN1 on adaptation mechanism of SSB and scenarios was summarized.

R1#116 Agreement Agreement For adaptation of SSB in time-domain, consider the following adaptation mechanisms for further study - Adaptation of SSB burst periodicity - Adaptation based on two SSB configurations where up to two configurations can be active - Adaptation based on skipping/transmitting some SSB bursts non-uniformly with single SSB configuration - Adapting the transmitted number of SSBs within a SSB burst - Cell DTX for SSB adaptation - Whether to support new SSB burst periodicity value(s) - Whether to support new SSB burst(s) (i.e. how SSB transmission is made within a burst) - New compact SSB burst(s) - Adapting the position of SSBs within a SSB burst - Other mechanisms/combinations are not precluded Agreement For the adaptation mechanisms of SSB in time-domain, study further following mechanisms: - Adaptation mechanism indicated or configured by gNB without UE trigger - Adaptation triggered by UE (if any) FFS: Details of associated signaling/indication/configuration R1#116bis Agreement Agreement For indication of adaptation of SSB in time-domain, - Support at least SSB adaptation provided by gNB without UE trigger Agreement Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s): - For cell with both legacy UEs and Rel-19 NES-capable UEs - Rel-19 NES-capable UE’s PCell (Connected mode) - Study from the following options: - Option A1: adaptation for CD-SSB - Option A2: adaptation for SSB that is not CD-SSB - Option A3: adaptation for SSB not on sync raster - Rel-19 NES-capable UE’s SCell - Study from the following options: - Option B1: adaptation for CD-SSB - Option B2: adaptation for SSB that is not CD-SSB - Option B3: adaptation for SSB not on sync raster - FFS: Rel-19 NES-capable UE in idle/inactive mode - Note: Impact to idle/inactive UEs shall be minimized R1#117 Agreement Agreement For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB R1#118bis Agreement Conclusion There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19 R1#119 Agreement Agreement Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?): - RAN1 agreed that at least SSB burst periodicity is adapted. - There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options. Agreement Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?): - The frequency location is same before and after SSB adaptation. Agreement Reply to Q3(What is the spatial relation before and after SSB adaptation?): - There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation. - At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported. Further update to be made based on RAN1#119 progress if any. Conclusion There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode)

  - L1 requirements impacts

In the last RAN4 meeting, L1 requirements due to the adaptation SSB transition was discussed. The following agreements was achieved.

I Issue 1-2-1: L1 requirements at transition Agreement: - L1 evaluation/measurement period is the maximum of the period corresponding to the SSB before and after adaptation. - FFS the applicable L1 requirements. Issue 1-2-2: L1 requirements subsequent to “transition period” Agreement: - Subsequent to transition period, requirements apply based on the SSB periodicity after transition for SSB adaptation of periodicity. - Note: The transition period is discussed in other issues.

In our understanding, RAN4 would define L1 measurement requirements in which SSBs both before and after adaptation would be measured. For an example, given the current the L1-RSRP measurement requirements in TS38.133 below, during a successful measurement reporting period the L1-RSRP measurement requirements can be defined as regardless DRX :

M*P* max⁡(Tcycle0,  Tcycle1 )      (1)

Wherein, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise.

Observation 1: In case of M=1 and P=1 (no gap sharing), only single SSB need to be considered within a successful measurement period. That is L1 measurement requirements at SSB transition defined by the equation (1) is not applicable.

Proposal 1:  L1 measurement requirement at transition agreed could not applicable for L1 measurement when M=1 and P=1.

Furthermore, given UE processing time on NW’s signaling, there are two possible scenarios.

- Scenario A: UE’s processing time for SSB adaptation signaling (DCI/MAC) is shorter than the interval (Ttrans ) between NW’s signaling of SSB adaptation and next latest available SSB with the updated SSB time domain parameter (e.g. periodicity)

- Scenario B: UE’s processing time for SSB adaptation signaling (DCI/MAC) is larger than the interval (Ttrans ) between NW’s signaling of SSB adaptation and next latest available SSB with the updated SSB time domain parameter (e.g. periodicity)

For the Scenario A, as illustrated in Figure 1 below, UE can know the exact SSB transmitted from gNB with after the adaptation. That is UE can meet the requirements with the maximum of the period corresponding to the SSB before and after adaptation.

Figure 1. UE processing time for DCI less than the transition time

However, for Scenario B as illustrated in Figure 2a and 2b below, in comparison with Scenario A above UE can’t know the exact SSB transmitted from gNB with as soon as the adaptation. That is at least one of SSB will be missed by UE due to the longer necessary signaling processing time in UE side. Especially for L1 measurement with signal sample required, one sample missed could result in the measurement or evaluation failure.

Figure 2a. Adaptation SSBs with the different periodicity only

Figure 2b. Adaptation SSBs with the different periodicity and time offset

Observation 2: the transition interval between gNB signaling on the SSB adaptation and next latest SSB could significantly impact on L1/L3 measurement in case of adaptation SSB.

For an example, in Figure 1b UE must be failure to meet the requirements during Tout window because UE assumed the SSB before adaptation to be measured.

From RAN4 perspective, in order to avoid the impacts from the missed SSB counted as the requirement, one of possible way is to redefine or clarify the measurement start point of L1/L3 measurement requirements when adaptation SSB occurred. For an example, in Figure 3, the two options to define the UE measurement start point.

- Alt. A: the start point is the next latest SSB

- Alt B: the start point is the next latest SSB after UE successfully received adaptation signalling

Figure 3. Possible way to define the measurement start point

Proposal 2: For L1/L3 measurement under SSB adaption, the start point of measurement period shall be clarified.

Alternatively, RAN4 can also clarified the applicable conditions for L1/L3 measurement requirement defined with the way agreed in the last meeting. For an example, the requirements can applicable to Option A only.

Therefore, we can propose that:

Proposal 2a: When defining the requirement with adaptation SSB, RAN4 needs only consider the case in which there is no any SSB adaptation period changed within one reporting interval.

  - L3 measurement

From RAN4 RRM perspective, in legacy RRM requirement, both intra and inter-frequency measurements requirement in TS38.133 section 9.2 and 9.3 can be summarized as:

Intra-frequency measurements: Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms Tidentify_intra_with_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra) ms Table 9.2.5.2-1: Measurement period for intra-frequency measurements without gaps (FR1) DRX cycle; T SSB_measurement_period_intra. No DRX; max(200ms, ceil( 5 x Kp) x SMTC period)Note 1 x CSSFintra. DRX cycle≤ 320ms; max(200ms, ceil(1.5x 5 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra. DRX cycle>320ms; ceil( 5 x Kp ) x DRX cycle x CSSFintra. NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified. Inter-frequency measurements: Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter) ms Tidentify_inter_with_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter + TSSB_time_index_inter) ms

Observation 3: In current spec, the total L3 measurement delay requirements depend on the SMTC periodicity which is configured by NW RRC message can be reused for L1/L3 measurement

Thus, UE can follow the indicated SSB adaptation was indicated by NW RRC message or other SIB message as well.

Observation 4: UE’s measurement requirements can be little impacted because SMTC period in TPSS/SSS_sync and T SSB_measurement_period can be explicitly indicated by NW.

Especially as agreed in the last meeting, RAN1 agreed the adaptation SSB could not used for PCell sync raster. In our view, that is the corresponding SSB based measurement requirements in clause 9 of TS38.133 shall be clarified because they can applicable for both PCell and SCell without any explicit indication (e.g. Table 9.2.5.2-3: for measurement on deactivated SCell).

R1#118 Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

However, since there is no agreements from RAN2 on new SMTC, we are also fine to rediscuss this issue upon further RAN2 agreements. This is algin with the previous RAN4 agreements.

Proposal 3: L3 measurement requirements with the adaptation SSB can be FFS upon LS reply on whether there is impact on SMTC.

- Conclusion

In this contribution, serval issues related to RRM in Rel19 NES are discussed. The observations and proposals can be summarized as:

Observation 1: In case of M=1 and P=1 (no gap sharing), only single SSB need to be considered within a successful measurement period. That is L1 measurement requirements at SSB transition defined by the equation (1) is not applicable.

Proposal 1:  L1 measurement requirement at transition agreed could not applicable for L1 measurement when M=1 and P=1.

Observation 2: the transition interval between gNB signaling on the SSB adaptation and next latest SSB could significantly impact on L1/L3 measurement in case of adaptation SSB.

Proposal 2: For L1/L3 measurement under SSB adaption, the start point of measurement period shall be clarified.

Proposal 2a: When defining the requirement with adaptation SSB, RAN4 needs only consider the case in which there is no any SSB adaptation period changed within one reporting interval.

Observation 3: In current spec, the total L3 measurement delay requirements depend on the SMTC periodicity which is configured by NW RRC message can be reused for L1/L3 measurement

Observation 4: UE’s measurement requirements can be little impacted because SMTC period in TPSS/SSS_sync and T SSB_measurement_period can be explicitly indicated by NW.

Proposal 3: L3 measurement requirements with the adaptation SSB can be FFS upon LS reply on whether there is impact on SMTC.

- References

- RP-240830

- TS38.133 v18.5.0

- TS38.331 v17.1.0

- R4-2413885