3GPP TSG-RAN WG4 Meeting # 114						R4-2500793

# Athens, Greece, 17-21 Feburary, 2025

Source:	Huawei, Hisilicon

Title:	draft TP to TR 38.719-02-01 include CA_n1-n41

Agenda item:			6.3.3(NR_CADC_R19_2BDL_xBUL)

Document for:	Approval

1 Introduction

The new basket WID was approved in RAN #106 meeting [1]. This contribution provides a text proposal for TR 38.719-02-01 to include CA_n1A-n41A BCS 4 and 5. Since CA_n1A-n41A BCS 0 and 1 already exist in TS38.101-1, in this TP only cross band analysis is performed.

2 References

[1]RP-242754, Revised WID: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells, Ericsson, ZTE, Huawei, HiSilicon, Dec.,2024, RAN#106

Text Proposal

<Start of Text Proposal>

5.x CA_n1-n41

5.x.1	Common for 1 band UL and 2 bands UL CA

5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex mode |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive |  |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz | FDD |
| n41 | 2496 MHz –2690 MHz | 2496 MHz –2690 MHz | TDD |

5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n1A-n41A | CA_n1A-n41A | n1 | n1 channel bandwidths in Table 5.3.5-1 of 38.101-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 of 38.101-1 |  |

5.x.1.3	UE co-existence studies for 1 band UL

For CA_n1-n41, there is no need to analyse harmonics and/or harmonics mixing since it has already been done.

Table 5.x.1.3-1 summarizes frequency ranges where cross band isolation may occur for CA_n1-n8.

Table 5.x.1.3-1: Cross-band isolation analysis

| Bands | n1 |  | n41 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 1920 | 1980 | 2496 | 2690 |
| fDL (MHz) | 2110 | 2170 | 2496 | 2690 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 50 | 5 | 100 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 1870 | 2030 | 2396 | 2790 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 1820 | 2080 | 2296 | 2890 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 1770 | 2130 | 2196 | 2990 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 1720 | 2180 | 2096 | 3090 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 1670 | 2230 | 1996 | 3190 |
| Analysis | There is no overlap of the band n1 ACLR range with the band n41 DL up to order 5. The sufficient rejection of the transmitter noise floor should be at band n8 so no cross-band MSD is needed. |  | There is an overlap of the band n41 ACLR 4 and ACLR 5 ranges with the band n1 DL. Therefore, the cross-band isolation MSD should be studied. |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible NOTE 2: The maximum UL channel bandwidth of the BCS (noted maxULCBW) is used to calculate the band ACLR ranges while the minimum DL channel bandwidth of the BCS (noted minDLCBW) is used for the DL band victim channel bandwidth. |  |  |  |  |

For the cross-band overlapping from uplink n41 to n1 downlink, Table 7.3A.6-1 in 38.101-1 has specified. No additional MSD is needed.

Table 7.3A.6-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n1 | n41 | 1955 | 50 | 15 | 128 (RBstart=142) | 2501 | 10 | 6.1 | >ACLR2 |
| n41 | n1 | 2546 | 100 | 30 | 270 (RBstart=0) | 2167.5 | 5 | 18.1 | >ACLR2 |

5.x.1.4	∆TIB,c and ∆RIB,c values

For CA_n1-n41, the TIB,c and RIB,c values have been specified.

5.x.1.5	REFSENS requirements

There are no additional MSD requirements for this band combination.

5.x.1.6	OOB blocking exception requirements

No additional OOB blocking exceptions are required for this CA band combination.

<End of Text Proposal>