3GPP TSG-RAN WG4 Meeting #114		R4-2500851

# Athens, Greece, 17th – 21st February, 2025

Agenda Item:	7.26.4.2

Source: 	CMCC

Title: 			 Discussion on RRM impact of LP-WUR

Document for:	Discussion

- Introduction

In last meeting, RAN4 further discussed detail LP-WUR requirements and MR RRM relaxation related issues. The WF has been approved in [1], and the open issues are captured in the topic summary [2]. In this contribution, we further analyze the RRM impact of LP-WUR, and provide our proposals.

- Discussion

  - General aspects

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

Recommendations:

Postpone the discussion until more RAN2 progress

Background:

Case #1 and #3 have been agreed

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

RAN4 to further discuss case #2

| RRM measurement case index | MR serving cell measurement | MR neighbouring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |

Based on our internal checking, for this issue, RAN2 is waiting for the agreement from RAN4.

The legacy cell re-selection procedure in TS 38.304 are:

- If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may not perform intra-frequency measurements.

- If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ,  the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority.

These two procedures should also be valid for LP-WUR capable UE.

- Observation 1: The legacy cell re-selection measurement rules should also be valid for LP-WUR capable UE.

Then, as long as network configures Case#3 entry condition lower than SnonIntraSearchP and/or SIntraSearchP, and Case#1 entry condition higher than SIntraSearchP, after UE entry in Case 3 and the Srelev becomes higher than SIntraSearchP and lower than the entry condition of Case#1, only MR serving cell relaxed measurement need to be performed, which is Case#2. Hence, Case#2 is a possible case, and its existence depends on network configuration.

- Proposal 1: Case#2 is a possible case, its existence depends on network configuration.

Issue 1-1-2: Threshold for switch between different cases

- Proposals

  - P1: Up to RAN2 decision (CMCC)

  - P2: LR measurement can be used to check the criteria for neighbor cell measurement triggering/relaxation (in case #1). LR measurement result shall be comparable to MR measurement result or shall be equivalent to MR measurement result with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin). (Apple)

  - P3: (Huawei)

    - For switch from Case #1 to Case #3 based on LR measurement.

      - Option 1: Use existing thresholds for SS-RSRP and SS-RSRQ, and define new thresholds for LP-RSRP and LP-RSRQ

      - Option 2: Use existing thresholds for SS-RSRP and SS-RSRQ, and define offsets for LP-RSRP and LP-RSRQ

    - For switch from Case #3 to Case #1 based on LR or both LR and MR measurements.

      - Option 1: LR measurement only

      - Option 2: both LR and MR measurements, and UE stops neighbor cell measurement when both measurements are below their corresponding thresholds

  - P4: Need to discuss whether case 2 will be considered or not first (LG)

  - P5: New LP-based threshold(s) based on LP-RSRP or LP-RSRQ used for switch into case 1 from other cases or exit from case 1 to other cases are needed (ZTE vivo)

Recommendations:

Check RAN2’s progress on this issue.

Discuss whether new LP-based threshold(s) used for switch into case 1 from other cases or exit from case 1 to other cases are needed.

Whether other cases include case 2 or not depends on issue 1-1-1, i.e., whether case 2 will be considered or not.

Which type of threshold will be used for RRM measurement relaxation and offloading criteria should be decided by RAN2, it has already been under discussion. The current progress is as following:

Agreement from R2_127 - For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results. Agreement from R2_128 The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed.

RAN4 should wait for more progress from RAN2.

- Proposal 2: RAN2 is responsible for the discussion of which type of threshold will be used for RRM measurement relaxation and offloading criteria.

Issue 1-1-3: Measurement requirements to be specified for LP-WUR

Recommendations:

Encourage company further check related RAN1’s agreements.

Background:

Agreement from RAN4#110bis

- At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following:

  - Measurement requirements for LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state

  - Measurement requirements for LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

- Other related requirements are FFS

Agreement from RAN1#118bis

Support overlaid OFDM sequence(s) for LP-SS:

- LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS.

- Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable.

  - Applicable at least for OOK-1 and FFS for OOK-4

- From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

RAN1 agreed that overlaid OFDM sequence over LP-SS is supported, but RAN4 requirements are not intended to be introduced. Therefore, RAN4 can only focus on the measurement requirement specification of following two cases:

- Measurement requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state

- Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

- Proposal 3: RAN4 specifies measurement requirements for the following two cases:

- Measurement requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state

- Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

Issue 1-1-7: On jointly consideration on Criteria (entry/exit conditions) for fully offloading/MR RRM measurement relaxation and LP-WUS monitoring

- Proposals

  - P1-1: Define same entry/exit conditions for LP-WUS monitoring and RRM measurement relaxed cases (case 3). (xiaomi LG Apple vivo).

  - P2: The LR-based LP-WUR measurement threshold can be less than to the LR-based LP-WUS monitoring threshold. (LG)

  - P3: RAN4 to suggest RAN2 to define same entry/exit condition for LP-WUS monitoring, serving cell RRM measurement offloading and MR RRM measurement relaxation. (Huawei)

Based on the latest RAN2 agreement, the summary of entry/exit conditions is:

| Case | Entry conditions | Exit conditions |
| --- | --- | --- |
| LP-WUS monitoring | MR threshold and optional LR threshold | LR threshold |
| RRM measurement fully offloading (Case 1) | MR threshold and optional LR threshold | LR threshold |
| RRM measurement relaxation (Case 3) | MR threshold and FFS LR threshold | FFS |

Currently, the entry/exit condition for LP-WUS monitoring and the entry/exit condition for LP-WUR measurement are discussed separately. However, from the perspective of UE power saving gain, it is meaningless that UE entry the LP-WUS monitoring while performing the measurement by MR. Two conditions should be considered jointly. To guarantee the power saving gain, the LP-WUS monitoring entry/exit threshold shall be equal to or higher than the threshold for RRM measurement relaxation (Case 3).

- Proposal 4: In order to guarantee the power saving gain, the LP-WUS monitoring entry/exit threshold shall be equal to or higher than the threshold for RRM measurement relaxation (Case 3).

Issue 1-1-8: LP-WUR operating carrier frequency

LS from RAN1#118 R1-2407559

- In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. - RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs. To RAN2 and RAN4: ACTION: RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

- After the discussion in RAN#106 meeting, the way forward are:

- For idle/inactive mode, consider a standardized solution for intra-frequency case (e.g. solution 1, 2, and 3 in RP-242656).  For Rel-19, inter-frequency solutions (e.g. solution 4) will not be considered.

- Solution 1: MR is prioritized (via SIBs) to camp on the same band as LR (System information, Paging, RACH on LR band)

- Solution 2: MR is prioritized to camp on the same band as LR (System information, Paging on LR band; and RACH on another MR band based on reselection criteria)

- Solution 3:  MR is not prioritized to camp on the same band as LR but can be redirected via dedicated signalling depending on LR band load (up to gNB implementation)

- Detailed discussions and final solution selection is left up to WGs.

- The expected specification efforts for three candidate solutions are mainly in RAN2. But we also identify that Solution 2 has some RAN4 impacts.

- For Solution 2, after MR receiving the paging on the camped carrier frequency which belongs to LR band, UE shall send RACH on the first rank carrier frequency based on reselection criteria, which means UE need to find the first rank carrier frequency based on cell re-selection measurement. Considering the time duration between paging and RACH shall be as short as possible, how to define the requirement for this evaluation, whether the current cell re-selection requirement can be reused should be further studied.

- Proposal 5: If solution 2 is agreed, then RAN4 should study how to define the requirement for the cell re-selection evaluation between Paging and RACH.

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

- Proposals

  - P1: LR status is ON (xiaomi)

  - P2: LP-WUR can be ON or OFF for serving cell measurement. If the threshold for LP-WUS monitoring and LP-WUR RRM measurement are same the LP-WUR can be OFF, else if the threshold for LP-WUS monitoring is higher than for the LP-WUR RRM measurement the LP-WUR can be ON. (LG)

  - P3-1: LP-WUR is ON for serving cell measurement, but when and how to turn on LR for serving cell measurement is up to UE implementation. (Apple)

  - P3-2: When a LP-based threshold is configured, at legacy case LP-WUR needs be “ON” at least some duration early before comparing with a LP-based threshold. (vivo Huawei Ericsson)

    - No need define any LR measurement requirements when a UE is at legacy case (vivo Huawei)

- Recommendations:

  - Check the following:

    - LP-WUR is ON for serving cell measurement, but when, how to turn on LR and the duration of “ON” for serving cell measurement is up to UE implementation, and no LP-WUR requirement will be applied.

Based on the latest RAN2 agreement, the summary of entry/exit conditions is:

| Case | Entry conditions | Exit conditions |
| --- | --- | --- |
| LP-WUS monitoring | MR threshold and optional LR threshold | LR threshold |
| RRM measurement fully offloading (Case 1) | MR threshold and optional LR threshold | LR threshold |
| RRM measurement relaxation (Case 3) | MR threshold and FFS LR threshold | FFS |

Based on our understanding, for the entry condition, if both MR threshold and LR threshold is configured, then the LR need to be ‘ON’ even if UE is in legacy case, the entry condition is fulfilled when the measurement results from MR and LR are higher than the threshold simultaneously.

As for when to turn on the LR, we have two potential solutions:

- Up to UE implementation. For example, when MR measurement result is near to the MR threshold, UE turn on its LR, or LR is always on. With such UE behavior, we are fine not to introduce requirement for LR measurement at legacy case.

- After MR measurement result is higher than MR threshold, UE turn on its LR. With such UE behavior, the requirement for LR measurement at legacy case should also be defined.

- Proposal 6: If LR-based threshold is configured for entry condition, then the LR need to be ‘ON’. The entry condition is fulfilled when the measurement results from MR and LR are higher than the threshold simultaneously.

- Proposal 7: Two potential solutions regarding when to turn on the LR:

- Up to UE implementation, and not to introduce requirement for LR measurement at legacy case.

- After MR measurement result is higher than MR threshold, UE turn on its LR. The requirement for LR measurement at legacy case should also be defined.

Issue 1-1-10: Considerations on higher priority frequency layer

- Proposals

  - P1: Support relaxed higher priority frequency layer search (xiaomi CMCC CT ZTE vivo Ericsson)

  - P1-1 (CMCC)

    - When fulling offloading case happens (Case#1), the relaxed higher priority layer measurement requirement can be applied, i.e., per higher priority layer within 1h

    - When partial offloading case happens (Case#3), open to further discuss which higher priority layer measurement requirement should be applied

  - P1-2: For case 1 and case 3, when the serving cell is above threshold for inter-frequency neighbour cell measurement, existing relaxed requirements K2*Thigher_priority_search are re-used for higher priority frequency search. (vivo Ericsson)

  - P2: Higher priority layer neighbor cell measurement shall also be OFF for case #1. (Apple)

  - P3: UE should perform measurement for higher priority frequency layers, if configured (Huawei)

    - If serving cell is below threshold for inter-frequency neighbor cell measurement (Case #3), UE meet the relaxed measurement requirements

    - If serving cell is above threshold for inter-frequency neighbor cell measurement (Case #4), FFS whether existing requirements (60 * Nlayers) are re-used or further relaxed.

Recommendations:

Could check P2 firstly, i.e., for case 1, whether higher priority frequency layer search will be performed or not.

The higher priority frequency measurement strategy is highly depends on Issue 1-1-8.

For instance, if Solution 1 is agreed, then we think the higher priority measurement can be OFF. If Solution 2 is agreed, the higher priority measurement need to be ‘ON’, but when to turn on the higher priority measurement shall be further studied. If Solution 3 is agreed, the higher priority measurement need to be ‘ON’, when fulling offloading case happens (Case#1), in order to guarantee UE power saving gain, we think the relaxed higher priority layer measurement requirement can be applied. When partial offloading case happens (Case#3), we are open to discuss which higher priority layer measurement requirement should be applied.

- Proposal 8: Whether to turn on higher priority measurement depends on the Solution for unaligned MR and LR operating bands:

- If Solution 1 is adopted, then the higher priority measurement can be OFF.

- If Solution 2 or 3 is adopted, the higher priority measurement should be ON, further study when to turn on the higher priority measurement and how to define the measurement requirements.

Issue 1-1-12: LP-SS frequency domain relation with SSB

- Proposals

  - P1: RAN4 to further discuss whether there is RRM impact due to the configuration of FDMed SSB and LP-SS. (Huawei)

  - P2: For LP-SS and SSB FDMed multiplex, RAN4 to wait RAN1’s progress (Ericsson)

Firstly, we think LP-SS and SSB FDMed multiplex is a valid NW configuration, which should be supported. Secondly, we don’t identify any RRM impact from this configuration so far, provided NW ensure suitable threshold configuration. Since such configuration is under discussion in RAN1, RAN4 can have detailed analysis based on RAN1 conclusion if any.

- Proposal 9: Further discuss detailed RRM impact for LP-SS and SSB FDMed multiplex configuration based on RAN1 conclusion if any.

  - Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Issue 1-2-1: Accuracy requirements

- Proposals

  - P1: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement. (xiaomi LG CMCC CT ZTE Apple Ericsson Huawei MTK)

  - P2: If “using a margin to reflect accuracy performance of LP-WUR in core specs”, is used for LP-WUR, for MR, the following options could be considered (vivo)

    - Option 1: For MR, margin is also introduced when MR serving cell measurement result comparing with a threshold, this margin will be based on legacy MR accuracy requirements.

    - Option 2: For MR, margin is not also introduced when MR serving cell measurement result comparing with a threshold, as that of the legacy specs.

  - P3: Conclude final accuracy requirement details after simulations have been collected and compared (Nokia)

Recommendations:

The question of P2 is based on the following table, for example entry conditions for case 1 where both MR threshold and optional LR threshold. When a margin is defined for LP-WUR, how to handle the MR, will a margin be defined for MR as well?

In legacy RRC_IDLE requirements, for MR, the measurement accuracy was considered as a margin when comparing the R value among the target cells, while the measurement accuracy was not considered when comparing the measurement results with the network configured thresholds.

Considering the LR measurement results will be mainly used for comparing with the network configured thresholds, follow the legacy methodology, it may not proper to consider the measurement accuracy as a margin in such comparison. Thus, either define dedicated accuracy requirement in the performance section for LR-WUR based RRM measurement in Idle/inactive states, or don’t define the accuracy requirement for LR-WUR based RRM measurement in Idle/inactive states but consider it in the measurement delay requirement simulation are both ok for us.

- Observation 1: In legacy, the measurement accuracy was not considered when comparing the measurement results with the network configured thresholds.

- Proposal 10: For the Accuracy requirements for LR-WUR based RRM measurement in Idle/inactive states, we are open to consider following two approach:

- define dedicated accuracy requirement in the performance section but no test case.

- don’t define the accuracy requirement for LR-WUR based RRM measurement in Idle/inactive states, but use it as the simulation target when evaluate the measurement delay requirement simulation.

Issue 1-2-2: Periodicity for SSB based LP-WUR measurement delay requirements

Agreement:

If LP-SS is not configured, periodicity for SSB based LP-WUR measurement delay requirements

- based on LO periodicity

  - FFS whether to define an upper bound

  - Note: DRX cycle and LO periodicity are the same based on RAN1 agreement.

Agreement:

When LP-SS is configured, apply the same agreement as for when LP-SS is not configured

- The above agreement applies if LO periodicity is equal to or larger than LP-SS periodicity, FFS if LO periodicity is smaller than LP-SS periodicity.

Agreement from RAN1#118b

Confirm the following working assumption for iDRX: For each UE, the periodicity of LO is the same as its iDRX cycle.

Since the LO periodicity is configured by network, we are ok not to define the upper bound.

For the case that LO periodicity is smaller than LP-SS periodicity, since anyway LR need to wake up and perform LP-WUS monitoring in each LO, the power saving gain will not be increased a lot assuming UE follow LP-SS periodicity to perform serving cell measurement. Therefore, we think the LO periodicity can still be the periodicity of SSB based LP-WUR measurement.

- Proposal 11: Periodicity of SSB based LP-WUR measurement will follow the LO periodicity regardless of LP-SS periodicity, no need to define the upper bound.

Issue 1-2-3: On requirements for LP-WUR for entry/exit criteria(threshold) evaluation

Recommendations:

Encourage companies firstly to consider whether to differentiate requirements based on cases

Note: For entry condition evaluation requirements,  requirements from legacy state to case 1 or case 3 or LP-WUS monitoring will be discussed separately.

If  No

Same requirements will be defined on entry criteria(threshold) evaluation from one case to the other case (excluding scenarios in the note).

Same requirements will be defined on exit criteria(threshold) evaluation from one case to the other case.

If Yes, consider the following cases one by one

Issue 1-2-3a: On requirements for entry/exit criteria(threshold) evaluation for Case 1

Issue 1-2-3b: On requirements for entry/exit criteria(threshold) evaluation for Case 3

Issue 1-2-3c: On requirements for entry/exit criteria(threshold) evaluation for LP-WUS monitoring

On detailed evaluation requirements, the following options are available

- Option 1: Evaluation requirements for entry/exit evaluation is the same as that of LP-WUR measurement period.

- Option 2: Evaluation requirements for entry/exit conditions is longer than that of LP-WUR measurement period.

- Option 3: Evaluation requirements for entry evaluation is longer than that of LP-WUR measurement period; evaluation requirements for exit evaluation is the same as that of the LP-WUR measurement period.

Issue 1-2-3-1: On LP-WUR/MR evaluation requirement when both MR and LR threshold are configured

- Proposals

  - P1: (vivo Huawei MTK)

    - For the evaluation period for evaluating an entry threshold when both MR and LR threshold are configured, defined MR and LR evaluation requirements separately

  - P2: RAN4 need to further how to specify evaluation period for LP-WUS monitoring entrance if both LR and MR threshold configured. (Samsung)

  - P3: (LGE)

    - For the evaluation period for evaluating an entry threshold for LP-WUS monitoring or LP-WUR measurement

      - LP-SS based LP-WUR measurement period

    - For the evaluation period for evaluating exit threshold for fully offloading

      - The MR serving cell measurement period

Recommendations:

Check P1 is agreeable

In past few meetings, we propose to use LP-WUR/MR measurement period to define the entry/exit criteria threshold evaluation requirement, which similar as R16 relaxation criteria evaluation.

However, for fully offloading case, considering MR will go into ultra deep sleep mode, and there will be MR activation time between ultra deep sleep mode and normal mode, it is necessary to further avoid the Ping-Pong effect for fully offloading case. The evaluation requirements for the entry/exit condition of fully offloading case could include multiple measurement periods.

The MR and LR evaluation requirements should be defined separately, regardless how network configure the thresholds.

Proposal 12: For the entry/exit criteria threshold evaluation requirement of LP-WUR and MR,

- For entry/exit between legacy state and Case 3, and LP-WUS monitoring, define the requirement based on single measurement period

- For entry/exit between legacy state and Case 1, Case 3 and Case 1, Case 2 and Case 1, define the requirement based on multiple measurement period

Issue 1-2-4: On interruption related requirements for LP-WUR

- Proposals

  - P1: Wait for RAN1 further progress or based on RAN1’s conclusion. (CATT CMCC oppo Huawei)

  - P2: RAN4 only needs to define the wake-up delay requirement, if such requirement is necessary from majority companies’ view, where the wake-up delay means minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. (Apple)

  - P3: Suggest to use the term “MR activation” time to replace “interruption” time;  Defining interruption requirement for the normal case and worst case where a cell reselection is included in the worst case, ramping duration is based on RAN1 conclusion, detailed conditions are FFS. (vivo)

    - “MR activation” duration = Ramping duration + (2 + 2) samples * SSB_ periodicity

    - “MR activation” duration = Ramping duration + (2 + 2 + 23) samples * SSB_ periodicity + TSI-NR + 2*Ttarget_cell_SMTC_period

  - P4: MR wake-up time is pending on UE capability which can be 400ms and 800ms based on RAN1 agreement; MR sync time, need to be further discuss with different side conditions (Samsung)

    - Condition 1: MR and LR well sync, and LR sync info can be fully reused e.g.  1~2 SSB sufficient

    - Condition 2: MR and LR not well synced and MR need to apply sync procedure e.g. at least 10 SSBs required

After UE entry the Case 1 (fully offloading case),  MR will go into ultra-deep sleep mode, so MR can’t receive paging immediately after LR receiving LP-WUS with paging indication.

RAN1 also discussed this issue, how does UE report the wake up delay capability, and how does network configure the offset value(s) between an LO and a reference PO/PF, following agreements have been achieved:

Agreement For the offset value(s) between an LO and a reference PO/PF, consider the following options: - Definition: The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay. - Option 1: gNB configures a single offset value. - If the gap between an LO and the PO associated with the offset is no less than the wake-up delay a UE supports, the UE monitors the PO associated with the offset after receiving a wake-up indication in a LP-WUS. - Otherwise, - Option 1-1: the UE follows the legacy paging monitoring procedure. - Option 1-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - Option 2: gNB configures one or multiple offset values. - For the same PO, each offset corresponds to a LO. - This does not preclude the possibility that the same LO may correspond to different POs with different offset values. - Option 2A: A UE does not expect that the gap between the LO associated with the largest offset and the corresponding PO is less than the wake-up delay the UE supports. The UE monitors the LO associated with one offset that has a gap between the LO and the corresponding PO no less than the wake-up delay. - This implies that the gNB needs to configure at least one offset value that is no less than the largest wake-up delay supported by the UEs. - FFS exactly how to choose the offset - Option 2B: - If the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay a UE supports, the UE monitors the LO associated with one offset that has a gap between the LO and the PO associated with the offset no less than the wake-up delay. - FFS exactly how to choose the offset - Otherwise, - Option 2B-1: the UE follows the legacy paging monitoring procedure. - Option 2B-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - FFS exactly how to choose the offset - FFS the UE shall monitor the LO associated with additional offset(s) Note: The PO mentioned above refers to legacy PO configured for the UE. Agreement Support 3 candidate values for the wake-up delay capability a UE reports, two values for ultra-deep sleep state and one value for deep sleep state. - FFS the values - Note: Ultra-deep sleep state and deep sleep state are mentioned above solely for the purpose of RAN1 discussions and it is not intended to be captured in specifications from RAN1 perspective Agreement from RAN1#119 For the offset value(s) between an LO and a reference PO/PF, down-select between - Option 1-1 - Configurability between Option 1-1 and Option 1-2 - Option 2B-1

If RAN1 will specify the offset values and related UE behaviors, RAN4 don’t need to define the duplicate interruption requirement. Hence, we can postpone the discussion and wait for more RAN1 progress.

- Proposal 13: For the interruption between LP-WUS (with paging indication) reception and MR to start paging monitoring, postpone the discussion and wait for more RAN1 progress.

Issue 1-2-5: On LP-WUR cell selection evaluation requirements for case 1

- Proposals

  - P1: No need to define the LR-WUR based cell selection criterion evaluation in case 1. (Apple vivo Huawei MTK)

  - P2: Define LP-WUR based serving cell selection criterion S evaluation requirement in case 1. (CATT Ericsson)

  - P2-1：The existing MR based evaluation requirements can be the baseline (CATT)

UE derive the Srxlev and Squal in cell selection criterion evaluation, the results are used for following case:

- For initial cell selection, after a UE has switched on and a PLMN has been selected

- For R criterion checking, UE shall perform ranking of all cells that fulfil the cell selection criterion S

- For neighbour cell measurement checking, UE may not perform intra/inter-frequency measurements when Srxlev > Snon/IntraSearchP and Squal > Snon/IntraSearchQ

- In Case 1 (fully offloading case, no neighbour cell measurement), UE has already camped on a suitable cell, and network should guarantee that UE enters the Case 1 after Srxlev > SIntraSearchP and Squal > SIntraSearchQ. With such assumption, in case 1, UE don’t need to evaluate the S criterion anymore.

- Proposal 14: No need to define the LR-WUR based cell selection criterion evaluation in case 1

Issue 1-2-7: Accuracy for relaxed MR measurement

- Proposals

  - P1: The legacy accuracy for relaxed MR measurement should be reused if Case#2 and/or Case#3 introduced. (CMCC vivo Huawei)

Recommendations:

Check P1 is agreeable

The MR measurement result will be used to determine serving cell quality and neighbouring cell quality for cell selection and cell re-selection, and whether to trigger the measurement offloading, the accuracy is rather important among these procedure to guarantee UE mobility and camp on. The legacy accuracy should be reused.

- Proposal 15: The legacy accuracy for relaxed MR measurement should be reused for Case#3 and Case#2 (if introduced).

  - MR RRM relaxation

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell for case 3

Agreement:

Scaling factor(s) is/are >= 4. Further down-select from

- Option 1:  1 fixed scaling factor

- Option 2: multiple values configured by NW

- Option 3: multiple values, scaling factor value depends on DRX cycle length (QC MTK apple xiaomi vivo)

| DRX cycle length [s] | Scaling factor |
| --- | --- |
| 0.32 | TBD |
| 0.64 | TBD |
| 1.28 | TBD |
| 2.56 | TBD |

The first key point is the power saving gain. Based on the output observation from TR38.869 clause 8.1.1.5.2, for I-DRX=1.28s, if X<8, then very limited power saving gain will be achieved (average: 4%, range: -18%~50%). Therefore, the MR should at least be relaxed 8 times. We propose to use 8 as the baseline.

The second key point is the mobility performance, the whole evaluation delay should not be too long, to prevent untimely detection of mobility. The scaling factor larger than 8 should be avoid, especially for long DRX.

To be simplicity from both UE and network, we propose to use 1 fixed scaling factor which is 8 for each DRX cycle configuration.

- Proposal 16: For the measurement based on both MR with RRM relaxation (X time relaxation) and LP-WUR, X =8.

  - Others

Issue 1-5-1: LR based RRM with EMR in IDLE/inactive mode

- Proposals

  - P1: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not. (Apple)

  - P2: The principle of P1 is reasonable. The issue can be discussed later (vivo)

  - P3: The methodology of EMR requirement definition under each Cases can refer to the methodology of high priority layer measurement requirement definition. (CMCC)

  - P4: Discuss the UE behaviour while MR relaxation is applied to idle-mode measurements.  (Nokia)

    - If T331 timer is configured UE shall not enter LP-WUS mode (MR relaxation / offloading) (e.g. for the first 300 seconds in idle-mode)

    - RAN4 to also discuss if the rel-18 idle-mode behavior follows the same principle where UE shall keep the MR ON.

This issue is similar as the issue 1-1-10, that network configures one measurement which can only be performed by MR. Then when UE enters Case#1, whether the measurement need to be done by MR or not should be further investigate.

We think the importance of EMR measurement is lower than high priority measurement. Currently, to maximum the power saving gain of LP-WUR capable UE, whether and how to perform high priority frequency measurement is still FFS. We prefer to discuss the high priority measurement first, and then discuss this issue.

- Proposal 17: Before RAN4 conclude whether and how to perform high priority frequency measurement, postpone the EMR issue.

- Conclusion

Based on the discussion above, the following proposals are concluded.

- Observation 1: The legacy cell re-selection measurement rules should also be valid for LP-WUR capable UE.

- Proposal 1: Case#2 is a possible case, its existence depends on network configuration.

- Proposal 2: RAN2 is responsible for the discussion of which type of threshold will be used for RRM measurement relaxation and offloading criteria.

- Proposal 3: RAN4 specifies measurement requirements for the following two cases:

- Measurement requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state

- Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

- Proposal 4: In order to guarantee the power saving gain, the LP-WUS monitoring entry/exit threshold shall be equal to or higher than the threshold for RRM measurement relaxation (Case 3).

- Proposal 5: If solution 2 is agreed, then RAN4 should study how to define the requirement for the cell re-selection evaluation between Paging and RACH.

- Proposal 6: If LR-based threshold is configured for entry condition, then the LR need to be ‘ON’. The entry condition is fulfilled when the measurement results from MR and LR are higher than the threshold simultaneously.

- Proposal 7: Two potential solutions regarding when to turn on the LR:

- Up to UE implementation, and not to introduce requirement for LR measurement at legacy case.

- After MR measurement result is higher than MR threshold, UE turn on its LR. The requirement for LR measurement at legacy case should also be defined.

- Proposal 8: Whether to turn on higher priority measurement depends on the Solution for unaligned MR and LR operating bands:

- If Solution 1 is adopted, then the higher priority measurement can be OFF.

- If Solution 2 or 3 is adopted, the higher priority measurement should be ON, further study when to turn on the higher priority measurement and how to define the measurement requirements.

- Proposal 9: Further discuss detailed RRM impact for LP-SS and SSB FDMed multiplex configuration based on RAN1 conclusion if any.

- Observation 1: In legacy, the measurement accuracy was not considered when comparing the measurement results with the network configured thresholds.

- Proposal 10: For the Accuracy requirements for LR-WUR based RRM measurement in Idle/inactive states, we are open to consider following two approach:

- define dedicated accuracy requirement in the performance section but no test case.

- don’t define the accuracy requirement for LR-WUR based RRM measurement in Idle/inactive states, but use it as the simulation target when evaluate the measurement delay requirement simulation.

- Proposal 11: Periodicity of SSB based LP-WUR measurement will follow the LO periodicity regardless of LP-SS periodicity, no need to define the upper bound.

Proposal 12: For the entry/exit criteria threshold evaluation requirement of LP-WUR and MR,

- For entry/exit between legacy state and Case 3, and LP-WUS monitoring, define the requirement based on single measurement period

- For entry/exit between legacy state and Case 1, Case 3 and Case 1, Case 2 and Case 1, define the requirement based on multiple measurement period

- Proposal 13: For the interruption between LP-WUS (with paging indication) reception and MR to start paging monitoring, postpone the discussion and wait for more RAN1 progress.

- Proposal 14: No need to define the LR-WUR based cell selection criterion evaluation in case 1

- Proposal 15: The legacy accuracy for relaxed MR measurement should be reused for Case#3 and Case#2 (if introduced).

- Proposal 16: For the measurement based on both MR with RRM relaxation (X time relaxation) and LP-WUR, X =8.

- Proposal 17: Before RAN4 conclude whether and how to perform high priority frequency measurement, postpone the EMR issue.

- Reference

- R4-2420100, WF on RRM requirements for NR_LPWUS, vivo

- R4-2418284, Topic summary for [113][226] NR_LPWUS, vivo