3GPP TSG-RAN WG4 Meeting # 114                                   R4-2500864

Athens, GR, 17th – 21st February, 2025

Agenda item:	7.26.3

Source: 	CMCC

Title: 	Discussion on LP-WUS BS RF requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #105 meeting, revised WID of LP-WUS is approved with following RF part objectives:

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance

    - Define guard RBs for ACS and ASCS cases

    - Study testability of above requirements

    - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

In RAN4#113, the LP-WUS BS RF requirements had been discussed and the WF was approved in [1]. This paper will focus on some issue about LP-WUS BS RF requirements.

- Discussion

2.1 Core requirement for LP-WUS power boosting

Issue 1-3: Core requirement for LP-WUS power boosting - WF - FFS the following options - Option 1: Leave the power boosting level to manufacturer declaration - Option 2: If the manufacturer declares power boosting, the minimum LP-WUS power boosting level should be X (>0) dB for Y MHz CBW - Other options are not precluded

From our perspective, it is challenging to establish a minimum requirement specific to our network. Given that different operators employ this technology across a range of scenarios, setting a uniform minimum requirement is complex. To preserve the flexibility associated with power boosting, we propose not to stipulate a minimum level for power boosting. Instead, the power boosting level should be determined based on the specific declarations made.

Proposal 1: Leave the power boosting level to manufacturer declaration.

### 2.2 Concept of LP-WUS power boosting

Issue 1-1: Concept of LP-WUS power boosting - Proposals - Proposal 1: EPRE ratio can be used for the LP-WUS power boosting concept. - The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power NR REs) when both LP-WUS and NR are transmitted at the same time in the NR carrier. (CATT) - The case when there is no NR REs allocation should be covered. (Nokia) - Proposal 2: Adopt following definition rather than introduce EPRE ratio. (ZTE, Ericsson, Nokia) - The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs). - Proposal 3: An NB-IoT-like approach to defining power boosting could serve as a baseline, offering flexibility in declaration. Additionally, the option for a single-value declaration should also be permitted, accommodating different scenarios. (Huawei) - The LP-WUS RB power dynamic range is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs). If a single power booting value is to be declared irrespective of channel bandwidth, the difference in per-RE power ratios between the average power of LP-WUS REs (within the NR transmission bandwidth configuration) and the average power NR REs (excluding LP-WUS REs) could alternatively be used. - Proposal 4: RAN4 should use LP-WUS power boosting concept rather than “dynamic range” as definition for this requirement. The power boosting concept and RE dynamic range concept for LP-WUS be separated. (vivo) - Adhoc WF - “The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs)” is considered as starting point for the definition. - FFS whether other cases can be additionally considered. E.g. single value declaration.

Based on the understanding of most companies, the definition of LP-WUS power boosting should be similar to that of NB-IoT. In addition, considering that the power boosting level proposed in Proposal 1 is entirely dependent on the declaration, the choice of the definition for X will not have a significant impact on the formulation of the RF requirements. Therefore, we think that the definition in Proposal 2 should be more acceptable.

Proposal 2: “The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs)” is considered as starting point for the definition.

### 2.3 De-boost on the NR transmission power

- WF - Consider the de-boosting behaviour together with other relevant issues - Companies are encouraged to think about the draft spec langrage to reflect corresponding proposals

Whether to reduce the transmission power of the NR signal when transmitting the WUS signal simultaneously should be decided by the network. As the operator, in most cases, we do not want the power of the NR signal to be affected, especially for user at the cell edge, the network should not reduce the transmission power, otherwise it will affect the throughput. If the output power of LP-WUS signal is boosting while the NR signal is not de-boost at the same time, the total transmission power of this carrier may exceed the rated carrier power. So we think that the level of power boosting should be determined by the declaration, while minimizing any impact on the NR signal of the base station.

Observation 1: Whether to de-boost NR signal transmission power should be up to network implementation.

Proposal 3: level of power boosting should be determined by the declaration, while not considering de-boost of the NR signal power.

### 2.4 BS Tx requirements for LP-WUS

- Adhoc WF - Discuss on whether transmit signal quality should be defined - FFS whether equivalent EVM requirement could be considered, - E.g. NR needs to pass 64QAM EVM test with LP-WUS embedded in the NR carrier - FFS whether no other new RF requirements than the dynamic range or potential transmit quality requirement identified for LP-WUS

For BS RF requirements, in order to regulate the use of LP-WUS and its performance, we suggest to define a set of the BS Tx requirements. Unwanted emissions should also to be defined.

Proposal 4: Define a set of the BS Tx requirements for LP-WUS.

# 3. Conclusions

Proposal 1: Leave the power boosting level to manufacturer declaration.

Proposal 2: “The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs)” is considered as starting point for the definition.

Observation 1: Whether to de-boost NR signal transmission power should be up to network implementation.

Proposal 3: level of power boosting should be determined by the declaration, while not considering de-boost of the NR signal power.

Proposal 4: Define a set of the BS Tx requirements for LP-WUS.

# 4. Reference

- R4-2419802, WF for [113][308] NR_LPWUS