3GPP TSG-RAN WG4 Meeting#114		R4-2500896

Athens, GR, Feb 17th -21st, 2025

Agenda item:	7.1.1.3.3

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on SRS antenna switching requirements for 6RX UE

Document for:	Discussion

Introduction

In RAN4#113meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follows.

Discussion

General considerations for SRS antenna switching and ΔTRxSRS

- Sub-topic 1-1: General considerations for SRS antenna switching and ΔTRxSRS Issue 1-1-1: Whether to consider a subset of available patterns for 3T6R AS-SRS Way Forward: RAN4 to further discuss the following options. - Option 1: Consider a subset of available patterns for 3T6R AS-SRS. FFS on the subset details. - Option 2: Consider all available patterns for 3T6R AS-SRS.

According to the discussion in RAN4#113 meeting, we just talked about 4 patterns for 3T6R AS-SRS, in our understanding, there is no need to simplify the available patterns.

Proposal 1: Consider all available patterns for 3T6R AS-SRS.

ΔTRxSRS values

Way Forward: RAN4 to check whether the ∆TRxSRS values in the table below could be agreed. The average values from company proposals, as well as the existing values for other AS-SRS configurations, were considered as the starting point and were rounded to align with 0.5dB increments to keep consistency with the existing specification approach. Table: ΔTRxSRS values Operating Bands; ΔTRxSRS t3r6 (dB); ΔTRxSRS t1r6-t3r6 (dB); ΔTRxSRS t2r6-t3r6 (dB); ΔTRxSRS t1r6-t2r6-t3r6 (dB). Band n41, n77, n78; 3.0; 4.5; 4.0; 5.5. Band n79; 4.5; 6.0; 5.5; 7.0. Band n104; 5.0; 7.0; 6.0; 8.0.

In RAN4#112 meeting, ΔTRXSRS value for 1T6R, 2T6R, 1T6R-2T6R for PC3 has reached agreement. Hence, we need to discuss ΔTRXSRS value for 3T6R, 1T6R-3T6R, 2T6R-3T6R, and 1T6R-2T6R-3T6R. It is observed that 3T6R can be divided into three1T2R. Thus we can get the value of ΔTRXSRS for 3T6R is 3 dB. For 1T6R-2T6R-3T6R, we can see the difference with 1T6R (in Figure1 and Figure2 in Annex) is the length of SP4T and the length of PCB (0.5dB), thus we can get ΔTRXSRS of 1T6R-2T6R-3T6R is 5.5dB. 1TR6-3T6R and 2TR6-3T6R is similar with 1T6R-2T6R, thus we can get ΔTRXSRS of 1TR6-3T6R and 2TR6-3T6R is 4.5dB. ΔTRXSRS of 1T6R is 4.0dB and ΔTRXSRS of 1T6R-2T6R is 4.5dB based on the RAN4#112 meeting agreements. For n79, we can add 1.5dB based on n41/n77/n78. However, an additional breakpoint is introduced for n104, we can add 1.0dB based on n79.

Proposal 2: Adopt the value of ΔTRXSRS in Table 1 for 3T6R, 1T6R-3T6R, 2T6R-3T6R and 1T6R-2T6R-3T6R.

Table 1: ΔTRXSRS for 3T6R, 1T6R-3T6R, 2T6R-3T6R and 1T6R-2T6R-3T6R.

| Operating bands | ΔTRXSRS 3T6R(dB) | ΔTRXSRS 1T6R-3T6R(dB) | ΔTRXSRS 2T6R-3T6R(dB) | ΔTRXSRS 1T6R-2T6R-3T6R(dB) |
| --- | --- | --- | --- | --- |
| N41/N77/N78 | 3 | 4.5 | 4.5 | 5.5 |
| N79 | 4.5 | 6 | 6 | 7 |
| N104 | 5.5 | 7 | 7 | 8 |

In addition, FL has summarised the average values from company proposals, and the averages values were rounded to align with 0.5dB increments to keep consistency with the existing specification approach. In our understanding, we can accept the proposed ∆TRxSRS from company proposals.

Table2: ΔTRxSRS values

| Operating Bands | ΔTRxSRS t3r6 (dB) | ΔTRxSRS t1r6-t3r6 (dB) | ΔTRxSRS t2r6-t3r6 (dB) | ΔTRxSRS t1r6-t2r6-t3r6 (dB) |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 3.0 | 4.5 | 4.0 | 5.5 |
| Band n79 | 4.5 | 6.0 | 5.5 | 7.0 |
| Band n104 | 5.0 | 7.0 | 6.0 | 8.0 |

Proposal 3: We can accept the Table 2 from the average values from company proposals as the ΔTRxSRS values for 3T6R, 1T6R-3T6R, 2T6R-3T6R and 1T6R-2T6R-3T6R.

Conclusions

In this paper, we provide our views on 6RX UE RF requirements, we have made the following proposals.

Proposal 1: Consider all available patterns for 3T6R AS-SRS.

Proposal 2: Adopt the value of ΔTRXSRS in Table 1 for 3T6R, 1T6R-3T6R, 2T6R-3T6R and 1T6R-2T6R-3T6R.

Table 1: ΔTRXSRS for 3T6R, 1T6R-3T6R, 2T6R-3T6R and 1T6R-2T6R-3T6R.

| Operating bands | ΔTRXSRS 3T6R(dB) | ΔTRXSRS 1T6R-3T6R(dB) | ΔTRXSRS 2T6R-3T6R(dB) | ΔTRXSRS 1T6R-2T6R-3T6R(dB) |
| --- | --- | --- | --- | --- |
| N41/N77/N78 | 3 | 4.5 | 4.5 | 5.5 |
| N79 | 4.5 | 6 | 6 | 7 |
| N104 | 5.5 | 7 | 7 | 8 |

Table2: ΔTRxSRS values

| Operating Bands | ΔTRxSRS t3r6 (dB) | ΔTRxSRS t1r6-t3r6 (dB) | ΔTRxSRS t2r6-t3r6 (dB) | ΔTRxSRS t1r6-t2r6-t3r6 (dB) |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 3.0 | 4.5 | 4.0 | 5.5 |
| Band n79 | 4.5 | 6.0 | 5.5 | 7.0 |
| Band n104 | 5.0 | 7.0 | 6.0 | 8.0 |

Proposal 3: We can accept the Table 2 from the average values from company proposals as the ΔTRxSRS values for 3T6R, 1T6R-3T6R, 2T6R-3T6R and 1T6R-2T6R-3T6R.

- Reference

- R4-2420376, WF on requirements for 6RX, AT&T.TSG-RAN4#113 meeting.

- Annex

Figure 1: an example of 1T6R architecture                         Figure 2: an example of 1T6R-2T6R-3T6R architecture