3GPP TSG-RAN4 Meeting #114	R4-2500899

# Ahtens, Greece, 17th Feb 2025 – 21st Feb 2025

Source:	LG Electronics

Title:	Discussion for 6 Rx ΔTRxSRS

Agenda item:	7.1.1.3.3

Document for:	Discussion

Introduction

In the RAN4#113 meeting,  the WF on requirements for 6Rx was approved. In this discussion, we share our point of view for the ΔTRxSRS requirement based on the approved WF[1]

Discussion

Table 1 Way forward for 6 Rx SRS antenna switching and ΔTRxSRS [1]

Sub-topic 1-1: General considerations for SRS antenna switching and ΔTRxSRS Issue 1-1-1: Whether to consider a subset of available patterns for 3T6R AS-SRS Way Forward: RAN4 to further discuss the following options. - Option 1: Consider a subset of available patterns for 3T6R AS-SRS. FFS on the subset details. - Option 2: Consider all available patterns for 3T6R AS-SRS. Sub-topic 1-2: ΔTRxSRS values Issue 1-2-1: Proposed ∆TRxSRS values for t3r6, t1r6-t3r6, t2r6-t3r6, and t1r6-t2r6-t3r6 Way Forward: RAN4 to check whether the ∆TRxSRS values in the table below could be agreed. The average values from company proposals, as well as the existing values for other AS-SRS configurations, were considered as the starting point and were rounded to align with 0.5dB increments to keep consistency with the existing specification approach. Table: ΔTRxSRS values Operating Bands; ΔTRxSRS t3r6 (dB); ΔTRxSRS t1r6-t3r6 (dB); ΔTRxSRS t2r6-t3r6 (dB); ΔTRxSRS t1r6-t2r6-t3r6 (dB). Band n41, n77, n78; 3.0; 4.5; 4.0; 5.5. Band n79; 4.5; 6.0; 5.5; 7.0. Band n104; 5.0; 7.0; 6.0; 8.0.

In the last meeting, the ΔTRxSRS table was created based on the contributions of the companies. It appears that there are no significant differences in each value among the companies, and therefore, we agree on these values

Proposal : Agree on the ΔTRxSRS table for WF[1]

Table: ΔTRxSRS values

| Operating Bands | ΔTRxSRS t3r6 (dB) | ΔTRxSRS t1r6-t3r6 (dB) | ΔTRxSRS t2r6-t3r6 (dB) | ΔTRxSRS t1r6-t2r6-t3r6 (dB) |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 3.0 | 4.5 | 4.0 | 5.5 |
| Band n79 | 4.5 | 6.0 | 5.5 | 7.0 |
| Band n104 | 5.0 | 7.0 | 6.0 | 8.0 |

Conclusion

Proposal : Agree on the ΔTRxSRS table for WF[1]

Table: ΔTRxSRS values

| Operating Bands | ΔTRxSRS t3r6 (dB) | ΔTRxSRS t1r6-t3r6 (dB) | ΔTRxSRS t2r6-t3r6 (dB) | ΔTRxSRS t1r6-t2r6-t3r6 (dB) |
| --- | --- | --- | --- | --- |
| Band n41, n77, n78 | 3.0 | 4.5 | 4.0 | 5.5 |
| Band n79 | 4.5 | 6.0 | 5.5 | 7.0 |
| Band n104 | 5.0 | 7.0 | 6.0 | 8.0 |

- References

# [1] R4-2418140, WF on requirements for 6Rx  (AT&T)