3GPP TSG-RAN WG4 Meeting #114                                                                                       R4-2500988

Athens, GR, 17th –21st February, 2025

Source: 	CATT

Title: 	Further discussion on system parameters for Ku-band for NR NTN

Agenda item:	7.11.3

Document for:	Approval

- Introduction

In RAN4#113 meeting, the FR1 and FR2 down selection for Ku- band NTN in WF [1] was agreed to define two sets of bands, one for FR1 numerology and other for FR2 numerology. Meanwhile, A LS [2] on Ku band numerology to RAN WG1 and RAN WG2 was agreed. In RAN#106 meeting, the numerology for Ku-band NTN was also discussed to form a WF (RP-243301) and the WID for Ku-band NTN was also updated (RP-243314). The remaining issues on Ku-band system parameter require further discussion.

In this contribution, we provide our views on system parameters for Ku-band for NR NTN.

- Discussion

Impact of FR1 numerology and FR2 numerology for Ku-band on system parameters

As per the updated WID for Ku-band NTN [3], the objectives concerning system parameters are shown as below:

- Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A. - RAN#107 checkpoint: Depending on the feedback from RAN1 and RAN2 on the LS in R4-2419902, further WID updates may be needed to address feedback on specifying bands for both FR1-NTN numerology and FR2-NTN numerology or for other issues such as support for a single SCS per frequency band.

As above, it is agreed to define two sets of bands for Ku-band NTN, one for FR1 numerology and other for FR2 numerology. If the feedback from RAN1 and RAN2 on the LS in R4-2419902shows that the approach for Ku band NTN is feasible from technical perspective, the approach will impact on the following system parameters for Ku-band NTN.

- Definition of NTN frequency ranges

- Operating bands

- Transmission bandwidth configuration

- Minimum guardband and transmission bandwidth configuration

- SAN channel bandwidth per operating band

- Channel raster entries for each operating band

- Synchronization raster entries for each operating band

Definition of NTN frequency ranges

As per the WF for Ku-band NTN [1], the remaining issues concerning FR1 or FR2 frequency range are shown as below:

WF on numerology: Agree to define two sets of bands, one for FR1 numerology and the other for FR2 numerology. RAN4 will further discuss SCS options.

As above WF in RAN4 and updated WID for Ku-band NTN in RAN [3], RAN4 and RAN agreed to define two sets of bands for Ku-band NTN, one for FR1 numerology and other for FR2 numerology, so it is necessary to extend upper frequency of FR1-NTN to 14500MHz and lower frequency of FR2-NTN to 10700MHz, and it is reasonable to add  “unless otherwise stated” in NOTE 1 to avoid FR1 and FR1-NTN confusion in other specification since the frequency range of new FR1-NTN is not same with that of FR1 for TN. Since the frequencies in new FR1-NTN and new FR2-NTN is overlapped from 10700MHz to 14500MHz, so it is necessary to add description of the band number for NTN bands in FR1-NTN and FR2-NTN to indicate the difference for bands between FR1-NTN and FR2-NTN . The definition of NTN frequency ranges for Ku-band can be as following table 2-1.

Table 2-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range (MHz) |
| --- | --- |
| FR1-NTN (NOTE 1) | 410 – 14500 |
| FR2-NTN (NOTE 2) | 10700 – 30000 |
| NOTE 1: NTN bands using band number starting with n256 defined in Table 5.2.2-1 within this frequency range are regarded as a FR1 band when references from other specifications unless otherwise stated. NOTE 2: NTN bands using band number starting with n512 defined in Table 5.2.3-1 within this frequency range are regarded as a FR2-1 band when references from other specifications unless otherwise stated. |  |

Based on above discussion, we propose,

Proposal 1: Extend upper frequency of FR1-NTN to 14500MHz and lower frequency of FR2-NTN to 10700MHz, add “unless otherwise stated” in NOTE 1 to avoid FR1 and FR1-NTN confusion in other specification, and add description of the band number for NTN bands in FR1-NTN and FR2-NTN to indicate the difference for bands between FR1-NTN and FR2-NTN, as following table 2-1.

Table 2-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range (MHz) |
| --- | --- |
| FR1-NTN (NOTE 1) | 410 – 14500 |
| FR2-NTN (NOTE 2) | 10700 – 30000 |
| NOTE 1: NTN bands using band number starting with n256 defined in Table 5.2.2-1 within this frequency range are regarded as a FR1 band when references from other specifications unless otherwise stated. NOTE 2: NTN bands using band number starting with n512 defined in Table 5.2.3-1 within this frequency range are regarded as a FR2-1 band when references from other specifications unless otherwise stated. |  |

Operating bands

As per the WF for Ku-band NTN [1], the agreements concerning bands definition are shown as below:

### Sub-topic 1-1: Defining bands by regulatory authority Agree the following band definition Table X: Band definitions for Ku band Satellite operating band; Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high; Duplex mode. nX(Note 1); 14000 MHz - 14500 MHz; 10700 MHz - 12750 MHz; FDD. nY(Note 2); 14000 MHz - 14500 MHz; 10700 MHz - 12700 MHz; FDD. NOTE 1: This band is applicable in the countries and or regions subject to or referring to CEPT ECC Decision(05)01 and ECC Decision (13)01. NOTE 2: This band is applicable in the countries and or regions referring to or subject to FCC 47 CFR part 25..

As above, RAN4 agrees to define above two operating bands for Ku-band NTN, so if we define two sets of bands for Ku-band NTN, one for FR1 numerology and other for FR2 numerology. It is sufficient to define operating bands n248 and n247 for FR1 numerology for Ku-band NTN as following Table 2-2 and define operating bands n509 and n508 for FR2 numerology for Ku-band NTN as following Table 2-3.

Table 2-2: Satellite operating bands in FR1-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n254 | 1610 MHz – 1626.5 MHz | 2483.5 MHz – 2500 MHz | FDD |
| n248 (NOTE 2) | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| n247 (NOTE3) | 14000 MHz - 14500 MHz | 10700 MHz - 12700 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from n256. NOTE 2 This band is applicable in the countries subject to CEPT ECC Decisions and ETSI Harmonized Standards. NOTE 3: This band is applicable in the countries subject to FCC 47 CFR part 25. |  |  |  |

Table 2-3: Satellite operating bands in FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n512 (NOTE 1) | 27500 - 30000 MHz | 17300 - 20200 MHz | FDD |
| n511 (NOTE 2) | 28350 - 30000 MHz | 17300 - 20200 MHz | FDD |
| n510 (NOTE 3) | 27500 - 28350 MHz | 17300 - 20200 MHz | FDD |
| n509 (NOTE 4) | 14000 - 14500 MHz | 10700 - 12750 MHz | FDD |
| n508 (NOTE 5) | 14000 - 14500 MHz | 10700 - 12700 MHz | FDD |
| NOTE 1: This band is applicable in the countries subject to CEPT ECC Decision(05)01 [14] and CEPT ECC Decision (13)01 [15]. NOTE 2: This band is applicable in the USA subject to FCC 47 CFR part 25 [16]. NOTE 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25 [16]. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202 [17]). NOTE 4: This band is applicable in the countries subject to CEPT ECC Decisions and ETSI Harmonized Standards. NOTE 5: This band is applicable in the countries subject to FCC 47 CFR part 25. |  |  |  |

Based on above discussion, we propose,

Proposal 2: Define operating bands n248 and n247 for FR1 numerology for Ku-band NTN as following Table 2-2 and define operating bands n509 and n508 for FR2 numerology for Ku-band NTN as following Table 2-3.

Table 2-2: Satellite operating bands in FR1-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n254 | 1610 MHz – 1626.5 MHz | 2483.5 MHz – 2500 MHz | FDD |
| n248 (NOTE 2) | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| n247 (NOTE3) | 14000 MHz - 14500 MHz | 10700 MHz - 12700 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from n256. NOTE 2 This band is applicable in the countries subject to CEPT ECC Decisions and ETSI Harmonized Standards. NOTE 3: This band is applicable in the countries subject to FCC 47 CFR part 25. |  |  |  |

Table 2-3: Satellite operating bands in FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n512 (NOTE 1) | 27500 - 30000 MHz | 17300 - 20200 MHz | FDD |
| n511 (NOTE 2) | 28350 - 30000 MHz | 17300 - 20200 MHz | FDD |
| n510 (NOTE 3) | 27500 - 28350 MHz | 17300 - 20200 MHz | FDD |
| n509 (NOTE 4) | 14000 - 14500 MHz | 10700 - 12750 MHz | FDD |
| n508 (NOTE 5) | 14000 - 14500 MHz | 10700 - 12700 MHz | FDD |
| NOTE 1: This band is applicable in the countries subject to CEPT ECC Decision(05)01 [14] and CEPT ECC Decision (13)01 [15]. NOTE 2: This band is applicable in the USA subject to FCC 47 CFR part 25 [16]. NOTE 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25 [16]. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202 [17]). NOTE 4: This band is applicable in the countries subject to CEPT ECC Decisions and ETSI Harmonized Standards. NOTE 5: This band is applicable in the countries subject to FCC 47 CFR part 25. |  |  |  |

Transmission bandwidth configuration

As per the updated WID for Ku-band NTN [3], the objectives concerning transmission bandwidth configurations are shown as below:

- Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing.

According to above objective of updated WID, the 10MHz, 20MHz CBW with 30kHz SCS for FR1-NTN were defined in existing TS 38.108, and the 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) CBW with 120 kHz SCS for FR2-NTN were also defined in existing TS38.108, so it is necessary to add the 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing for FR1 numerology for Ku-band. The transmission bandwidth configuration NRB for 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing for TN can be reused for Ku-band NTN as following Table 2-4. The existing transmission bandwidth configuration NRB for FR2-NTN in TS 38.108 is applicable for FR2 numerology for Ku-band as following Table 2-5.

Table 2-4: Transmission bandwidth configuration NRB for FR1-NTN

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25MHz | 30 MHz | 35MHz | 50MHz | 70MHz | 100MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | N/A | 160 | N/A | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 133 | 189 | 273 |
| 60 | N/A | 11 | 18 | 24 | N/A | 38 | N/A | N/A | N/A | N/A |

Table 2-5: Transmission bandwidth configuration NRB for FR2-NTN

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 60 | 66 | 132 | 264 | N/A |
| 120 | 32 | 66 | 132 | 264 |

Based on above discussion, we propose,

Proposal 3: Add the 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing for FR1 numerology for Ku-band.

Proposal 4: The transmission bandwidth configuration NRB for 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing for TN can be reused for FR1 numerology for Ku-band NTN as following Table 2-4.

Table 2-4: Transmission bandwidth configuration NRB for FR1-NTN

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25MHz | 30 MHz | 35MHz | 50MHz | 70MHz | 100MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | N/A | 160 | N/A | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 133 | 189 | 273 |
| 60 | N/A | 11 | 18 | 24 | N/A | 38 | N/A | N/A | N/A | N/A |

Minimum guardband and transmission bandwidth configuration

As above proposal, for adding the 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing for FR1 numerology for Ku-band, it is necessary to add the guard band for the 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing for FR1 numerology for Ku-band. The guard band for the 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing for TN can be reused for FR1 numerology for Ku-band as following Table 2-6. The existing guard band for FR2-NTN in TS 38.108 is applicable for FR2 numerology for Ku-band as following Table 2-7.

Table 2-6: Minimum guard band (kHz) (FR1-NTN)

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25MHz | 30 MHz | 35MHz | 50MHz | 70MHz | 100MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | N/A | 592.5 | N/A | N/A | N/A | N/A |
| 30 | 505 | 665 | 645 | 805 | 785 | 945 | 925 | 1045 | 965 | 845 |
| 60 | N/A | 1010 | 990 | 1330 | N/A | 1290 | N/A | N/A | N/A | N/A |

Table 2-7: Minimum guardband (kHz) (FR2-NTN)

| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| --- | --- | --- | --- | --- |
| 60 | 1210 | 2450 | 4930 | N/A |
| 120 | 1900 | 2420 | 4900 | 9860 |

Based on above discussion, we propose,

Proposal 5: The minimum guard band for 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing for TN can be reused for FR1 numerology for Ku-band NTN as following Table 2-6.

Table 2-6: Minimum guard band (kHz) (FR1-NTN)

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25MHz | 30 MHz | 35MHz | 50MHz | 70MHz | 100MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | N/A | 592.5 | N/A | N/A | N/A | N/A |
| 30 | 505 | 665 | 645 | 805 | 785 | 945 | 925 | 1045 | 965 | 845 |
| 60 | N/A | 1010 | 990 | 1330 | N/A | 1290 | N/A | N/A | N/A | N/A |

SAN channel bandwidth per operating band

As above objective of updated WID for Ku-band, for proposed band n248 and n247 for FR1 numerology for Ku-band, it is necessary to define that the 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz SAN channel bandwidths with 30 kHz subcarrier spacing are applicable for band n248 and n247 as following Table 2-7.  And for proposed band n509 and n508 for FR2 numerology for Ku-band, it is necessary to define that the 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing are applicable for band n509 and n508 as following Table 2-8. The note for 30MHz is not reasonable, and we can define applicable Tx RF, Rx RF, and demodulation requirements for 30MHz to remove this note in this work item if possible.

Table 2-7: SAN channel bandwidths and SCS per operating band in FR1-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 (NOTE) | 35 | 50 | 70 | 100 |
|  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |
| n256 | 30 |  | 10 | 15 | 20 |  |  |  |  |  |  |
|  | 60 |  | 10 | 15 | 20 |  |  |  |  |  |  |
|  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |
| n255 | 30 |  | 10 | 15 | 20 |  |  |  |  |  |  |
|  | 60 |  | 10 | 15 | 20 |  |  |  |  |  |  |
|  | 15 | 5 | 10 | 15 |  |  |  |  |  |  |  |
| n254 | 30 |  | 10 | 15 |  |  |  |  |  |  |  |
|  | 60 |  | 10 | 15 |  |  |  |  |  |  |  |
| n248 | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
| n247 | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
|  | NOTE: Deployment of 30 MHz channel bandwidth for NTN SAN needs to be preceded by introduction of all applicable Tx RF, Rx RF, and demodulation requirements. |  |  |  |  |  |  |  |  |  |  |

Table 2-8: SAN channel bandwidths and SCS per operating band in FR2-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n512 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n511 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n510 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n509 | 120 | 50 | 100 | 200 | 400 |
| n508 | 120 | 50 | 100 | 200 | 400 |

Based on above discussion, we propose,

Proposal 6: Define SAN channel bandwidth and SCS for band n248 and n247 in FR1-NTN for Ku-band as following Table 2-7, and define SAN channel bandwidth and SCS for band n509 and n508 in FR2-NTN for Ku-band as following Table 2-8.

Table 2-7: SAN channel bandwidths and SCS per operating band in FR1-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 (NOTE) | 35 | 50 | 70 | 100 |
|  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |
| n256 | 30 |  | 10 | 15 | 20 |  |  |  |  |  |  |
|  | 60 |  | 10 | 15 | 20 |  |  |  |  |  |  |
|  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |
| n255 | 30 |  | 10 | 15 | 20 |  |  |  |  |  |  |
|  | 60 |  | 10 | 15 | 20 |  |  |  |  |  |  |
|  | 15 | 5 | 10 | 15 |  |  |  |  |  |  |  |
| n254 | 30 |  | 10 | 15 |  |  |  |  |  |  |  |
|  | 60 |  | 10 | 15 |  |  |  |  |  |  |  |
| n248 | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
| n247 | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
|  | NOTE: Deployment of 30 MHz channel bandwidth for NTN SAN needs to be preceded by introduction of all applicable Tx RF, Rx RF, and demodulation requirements. |  |  |  |  |  |  |  |  |  |  |

Table 2-8: SAN channel bandwidths and SCS per operating band in FR2-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n512 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n511 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n510 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n509 | 120 | 50 | 100 | 200 | 400 |
| n508 | 120 | 50 | 100 | 200 | 400 |

Channel raster entries for each operating band

The channel raster for Ku-band can be calculated according to the following equation in TS 38.104.

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs) Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster Range of frequencies (MHz); ΔFGlobal (kHz); FREF-Offs (MHz); NREF-Offs; Range of NREF. 0 – 3000; 5; 0; 0; 0 – 599999. 3000 – 24250; 15; 3000; 600000; 600000 – 2016666. 24250 – 100000; 60; 24250.08; 2016667; 2016667 – 3279165.

As above, the frequencies in Ku-band are in range of 3000-24250MHz, so the ΔFGlobal = 15kHz,  FREF-Offs =3000MHz, and  NREF-Offs =600000 are applicable for bands for Ku-band. Generally, the ΔFRaster should be an integer multiple of ΔFGlobal, so for proposed band n248 and n247 in FR1-NTN for Ku-band, ΔFRaster =15kHz is sufficient, since the smaller CBW and smaller SCS are defined for FR1 numerology. And for proposed band n509 and n508 in FR2-NTN for Ku-band, like Ka-band, the ΔFRaster =60kHz and 120kHz are reasonable, since the larger CBW and larger SCS are defined for FR2 numerology. So the channel raster for n248 and n247 in FR1-NTN is calculated as following Table 2-9, and channel raster for n509 and n508 in FR2-NTN is calculated in Table 2-11.

Table 2-9: Applicable NR-ARFCN per operating band in FR1-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 100 | 396000 – <20> – 402000 | 434000 – <20> – 440000 |
| n255 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |
| n254 | 100 | 322000 – <20> – 325300 | 496700 – <20> – 500000 |
| n248 | 15 | 1333334 – <1> – 1366666 | 1113334 – <1> – 1250000 |
| n247 | 15 | 1333334 – <1> – 1366666 | 1113334 – <1> – 1246666 |

Table 2-10: Applicable NR-ARFCN per operating band for enhanced channel raster

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 10 | 396000 – <2> – 402000 | 434000 – <2> – 440000 |
| n255 | 10 | 325300 – <2> – 332100 | 305000 – <2> – 311800 |
| n254 | 10 | 322000 – <2> – 325300 | 496700 – <2> – 500000 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. These channel numbers shall also be such that the minimum guard band for each channel bandwidth and SCS specified in Table 5.3.3-1 are met for carriers located at the upper or lower edge of an operating band. |  |  |  |

Table 2-11: Applicable NR-ARFCN per operating band in FR2-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 60 | 2070833 – <1> – 2112499 | 1553336 – <4> – 1746664 |
|  | 120 | 2070833 – <2> – 2112499 | 1553336 – <8> – 1746664 |
| n511 | 60 | 2084999 – <1> –2112499 | 1553336 – <4> – 1746664 |
|  | 120 | 2084999 – <2> –2112499 | 1553336 – <8> – 1746664 |
| n510 | 60 | 2070833 – <1> – 2084999 | 1553336 – <4> – 1746664 |
|  | 120 | 2070833 – <2> – 2084999 | 1553336 – <8> – 1746664 |
| n509 | 60 | 1333336 – <4> – 1366664 | 1113336 – <4> – 1250000 |
|  | 120 | 1333336 – <8> – 1366664 | 1113336 – <8> – 1250000 |
| n508 | 60 | 1333336 – <4> – 1366664 | 1113336 – <4> – 1246664 |
|  | 120 | 1333336 – <8> – 1366664 | 1113336 – <8> – 1246664 |

Based on above discussion, we propose,

Proposal 7: Define channel raster for n248 and n247 in FR1-NTN with ΔFRaster =15kHz as following Table 2-9, and define channel raster for n509 and n508 in FR2-NTN with ΔFRaster =60kHz and 120kHz as following Table 2-11.

Table 2-9: Applicable NR-ARFCN per operating band in FR1-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 100 | 396000 – <20> – 402000 | 434000 – <20> – 440000 |
| n255 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |
| n254 | 100 | 322000 – <20> – 325300 | 496700 – <20> – 500000 |
| n248 | 15 | 1333334 – <1> – 1366666 | 1113334 – <1> – 1250000 |
| n247 | 15 | 1333334 – <1> – 1366666 | 1113334 – <1> – 1246666 |

Table 2-11: Applicable NR-ARFCN per operating band in FR2-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 60 | 2070833 – <1> – 2112499 | 1553336 – <4> – 1746664 |
|  | 120 | 2070833 – <2> – 2112499 | 1553336 – <8> – 1746664 |
| n511 | 60 | 2084999 – <1> –2112499 | 1553336 – <4> – 1746664 |
|  | 120 | 2084999 – <2> –2112499 | 1553336 – <8> – 1746664 |
| n510 | 60 | 2070833 – <1> – 2084999 | 1553336 – <4> – 1746664 |
|  | 120 | 2070833 – <2> – 2084999 | 1553336 – <8> – 1746664 |
| n509 | 60 | 1333336 – <4> – 1366664 | 1113336 – <4> – 1250000 |
|  | 120 | 1333336 – <8> – 1366664 | 1113336 – <8> – 1250000 |
| n508 | 60 | 1333336 – <4> – 1366664 | 1113336 – <4> – 1246664 |
|  | 120 | 1333336 – <8> – 1366664 | 1113336 – <8> – 1246664 |

Synchronization raster entries for each operating band

As per the updated WID for Ku-band NTN [3], the objectives of WID is to define 30kHz SCS for FR1 numerology for Ku-band and 120kHz SCS for FR2 numerology for Ku-band. Since only 30kHz SCS for FR1 numerology is needed to be defined in WID, it is reasonable to define 30kHz SS block SCS for for FR1 numerology for Ku-band. According to TS 38.104, case B SS Block pattern for 30kHz SS block SCS is more applicable for FDD.  So it is sufficient to define 30kHz SS block SCS with case B SS Block pattern for sync raster for proposed n248 and n247 for FR1 numerology for Ku-band as following Table 2-12.

Proposal 8: Define 30kHz SS block SCS with case B SS Block pattern for sync raster for FR1 numerology for Ku-band since only 30kHz SCS for FR1 numerology is needed to be defined in WID.

and it is sufficient to define 120kHz SS block SCS with case D SS Block pattern and 240kHz SS Block SCS with case E SS Block pattern for proposed n509 and n508 for FR2 numerology for Ku-band as following Table 2-13. We can use the excel file for sync raster in 38817-01-f50.zip file to calculate sync raster for Ku-band, and the detail calculation is in attached excel file.

Table 2-12: Applicable SS raster entries per operating band (FR1-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n254 | 15 kHz | Case A | 6215 – <1> – 6244 |
|  | 30kHz | Case C | 6218 – <1> – 6241 |
| n248 | 30kHz | Case B | 12850 – <1> – 14266 |
| n247 | 30kHz | Case B | 12850 – <1> – 14232 |
| NOTE: SS Block pattern is defined in clause 4.1 in TS 38.213 [7]. |  |  |  |

Table 2-13: Applicable SS raster entries per operating band (FR2-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | (Case E | 17472 – <24> – 19416 |
| n511 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n510 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n509 | 120 kHz | Case D | 12864 – <12> – 14256 |
|  | 240 kHz | Case E | 12888 – <24> – 14232 |
| n508 | 120 kHz | Case D | 12864 – <12> – 14220 |
|  | 240 kHz | Case E | 12888 – <24> – 14208 |
| NOTE: SS Block pattern is defined in section 4.1 in TS 38.213 [7]. |  |  |  |

Based on above, we propose,

Proposal 9: Define sync raster for n248 and n247 in FR1-NTN with 30kHz SS Bloc SCS (Case B SS Block pattern) as following Table 2-12, and define sync raster for n509 and n508 in FR2-NTN with 120kHz SS block SCS(case D SS Block pattern) and 240kHz SS Block SCS (case E SS Block pattern) as following Table 2-13.

Table 2-12: Applicable SS raster entries per operating band (FR1-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n254 | 15 kHz | Case A | 6215 – <1> – 6244 |
|  | 30kHz | Case C | 6218 – <1> – 6241 |
| n248 | 30kHz | Case B | 12850 – <1> – 14266 |
| n247 | 30kHz | Case B | 12850 – <1> – 14232 |
| NOTE: SS Block pattern is defined in clause 4.1 in TS 38.213 [7]. |  |  |  |

Table 2-13: Applicable SS raster entries per operating band (FR2-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | (Case E | 17472 – <24> – 19416 |
| n511 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n510 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n509 | 120 kHz | Case D | 12864 – <12> – 14256 |
|  | 240 kHz | Case E | 12888 – <24> – 14232 |
| n508 | 120 kHz | Case D | 12864 – <12> – 14220 |
|  | 240 kHz | Case E | 12888 – <24> – 14208 |
| NOTE: SS Block pattern is defined in section 4.1 in TS 38.213 [7]. |  |  |  |

- Conclusion

This contribution provides analysis on system parameters for Ku-band for NR NTN, and the following proposals and observations are concluded as follows:

Proposal 1: Extend upper frequency of FR1-NTN to 14500MHz and lower frequency of FR2-NTN to 10700MHz, add “unless otherwise stated” in NOTE 1 to avoid FR1 and FR1-NTN confusion in other specification, and add description of the band number for NTN bands in FR1-NTN and FR2-NTN to indicate the difference for bands between FR1-NTN and FR2-NTN, as following table 2-1.

Table 2-1: Definition of NTN frequency ranges

| Frequency range designation | Corresponding frequency range (MHz) |
| --- | --- |
| FR1-NTN (NOTE 1) | 410 – 14500 |
| FR2-NTN (NOTE 2) | 10700 – 30000 |
| NOTE 1: NTN bands using band number starting with n256 defined in Table 5.2.2-1 within this frequency range are regarded as a FR1 band when references from other specifications unless otherwise stated. NOTE 2: NTN bands using band number starting with n512 defined in Table 5.2.3-1 within this frequency range are regarded as a FR2-1 band when references from other specifications unless otherwise stated. |  |

Proposal 2: Define operating bands n248 and n247 for FR1 numerology for Ku-band NTN as following Table 2-2 and define operating bands n509 and n508 for FR2 numerology for Ku-band NTN as following Table 2-3.

Table 2-2: Satellite operating bands in FR1-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n254 | 1610 MHz – 1626.5 MHz | 2483.5 MHz – 2500 MHz | FDD |
| n248 (NOTE 2) | 14000 MHz - 14500 MHz | 10700 MHz - 12750 MHz | FDD |
| n247 (NOTE3) | 14000 MHz - 14500 MHz | 10700 MHz - 12700 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from n256. NOTE 2 This band is applicable in the countries subject to CEPT ECC Decisions and ETSI Harmonized Standards. NOTE 3: This band is applicable in the countries subject to FCC 47 CFR part 25. |  |  |  |

Table 2-3: Satellite operating bands in FR2-NTN

| Satellite operating band | Uplink (UL) operating band SAN receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band SAN transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n512 (NOTE 1) | 27500 - 30000 MHz | 17300 - 20200 MHz | FDD |
| n511 (NOTE 2) | 28350 - 30000 MHz | 17300 - 20200 MHz | FDD |
| n510 (NOTE 3) | 27500 - 28350 MHz | 17300 - 20200 MHz | FDD |
| n509 (NOTE 4) | 14000 - 14500 MHz | 10700 - 12750 MHz | FDD |
| n508 (NOTE 5) | 14000 - 14500 MHz | 10700 - 12700 MHz | FDD |
| NOTE 1: This band is applicable in the countries subject to CEPT ECC Decision(05)01 [14] and CEPT ECC Decision (13)01 [15]. NOTE 2: This band is applicable in the USA subject to FCC 47 CFR part 25 [16]. NOTE 3: This band is applicable for Earth Station operations in the USA subject to FCC 47 CFR part 25 [16]. FCC rules currently do not include ESIM operations in this band (47 CFR 25.202 [17]). NOTE 4: This band is applicable in the countries subject to CEPT ECC Decisions and ETSI Harmonized Standards. NOTE 5: This band is applicable in the countries subject to FCC 47 CFR part 25. |  |  |  |

Proposal 3: Add the 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing for FR1 numerology for Ku-band.

Proposal 4: The transmission bandwidth configuration NRB for 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing for TN can be reused for FR1 numerology for Ku-band NTN as following Table 2-4.

Table 2-4: Transmission bandwidth configuration NRB for FR1-NTN

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25MHz | 30 MHz | 35MHz | 50MHz | 70MHz | 100MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | N/A | 160 | N/A | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 133 | 189 | 273 |
| 60 | N/A | 11 | 18 | 24 | N/A | 38 | N/A | N/A | N/A | N/A |

Proposal 5: The minimum guard band for 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing for TN can be reused for FR1 numerology for Ku-band NTN as following Table 2-6.

Table 2-6: Minimum guard band (kHz) (FR1-NTN)

| SCS (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25MHz | 30 MHz | 35MHz | 50MHz | 70MHz | 100MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | 242.5 | 312.5 | 382.5 | 452.5 | N/A | 592.5 | N/A | N/A | N/A | N/A |
| 30 | 505 | 665 | 645 | 805 | 785 | 945 | 925 | 1045 | 965 | 845 |
| 60 | N/A | 1010 | 990 | 1330 | N/A | 1290 | N/A | N/A | N/A | N/A |

Proposal 6: Define SAN channel bandwidth and SCS for band n248 and n247 in FR1-NTN for Ku-band as following Table 2-7, and define SAN channel bandwidth and SCS for band n509 and n508 in FR2-NTN for Ku-band as following Table 2-8.

Table 2-7: SAN channel bandwidths and SCS per operating band in FR1-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 25 | 30 (NOTE) | 35 | 50 | 70 | 100 |
|  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |
| n256 | 30 |  | 10 | 15 | 20 |  |  |  |  |  |  |
|  | 60 |  | 10 | 15 | 20 |  |  |  |  |  |  |
|  | 15 | 5 | 10 | 15 | 20 |  |  |  |  |  |  |
| n255 | 30 |  | 10 | 15 | 20 |  |  |  |  |  |  |
|  | 60 |  | 10 | 15 | 20 |  |  |  |  |  |  |
|  | 15 | 5 | 10 | 15 |  |  |  |  |  |  |  |
| n254 | 30 |  | 10 | 15 |  |  |  |  |  |  |  |
|  | 60 |  | 10 | 15 |  |  |  |  |  |  |  |
| n248 | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
| n247 | 30 |  | 10 |  | 20 | 25 |  | 35 | 50 | 70 | 100 |
|  | NOTE: Deployment of 30 MHz channel bandwidth for NTN SAN needs to be preceded by introduction of all applicable Tx RF, Rx RF, and demodulation requirements. |  |  |  |  |  |  |  |  |  |  |

Table 2-8: SAN channel bandwidths and SCS per operating band in FR2-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n512 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n511 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n510 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n509 | 120 | 50 | 100 | 200 | 400 |
| n508 | 120 | 50 | 100 | 200 | 400 |

Proposal 7: Define channel raster for n248 and n247 in FR1-NTN with ΔFRaster =15kHz as following Table 2-9, and define channel raster for n509 and n508 in FR2-NTN with ΔFRaster =60kHz and 120kHz as following Table 2-11.

Table 2-9: Applicable NR-ARFCN per operating band in FR1-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 100 | 396000 – <20> – 402000 | 434000 – <20> – 440000 |
| n255 | 100 | 325300 – <20> – 332100 | 305000 – <20> – 311800 |
| n254 | 100 | 322000 – <20> – 325300 | 496700 – <20> – 500000 |
| n248 | 15 | 1333334 – <1> – 1366666 | 1113334 – <1> – 1250000 |
| n247 | 15 | 1333334 – <1> – 1366666 | 1113334 – <1> – 1246666 |

Table 2-11: Applicable NR-ARFCN per operating band in FR2-NTN

| SAN operating band | ΔFRaster (kHz) | Uplink range of NREF (First – <Step size> – Last) | Downlink range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 60 | 2070833 – <1> – 2112499 | 1553336 – <4> – 1746664 |
|  | 120 | 2070833 – <2> – 2112499 | 1553336 – <8> – 1746664 |
| n511 | 60 | 2084999 – <1> –2112499 | 1553336 – <4> – 1746664 |
|  | 120 | 2084999 – <2> –2112499 | 1553336 – <8> – 1746664 |
| n510 | 60 | 2070833 – <1> – 2084999 | 1553336 – <4> – 1746664 |
|  | 120 | 2070833 – <2> – 2084999 | 1553336 – <8> – 1746664 |
| n509 | 60 | 1333336 – <4> – 1366664 | 1113336 – <4> – 1250000 |
|  | 120 | 1333336 – <8> – 1366664 | 1113336 – <8> – 1250000 |
| n508 | 60 | 1333336 – <4> – 1366664 | 1113336 – <4> – 1246664 |
|  | 120 | 1333336 – <8> – 1366664 | 1113336 – <8> – 1246664 |

Proposal 8: Define 30kHz SS block SCS with case B SS Block pattern for sync raster for FR1 numerology for Ku-band since only 30kHz SCS for FR1 numerology is needed to be defined in WID.

Proposal 9: Define sync raster for n248 and n247 in FR1-NTN with 30kHz SS Bloc SCS (Case B SS Block pattern) as following Table 2-12, and define sync raster for n509 and n508 in FR2-NTN with 120kHz SS block SCS(case D SS Block pattern) and 240kHz SS Block SCS (case E SS Block pattern) as following Table 2-13.

Table 2-12: Applicable SS raster entries per operating band (FR1-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n254 | 15 kHz | Case A | 6215 – <1> – 6244 |
|  | 30kHz | Case C | 6218 – <1> – 6241 |
| n248 | 30kHz | Case B | 12850 – <1> – 14266 |
| n247 | 30kHz | Case B | 12850 – <1> – 14232 |
| NOTE: SS Block pattern is defined in clause 4.1 in TS 38.213 [7]. |  |  |  |

Table 2-13: Applicable SS raster entries per operating band (FR2-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | (Case E | 17472 – <24> – 19416 |
| n511 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n510 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n509 | 120 kHz | Case D | 12864 – <12> – 14256 |
|  | 240 kHz | Case E | 12888 – <24> – 14232 |
| n508 | 120 kHz | Case D | 12864 – <12> – 14220 |
|  | 240 kHz | Case E | 12888 – <24> – 14208 |
| NOTE: SS Block pattern is defined in section 4.1 in TS 38.213 [7]. |  |  |  |

- References

- R4-2419909, Way Forward for [112][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#113

- R4-2419902, LS on Ku band numerology, RAN WG4, RAN4#113

- R4-2416593, Way Forward for [112bis][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#112bis

- R4-2317374,RRM WF and CR work split for NR_NTN_enh,Moderator (Qualcomm Incorporated),RAN4#108bis

- R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#112

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, RAN#104