3GPP TSG RAN WG4 Meeting #114		R4-2501061

Athens, Greece, Feb 17th – 21st , 2025

Title: 	Discussion on BS RF requirements for 7MHz channel bandwidth

Agenda Item:	7.9.4

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN#106, a last-minute work item to introduce a new 7MHz channel bandwidth for band n5 and n26 was approved due to the strong and urgent commercial demand [1]. The following table shows the core part objectives:

The core part of this WI includes: - Specify the generic core requirements to support 7 MHz NR FR1 channel bandwidth [RAN4] - Introduce the channel bandwidth 7MHz - Only include 15 kHz SCS - Specify the spectrum utilization - Specify UE RF requirements for UL and DL - Specify BS RF requirements for UL and DL - Specify the band specific core requirements to introduce 7 MHz for NR band n5, n26. [RAN4] - BS and UE Channel bandwidth per operating band - UE reference sensitivity - When needed: - MPR (relative bandwidth criteria) - Additional Maximum Power Reduction (A-MPR) - Enable, if feasible, optional UE support of 7 MHz channel bandwidth from Rel-15 onwards in a release independent manner. [RAN2, RAN4] - No impact to 38.307 is expected. - Once RAN4 has introduced the new channel bandwidth, RAN2 will introduce the necessary signalling for this channel bandwidth. [RAN2] Note: This should be similar to the signalling for 35 and 45 MHz channel bandwidths. This signalling should take into account that also additional bands (not considered by this WI) may consider this new channel bandwidth in the future. RAN2 will be informed by RAN4 via LS. No additional RAN2 TUs will be requested for this objective. - Once RAN4 has introduced the new channel bandwidth, RAN3 will introduce the necessary signalling for this channel bandwidth [RAN3] Note: This should be similar to the signalling for 35 and 45 MHz channel bandwidths. This signalling should take into account that also additional bands (not considered by this WI) may consider this new channel bandwidth in the future. RAN3 will be informed by RAN4 via LS. No additional RAN3 TUs will be requested for this objective. NOTE: Once this WI is completed, the 7 MHz bandwidth can be introduced also for other bands (not addressed in this WI) via the appropriate basket WI.

In this contribution, we discuss the corresponding BS RF requirements for this new channel bandwidth.

# 2 Discussion

For maximum transmission bandwidth configuration, BS should have the same as UE side, and our companion paper proposes to set 35 PRBs for 7MHz channel bandwidth with SCS 15kHz. And the corresponding guard-band size is 342.5kHz.

Proposal 1: RAN4 to specify maximum transmission bandwidth configuration as 35 and the corresponding guard-band size as 342.5kHz for 7MHz channel bandwidth with SCS 15kHz.

For REFSENS, the situation is different from that at US side, which means that REFSENS is not scaled by channel bandwidth. And in the current TS 38.104, 5/10/15MHz channel bandwidths have the same REFSENS for different BS types and classes. Hence it is reasonable to set the same level as that for 5/10/15MHz.

Proposal 2: RAN4 to specify REFSENS as the same as that for channel bandwidths 5/10/15MHz.

The below tables illustrate the changes accordingly.

BS type 1-C and 1-H

For wide area BS:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 3 | 15 | G-FR1-A1-7 (Note 1) | -103.6 |
|  |  | G-FR1-A1-21 (Note 6) | -103.6 |
| 7 | 15 | G-FR1-A1-1 (Note 1) | -101.7 |
| 5, 10, 15 | 15 | G-FR1-A1-1 (Note 1) | -101.7 |
|  |  | G-FR1-A1-10 (Note 3) | -101.7 (Note 2) |

For middle range BS:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (Note 5) | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 3 | 15 | G-FR1-A1-7 (Note 1) | -98.6 |
|  |  | G-FR1-A1-21 (Note 6) | -98.6 |
| 7 | 15 | G-FR1-A1-1 (Note 1) | -96.7 |
| 5, 10, 15 | 15 | G-FR1-A1-1 (Note 1) | -96.7 |
|  |  | G-FR1-A1-10 (Note 3) | -96.7 (Note 2) |

For local area BS:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel (Note 5) | Reference sensitivity power level, PREFSENS (dBm) |
| --- | --- | --- | --- |
| 3 | 15 | G-FR1-A1-7 (Note 1) | -95.6 |
|  |  | G-FR1-A1-21 (Note 6) | -95.6 |
| 7 | 15 | G-FR1-A1-1 (Note 1) | -93.7 |
| 5, 10, 15 | 15 | G-FR1-A1-1 (Note 1) | -93.7 |
|  |  | G-FR1-A1-10 (Note 3) | -93.7 (Note 2) |

BS type 1-O

For WA BS:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 7, 10, 15 | 15 | G-FR1-A1-1 | -101.7 - ΔOTAREFSENS |

For MR BS:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 7, 10, 15 | 15 | G-FR1-A1-1 | -96.7 - ΔOTAREFSENS |

For LA BS:

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 5, 7, 10, 15 | 15 | G-FR1-A1-1 | -93.7 - ΔOTAREFSENS |

For Rx dynamic range, the current specs indicates that the interfering signal mean power is scaled with channel bandwidth for conducted requirement, which means a scaling factor of 1.5dB applied for 7MHz channel bandwidth on top of that for 5MHz channel bandwidth, i.e., -81dBm for WA, -76dBm for MR and -73dBm for LA.

Proposal 3: For conducted Rx dynamic range, RAN4 to specify the interfering signal mean power as -81dBm for WA, -76dBm for MR and -73dBm for LA.

The following tables show the required changes.

BS Type 1-C and 1-H:

For WA BS:

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 3 | 15 | G-FR1-A2-15 | -73.6 | -84.7 | AWGN |
| 5 | 15 | G-FR1-A2-1 | -70.7 | -82.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4 |  |  |
| 7 | 15 | G-FR1-A2-1 | -70.7 | -81 | AWGN |
| 10 | 15 | G-FR1-A2-1 | -70.7 | -79.3 | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4 |  |  |
|  | 60 | G-FR1-A2-3 | -68.4 |  |  |
| 15 | 15 | G-FR1-A2-1 | -70.7 | -77.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4 |  |  |
|  | 60 | G-FR1-A2-3 | -68.4 |  |  |

For MR BS:

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel (Note 2) | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 3 | 15 | G-FR1-A2-15 | -68.6 | -79.7 | AWGN |
| 5 | 15 | G-FR1-A2-1 | -65.7 | -77.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -66.4 |  |  |
| 7 | 15 | G-FR1-A2-1 | -65.7 | -76 | AWGN |
| 10 | 15 | G-FR1-A2-1 | -65.7 | -74.3 | AWGN |
|  | 30 | G-FR1-A2-2 | -66.4 |  |  |
|  | 60 | G-FR1-A2-3 | -63.4 |  |  |

For LA BS:

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 3 | 15 | G-FR1-A2-15 | -65.6 | -76.7 | AWGN |
| 5 | 15 | G-FR1-A2-1 | -62.7 | -74.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -63.4 |  |  |
| 7 | 15 | G-FR1-A2-1 | -62.7 | -73 | AWGN |
| 10 | 15 | G-FR1-A2-1 | -62.7 | -71.3 | AWGN |
|  | 30 | G-FR1-A2-2 | -63.4 |  |  |
|  | 60 | G-FR1-A2-3 | -60.4 |  |  |

And similarly, for radiated requirements, the interfering signal mean power could also be scaled as -81- ΔOTAREFSENS for WA, -76- ΔOTAREFSENS for MR, and -73- ΔOTAREFSENS for LA.

Proposal 4: For radiated Rx dynamic range, RAN4 to specify the interfering signal mean power as -81- ΔOTAREFSENS for WA, -76- ΔOTAREFSENS for MR, and -73- ΔOTAREFSENS for LA.

The below tables illustrate the required changes.

WA BS:

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -70.7- ΔOTAREFSENS | -82.5- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4- ΔOTAREFSENS |  |  |
| 7 | 15 | G-FR1-A2-1 | -70.7- ΔOTAREFSENS | -81- ΔOTAREFSENS | AWGN |
| 10 | 15 | G-FR1-A2-1 | -70.7- ΔOTAREFSENS | -79.3- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4- ΔOTAREFSENS |  |  |

MR BS:

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -65.7- ΔOTAREFSENS | -77.5- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -66.4- ΔOTAREFSENS |  |  |
| 7 | 15 | G-FR1-A2-1 | -65.7- ΔOTAREFSENS | -76- ΔOTAREFSENS | AWGN |
| 10 | 15 | G-FR1-A2-1 | -65.7- ΔOTAREFSENS | -74.3- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -66.4- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -63.4- ΔOTAREFSENS |  |  |

LA BS:

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -62.7- ΔOTAREFSENS | -74.5- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -64.4- ΔOTAREFSENS |  |  |
| 7 | 15 | G-FR1-A2-1 | -62.7- ΔOTAREFSENS | -73- ΔOTAREFSENS | AWGN |
| 10 | 15 | G-FR1-A2-1 | -62.7- ΔOTAREFSENS | -71.3- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -64.4- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -60.4- ΔOTAREFSENS |  |  |

# 3 Conclusion

In this contribution, we have the following proposals for BS RF requirements for 7MHz channel bandwidth:

Proposal 1: RAN4 to specify maximum transmission bandwidth configuration as 35 and the corresponding guard-band size as 342.5kHz for 7MHz channel bandwidth with SCS 15kHz.

Proposal 2: RAN4 to specify REFSENS as the same as that for channel bandwidths 5/10/15MHz.

Proposal 3: For conducted Rx dynamic range, RAN4 to specify the interfering signal mean power as -81dBm for WA, -76dBm for MR and -73dBm for LA.

Proposal 4: For radiated Rx dynamic range, RAN4 to specify the interfering signal mean power as -81- ΔOTAREFSENS for WA, -76- ΔOTAREFSENS for MR, and -73- ΔOTAREFSENS for LA.

# Reference

[1] RP-243323, “New WID on 7 MHz Channel Bandwidth for n26 and n5”, China Telecom, T-Mobile USA

[2] R4-2501060, “Discussion on spectrum utilization and UE RF requirements for 7MHz channel bandwidth”, CATT