# 3GPP TSG-RAN WG4 Meeting #114                                R4-2501097

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.20.2

Source: 	CMCC

Title: 			 Discussion on testing framework and procedure for one-sided models

Document for:	Discussion

- Introduction

In RAN #106 meeting, the SID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. The objectives for RAN4 are duplicated as following.

- Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [2][3]. This contribution provides discussion on testing framework and procedure for one-sided models.

- Discussion

Issue 3-6: Measurement error impact evaluation - Proposals - Option 1: model RF error as same within an RF module but varies across modules, FFS the distribution - Option 2: Gaussian distribution with +/- 2.5dB for BB errors and total of +/-6.5 for BB+RF - Option 3: model RF error as - An offset value in the original dataset; - Truncated Uniform distribution; - Truncated Gaussian distribution.

According to the agreed simulation assumptions for beam prediction, simulation assumptions for baseband error and RF error are also covered. For RF error, ±4.5dB is considered. As for the detail about how to generate RF errors, e.g. independent for each TX beam, or same for each TX beam, it is still open. This issue is more related with UE implementation, input from UE vendors is appreciated. As for baseband error, there are two options about how to apply it to the SLS. Option 1 is to apply error distribution from the post beamforming SNR of the strongest Tx-Rx beam pair to all Tx-Rx beam pair’s measurements in the SLS. Option 2 is to generate error distribution separately for each individual Tx-Rx beam pair’s SNRs and apply to the corresponding Tx-Rx beam pair measurement in the SLS. According to the initial simulation results from companies, the performance is different between the two options. From practical point of view, the error is different from different SNRs. It is better to allign with the real deployment. Considering that the main target is to evaluate the measurement error impact on prediction performance, no need to evaluate the impact due to BB error and RF error seperately, a single measurement error inlcudes both baseband error and RF error can be considered.

Proposal 1: for measurement error impact evaluation, it is proposed to consider a single error covering both baseband error and RF error. And it is proposed to consider baseband error under different SNR.

Issue 3-9: AoA/AoD - Proposals - Option 1: exclude test setup with single AoA(single probe) - Option 2: further study single AoA setup: - Companies should bring further analysis on relevance of such a test to beam prediction and how it would relate to field deployments - Option 3: other

Acorrding to previous discussion, some companies propose only to verify AI funtionality, since the model input is RSRP. We have different views, not only the functionality but also the prediction performance need to be verified. It is necessary to consider UE RX beam sweeping. From this point of view, multiple AoAs is preferred.

Proposal 2: to verify beam management performance, it is proposed to use multiple AoAs test systems.

Issue 3-11: Channel model for beam prediction - Proposals - Option 1: multi-AoA TDL - Option 2: simplified CDL - Option 3: further study both multi-AoA TDL and simplified CDL - Option 4: CDL without simplification - Option 5: others

Spatial/time domain prediction require to learn the sufficient spatial/time domain characteristic, which means that sufficient randomness and variation in time and spatial domain in the test is necessary. Compared with TDL, CDL channel provides more spatial characteristics and reflects field conditions accurately, and is more suitable for beam prediction testing. However, CDL channel requires multiple probes, and the test complexity and cost are increasing. Based on above consideration, we are also fine with simplified CDL with small number of clusters.

Proposal 3: for beam prediction testing, it is prefered to consider CDL, but we are also fine with simplified CDL.

WF in RAN4 #112bis: Issue 2-11: Beam prediction testing process - Proposals - Option 1: Test needs to have at least 2 parts: - 1. inference part of the test (UE has to make the actual prediction) - 2. predicted beam measurement (UE has to measure and report “the ground truth”) - Option 2: others

Since reported RSRP measurement is impacted by RX beamforming gain which is up to UE implementation, it was agreed in RAN4 #111 meeting that the ground truth for FR2 is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam. And assumption is that SNR is high enough for sufficient accuracy of reported RSRP.

Agreements in RAN4 #111 meeting Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded

With above agreements, during the test, it is necessary to have the procedure to get the ground truth. And it is straightforward that there is a part for prediction. Of course, the final KPI for BM is not conclided, if absolute RSRP accuracy is agreed as test metric, Option 1 can be used as baseline that test for BM at least include two parts: the part to get ground truth, and the part for prediction performance verification.

Proposal 4: If the absolute RSRP is agreed to be metric for the beam management prediction, it is proposed that tests for BM at least include two parts: the part to get ground truth, and the part for prediction verification.

- Conclusion

This contribution provides discussion on testing framework and procedure for one-sided models. The proposals are:

Proposal 1: for measurement error impact evaluation, it is proposed to consider a single error covering both baseband error and RF error. And it is proposed to consider baseband error under different SNR.

Proposal 2: to verify beam management performance, it is proposed to use multiple AoAs test systems.

Proposal 3: for beam prediction testing, it is prefered to consider CDL, but we are also fine with simplified CDL.

Proposal 4: If the absolute RSRP is agreed to be metric for the beam management prediction, it is proposed that tests for BM at least include two parts: the part to get ground truth, and the part for prediction verification.

- Reference

- RP-243245, New SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2

- R4-2420336, WF on AI/ML air interface part 2, Ericsson

- R4-2420513, WF on AI/ML air interface part 1, Qualcomm Incorporated

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

[5] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface