3GPP TSG-RAN WG4 Meeting #114		                 R4-2501158

Athens, GR, 17th – 21st February, 2025

Source:	Sony

Title:	REFSENS for NTN Redcap and eRedcap

Agenda item:	7.29.2.1

Document for:	Approval

# Background

The remaining RF requirement of NTN Redcap and eRedcap is on the REFSENS on n254 and n256 after the RAN4#113 [1]. In this paper, we reiterate our proposal on the REFNSES values that is feasible for those two bands.

# REFSENS for HD-FDD

In RAN4#112, it has been agreed that a 0.5 dB tightening factor is adopted for n256 for HD-FDD operation, which results in the REFSENS for 5MHz becoming -100 dBm for HD-FDD, and it aligns with the principle of how we derived the TN Redcap REFSENS.

In addition, In RAN4#111, it has also been agreed:

- [0.5] dB tightening for n254 and n256, FFS for n255. n256 to be confirmed in RAN4#112.

Observation 1: Tentative agreement has been made in RAN4#111 on the REFSENS of n254 with 0.5 dB tightening.

The 0.5 dB tightening on n254 and n256 followed the same principle as in the TN case, which was originally proposed in [2], and the principle is summarized in the table below:

| FD-FDD 5MHz REFSENS | HD-FDD REFSENS Tightening | Bands |
| --- | --- | --- |
| ≤ -100 dBm | 0 dB | n1, n18, n24, n70 |
| > -100 dBm and ≤ -99 dBm | 0.5 dB | n30, n65, n66, n74 |
| > -99 dBm | 0.8 dB | n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n28, n71, n85 |

In our view, the same approach should be taken as in the TN case so that the RF front end of TN Redcap and NTN Redcap can be largely reused to facilitate the cost and time-to-market of Redcap/eRedcap NTN system.

Observation 2: Adopting the same principle in defining the TN Redcap and NTN Redcap REFSENS can facilitate the time-to-market of the Redcap NTN system and reduce its cost.

From the coverage perspective, it is our understanding that the uplink coverage is the bottleneck for both legacy TN Redcap and legacy NTN networks. Therefore, further stringent REFSENS requirements may not significantly benefit network coverage.

Observation 3: Making the REFSENS for HD-FDD NTN Redcap more stringent may not significantly benefit network coverage as the uplink is usually the bottleneck.

Therefore, it is proposed that the same approach can also be used to derive the REFSENS for NTN Redcap HD-FDD, where RAN4 shall confirm 0.5 dB tightening for n254 and adopt 0 dB tightening for n255.

Proposal 1: RAN4 confirms the 0.5 dB tightening for n254 and 0 dB tightening for n255 for the REFSENS of HD-FDD Redcap NTN, e.g., -100 dBm for 5MHz BW for n255 and n254.

# Conclusion

In this paper, we presented our view on the RF requirement for NTN Redcap and eRedcap, where the following observations and proposals are made:

Observation 1: Tentative agreement has been made in RAN4#111 on the REFSENS of n254 with 0.5 dB tightening.

Observation 2: Adopting the same principle in defining the TN Redcap and NTN Redcap REFSENS can facilitate the time-to-market of the Redcap NTN system and reduce its cost.

Observation 3: Making the REFSENS for HD-FDD NTN Redcap more stringent may not significantly benefit network coverage as the uplink is usually the bottleneck.

Proposal 1: RAN4 confirms the 0.5 dB tightening for n254 and 0 dB tightening for n255 for the REFSENS of HD-FDD Redcap NTN, e.g., -100 dBm for 5MHz BW for n255 and n254.

# References

- R4-2419897 Way Forward for [113][310] NR_NTN_Ph3_UE_RF	Qualcomm