# 3GPP TSG-RAN WG4 Meeting #114	R4-2501159

# Athens, GR, 17th – 21st February, 2025

Source:	Sony

Title:	OCC scheme on the RF requirements for IoT NTN

Agenda item:	7.30.2

Document for:	Discussion

# Background

In RAN4#113, the impact of OCC scheme on IoT NTN RF requirements has been further discussed. Based on the progress in RAN1 and RAN4, it has been concluded that there will be no RF requirement impact for OCC-2 with the symbol level for 3.75 kHz SCS, and the UE RF requirement impact from 15 kHz single-tone NPUSCH with slot-level OCC2 is pending on RAN1 progress of resource mapping.

In addition, two other issues, including the gap in the middle of the OCC scheme and the mechanism to allow UE resync to avoid high and opposite frequency inaccuracy between a pair of OCC UEs, were discussed, with the following agreements made [1].:

#### Issue 1-5: Whether there would be UE RF requirement impact when OCC schemes would cross gap in UL transmission.

- Agreement:

  - Whether to study the case that gap between OCC slots may affect RF requirement pending on RAN1.

#### Issue 1-6: Whether UE would need to resynchronize its TX frequency periodically to avoid high and opposite frequency inaccuracy between a pair of OCC UEs.

- Agreement:

  - Further discuss whether UE would need to resynchronize its TX frequency periodically to avoid high and opposite frequency inaccuracy between a pair of OCC UEs.

In this contribution, we provide our further views on the OCC scheme crossing a gap and also how the UE can resync periodically to avoid high and opposite frequency inaccuracy between a pair of OCC UEs.

# OCC scheme across the gap

Whether OCC scheme would cross the gap is currently being discussed in RAN1 [2]. In particular, the following gap may happen For IoT-NTN NPUSCH format 1 transmissions:

- UL gaps for synchronization (from Rel-13)

- Gaps around NPRACH occasions

- UL timing adjustment gaps and segmentation for IoT-NTN (from Rel-17)

- TDM DMRS that are muted

Guard periods for 3.75kHz UL transmissions

During Rel-17 coverage enhancement WI, the NR UE capability to maintain the phase continuity over a gap in the UL transmission was analyzed, and it was concluded that the gap length below 14 symbols should be feasible for UE to maintain the phase continuity, and this can be translated to a gap duration of 1ms with 15kHz SCS.

For IoT devices, although the stability of the oscillator may be worse than NR UE, no fundamental difference is expected. A UL gap of around 1 ms will be possible for UE to maintain the phase continuity. Therefore, for gaps with relatively short duration, e.g., TDM DMRS that are muted and guard periods for 3.75kHz UL transmissions, it should be feasible for the OCC scheme to cross the gaps.

Observation 1: for the gaps with relatively short duration, e.g., TDM DMRS that are muted and guard periods for 3.75kHz UL transmissions, it should be feasible for the OCC scheme to cross the gaps.

For larger gaps that span over several ms, it can be expected that the phase continuity would not be able to be maintained over a gap unless the UE has some dedicated design. For example, the UE may need to buffer the phase information before the gap and use it to calibrate the phase of the UL transmission after the gap. In either case, this may not be generally applied to all UEs, and thus, a UE capability is needed if OCC crosses such gaps would be needed.

Observation 2: For larger gaps over several ms, dedicated UE implementation would be needed to support the phase continuity across the gap.

Therefore, based on the discussion above, at least from the RAN4 side, we can conclude that for a gap with a shorter duration than 1ms, it is feasible for the OCC scheme to cross the gap, while for a larger gap, if they need to be supported, dedicated UE capability shall be defined.

Proposal 1: RAN4 can conclude that from the RF aspect, for gap lengths less than 1ms, it is feasible for the OCC scheme to cross the gap.

# Resynchronize TX frequency periodically

Given that the errors in OCC transmission are most likely when paired UEs have high and opposite frequency inaccuracy (e.g., UE1’s frequency inaccuracy is +0.1ppm and UE2’s frequency inaccuracy is -0.1ppm), it would be beneficial for a UE to resynchronize its transmission frequency periodically during an ongoing transmission as this will randomize the relative error in the frequency of the UEs’ transmissions. For example, suppose a transmission consists of 16 OCC codewords, and resynchronization occurs between the first set of 8 codewords and the second set of 8 codewords. In that case, it is unlikely that both the first set and second set of OCC codewords would observe high and opposite frequency inaccuracy (e.g. the first set of OCC codewords might have inaccuracy of +/- 0.1ppm and the second set of OCC codewords might have an inaccuracy of 0/0ppm). The FEC and interleaving across the OCC codewords would then reduce the probability of block error.

Observation 3: From an RF perspective, errors in OCC transmission are most likely when paired UEs have high and opposite frequency inaccuracy (e.g., UE1’s frequency inaccuracy is +0.1ppm, and UE2’s frequency inaccuracy is -0.1ppm).

Proposal 2: Based on the RF behaviour of UE, it is beneficial if the UE can resynchronize its transmission frequency periodically to avoid high and opposite frequency inaccuracy between a pair of OCC UEs.

A small sync gap is needed to perform such a resynchronization. As discussed in Section 1, a small gap below 1 ms would not affect the OCC scheme to cross the gap. Then, the next question is how often such a resync is needed. For legacy NB-IoT UEs, a 40 ms measurement gap is scheduled after a 256 ms transmission. The 40 ms is based on the typical behavior of the frequency error estimator, so the frequency error could be corrected low enough. Some analysis has been given [3]. Therefore, with the assumption that a 40 ms measurement is needed to fix the frequency error accumulated over 256 ms, to have a gap of less than 1 ms, an accumulated error over a periodicity with 256/40 = 6.4 ms can be considered.

Proposal 3: Define a resynchronized gap of 1 ms with a periodicity of about 6.4 ms to avoid high and opposite frequency inaccuracy between a pair of OCC UEs.

If more relaxed frequency accuracy requirements could be considered in this case, a long periodicity or shorter gap duration can also be applied to minimize the impact the OCC scheme further.

Proposal 4: If more relaxed frequency accuracy requirements could be considered in this case, a long periodicity or shorter gap duration can also be applied to minimize the impact the OCC scheme further.

However, we would also like to mention that other approach to minimize the potential opposite high and opposite frequency inaccuracy between a pair of OCC UEs. For example, UE may perform multiple independent estimation before the OCC scheme starts and apply during the OCC scheme to randomize the relative errors. Those approaches can also be considered.

# Conclusion

In this contribution, we make the following observations and conclusions:

Observation 1: for the gaps with relatively short duration, e.g., TDM DMRS that are muted and guard periods for 3.75kHz UL transmissions, it should be feasible for the OCC scheme to cross the gaps.

Observation 2: For larger gaps over several ms, dedicated UE implementation would be needed to support the phase continuity across the gap.

Observation 3: From an RF perspective, errors in OCC transmission are most likely when paired UEs have high and opposite frequency inaccuracy (e.g., UE1’s frequency inaccuracy is +0.1ppm, and UE2’s frequency inaccuracy is -0.1ppm).

Proposal 1: RAN4 can conclude that from the RF aspect, for gap lengths less than 1ms, it is feasible for the OCC scheme to cross the gap.

Proposal 2: Based on the RF behaviour of UE, it is beneficial if the UE can resynchronize its transmission frequency periodically to avoid high and opposite frequency inaccuracy between a pair of OCC UEs.

Proposal 3: Define a resynchronized gap of 1 ms with a periodicity of about 6.4 ms to avoid high and opposite frequency inaccuracy between a pair of OCC UEs.

Proposal 4: If more relaxed frequency accuracy requirements could be considered in this case, a long periodicity or shorter gap duration can also be applied to minimize the impact the OCC scheme further.

# References

- R4-2419899	Way Forward for [113][315] IoT_NTN_Ph3		MediaTek

- R1-2410765	 Final FL Summary for IoT-NTN	Moderator (Sony)

- R4-163255	Simulation results of UCG parameters for NB-IoT	 Intel Corporation