3GPP TSG-RAN WG4 Meeting # 114	R4-2501226

Athens, GR, 17th – 21st February, 2025

Title: 	Discussion on maintenance for L1/L2-based inter-cell mobility

Source: 	Huawei, HiSilicon

Agenda item:	5.23.1

Document for:	Discussion

Introduction

L1/L2 based inter-cell mobility was discussed in last meeting [1]. This contribution provides further analysis on maintenance core and performance requirements on L1/L2 based inter-cell mobility from RRM requirement perspective.

Discussion

Remaining issues in core requirements

LTM Cell switch delay requirements

One open issue on LTM cell switch delay is listed as below [1].

Issue 1-3-1-2: Conditions of Tfirst-RS =0 in cell switch delay < Agreement>: Conclude the issue in RAN4#114 - Option 1: - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if - For intra-frequency case: the time gap between completion of TCI activation and cell switch command is larger than 480ms. - During the 480ms after the TCI state activation, longer L3 measurement delay may be expected. - For inter-frequency case: the time gap between completion of TCI activation and cell switch command is larger than 160ms. - Option 2: - In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if - For intra-frequency case: the time gap between completion of TCI activation and cell switch command is larger than 480ms. - For inter-frequency case: the time gap between completion of TCI activation and cell switch command is larger than 480ms with conditions - Supporters are encouraged to propose the conditions for discussion next meeting - During the 480ms after the TCI state activation, longer L3 measurement delay may be expected.

In RAN2#112bis, RAN4 agreed to extend the condition for Tfirst-RS = 0 from “the time gap between TCI state activation and the cell switch command is not more than 160 ms” to “480 ms” for UE not supporting/configured with L1 measurement in FR1. In theory for the case that L1 measurement is not supported/configured, UE would utilize normal L3 measurement procedure to acquire fine timing of the candidate cells. However legacy L3 measurement periodicity is larger than 160ms (especially in FR2), to satisfy UE timing requirement (tracking timing at least every 160ms), UE may need to perform additionally T/F tracking within 480ms duration. It shall be noted that it would put additional UE effort as UE needs to re-arrange its L3 measurement schedule. That is why there is another clarification to say “During the 480ms after the TCI state activation, longer L3 measurement delay may be expected.”

In our understanding, the above extension is only applicable to intra-frequency scenario with the expense of additional UE effort. If the candidate cells are inter-frequency cells, the measurement opportunities strictly rely on measurement gap occasion. When multiple inter-frequency layers are configured, there may no chance to additionally measure on the candidate cells. It is true in some cases, UE can track the T/F inter-frequency candidate cell within 480ms, however complicated auxiliary conditions would be further discussed. And it shall be emphasized that the extension would put UE implementation burden on measurement schedule. In a short, such kind of extension is not applied to inter-frequency case, that is, for inter-frequency case, Tfirst-RS = 0 when the time gap between completion of TCI activation and cell switch command is larger than 160ms (rather than extending to 480ms).

Proposal 1: In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking,

- For intra-frequency case: the time gap between completion of TCI activation and cell switch command is larger than 480ms.

  - During the 480ms after the TCI state activation, longer L3 measurement delay may be expected.

- For inter-frequency case: the time gap between completion of TCI activation and cell switch command is larger than 160ms (i.e., no extension).

Conditions of Fast RRC decoding

In LTM cell switch delay, TLTM_RRC-processing is the time for ASN.1 decoding and validity/compliance check for the RRC configuration of the LTM target cell indicated in the LTM cell switch command.

Issue 1-3-2-1: Which cell(s) TLTM-RRC-processing = 0 apply to when the total number of candidates triggered early TCI activation or early PDCCH order RACH exceed UE capability? <Agreement>: Conclude the issue in RAN4#114 - Option 1 (HW) - When the total number of candidates triggered early TCI activation or early PDCCH order RACH exceed UE capability, TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states (if any) or PDCCH-order PRACH transmission (if any) within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18. - The requirements are applicable to the case NW does not trigger TCI state activation or PDCCH-order RACH on different candidate cells at the same occasion, and the wording can be updated. - Option 2 (Nokia, ZTE, E///) - When the total number of candidates triggered early TCI activation or early PDCCH order RACH exceed UE capability, TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states (if any) or PDCCH-order PRACH transmission (if any) within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18. - Option 3 (Apple, HW) - The issue is closed

Regarding the conditions of TLTM-RRC-processing = 0, RAN4 has made multiple rounds of optimizations. At first, RAN4 agreed that when the number of LTM candidate cell configurations does not exceed maxNumberConfigs-r18 and the number of the configured serving cells and the cells in the LTM configuration does not exceed maxNumberStoredConfigCells-r18, the time for ASN.1 decoding and validity/compliance check can be zero. And then RAN4 further agreed that when the configured LTM configurations and cells exceed UE capability, but the LTM candidates with early TCI activation and early PDCCH order RACH does not exceed UE capability, TLTM_RRC-processing can also be zero. And now RAN4 is still discussing whether and how to further optimize the case where the total number of candidates triggered early TCI activation or early PDCCH order RACH exceed UE capability. To be honest, the further optimization would make UE implementation more complicated. In addition, the R18 mobility WI has already been declared “complete” and enormous work from R19 mobility is coming. So we suggest such further optimization is not pursued for R18 mobility.

Proposal 2: Not pursue the discussion on the further optimization on TLTM_RRC-processing =0 when the total number of candidates triggered early TCI activation or early PDCCH order RACH exceed UE capability (option 3).

Remaining issues in performance requirements

Improvement on SCell/SCG setup delay

For R18 idle mode CA/DC measurement test, the below three tests are defined [2]:

- For EMR, TC verified UE does not send measurement report which is measured exceeds X second before paging reception.

- For non-EMR,

  - TC verified that UE send measurement report which is measured within X second before [paging reception].

  - TC verified that UE does not send measurement report which is measured exceeds X second before [paging reception] if it is feasible for the testing.

In RAN4#112 meeting, companies are discussing whether more test is introduced for idle mode CA/DC measurement [1].

Issue 3-2-1: More test for Improvement on SCell/SCG setup delay < Way Forward > FFS the following option: - Proposal 1 (Nokia): - Introduce test cases with valid reporting for FR1 and FR2 also for the case when the UE is configured with EMR measurements.

One proposal is to add a test case for EMR-based measurements, where the UE reports valid measurements from the duration of X, when T331 is still running. We know in this case the correct UE behaviour is that UE shall report valid result which is obtained with X window. However the question is how to differentiate good UE who is performing validity check function and wrong UE who would never perform validity check. In other words, the suggested test could not verify UE have correctly executed validity check.

Observation 1: The new test for EMR based UE reporting valid measurement within X could not verify whether UE correctly executed validity check correctly.

Proposal 3: Suggest not to introduce new test for Idle mode fast CA/DC eEMR measurement.

LS on collision between SSB and RA occasion for LTM

RAN 1 identified an issue for LTM: a valid RA occasion for a LTM candidate cell (or serving cell) could overlap with a SSB from another LTM candidate cell (or serving cell). The reason is that determination of valid RA occasion is per cell (LTM candidate cell or serving cell) [2].

1. Overall Description: On R18 LTM, for unpaired spectrum, RAN1 identified possible collision between valid RA occasion (RO) and SSB associated with different cells in time domain, which is due to that RO validation is performed per cell. The collision could happen when the different cells are in the same frequency band or in different frequency band. It is unclear to RAN1 on whether/how to handle the collision between an SSB and a valid RO associated with different cells. 2. Actions: To RAN4: RAN1 respectfully asks RAN4 on whether/how to handle the collision between an SSB and a valid RO as described above and to provide answer for it.

Some companies in RAN1 point out that there is legacy requirement regarding the scheduling restriction for intra-frequency measurement without gap in RAN4. However it is observed that RAN4 specifies that UE is not expected to transmit PUCCH/PUSCH/SRS when these signals collide with SSB. There is no requirement for how to handle PRACH. The case is that intra-frequency TDD neighbor cell has different DL/UL configuration with serving cell.

- 9.2.5.3.1 Scheduling availability of UE performing measurements in TDD bands on FR1 When the UE performs intra-frequency measurements in a TDD band, the following restrictions apply due to SS-RSRP or SS-SINR measurement - The UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, and on 1 data symbol before each consecutive SSB symbols to be measured and 1 data symbol after each consecutive SSB symbols to be measured within SMTC window duration. If the high layer in TS 38.331 [2] signalling of smtc2 is configured, the SMTC periodicity follows smtc2; Otherwise SMTC periodicity follows smtc1.

Observation 2: In legacy RAN4 specifies that for intra-frequency measurement without gap, UE is not expected to transmit PUCCH/PUSCH/SRS when these signals collide with SSB. There is no requirement for how to handle PRACH in the case.

For the case that SSB and PRACH colliding in time domain happens at different frequency, it is related with “within gap” scenario. In TS 38.133, the below general description is captured. RAN4 directly refers to RAN2 TS38.321.

During the per-UE measurement gaps the UE: - is not required to conduct reception/transmission from/to the corresponding E-UTRAN PCell, E-UTRAN SCell(s) and NR serving cells for E-UTRA-NR dual connectivity except the reception of signals used for RRM measurement(s) and the signals used for random access procedure according to TS38.321 [7]. - is not required to conduct reception/transmission from/to the corresponding NR serving cells for SA (with single carrier or CA configured) except the reception of signals used for RRM measurement(s), PRS measurement(s) and the signals used for random access procedure according to [7]. - is not required to conduct reception/transmission from/to the corresponding PCell, SCell(s) and E-UTRAN serving cells for NR-E-UTRA dual connectivity except the reception of signals used for RRM measurement(s), PRS measurement(s) and the signals used for random access procedure according to [7]. - is not required to conduct reception/transmission from/to the corresponding NR serving cells for NR-DC except the reception of signals used for RRM measurement(s), PRS measurement(s) and the signals used for random access procedure according to [7].

After checking TS 38.321, it is specified that“the MAC entity may take into account the possible occurrence of measurement gaps and MUSIM gaps when determining the next available set of PRACH occasions corresponding to the selected SSB.”The modal verbs “may” means “permission to do”. MAC entity is not mandatory to consider gap occasion so UE may still transmit PRACH during gap occasion. In essence, it is left to UE implementation to transmit PRACH or perform SSB measurement within gap occasion.

Observation 3: In legacy RAN2 specifies that for inter-frequency measurement with gap, it is up to UE implementation to transmit PRACH or perform SSB measurement within gap occasion.

A similar issue has also been raised in R18 MIMO topic where a valid RA occasion for serving cell could overlap with a SSB from a neighboring cell. The CR R1-2409281 was finally agreed as below:

For a UE configured with SSB-MTC-AdditionalPCI, it does not transmit PRACH and receive SS/PBCH associated with different cells in a same slot.

For LTM, we don’t observe there is any issue if the collision handling for both intra-frequency and inter-frequency with gap case follows the same way of MIMO, i.e., left to UE implementation. From UE side, (1) As either the PDCCH ordered PRACH or measurement on neighbor cell is for candidate cell, it is uncertain whether the candidate cell which has performed early UL sync or early measurement will become the target cell in the future. So UE can have free room to decide to prioritize PRACH transmission or DL measurement. (2) If the RA for serving cell is colliding with SSB from neighbor cell, this resembles the legacy L3 measurement case. So the legacy handling scheme can be applied here (up to UE implementation). (3) If the RA occasion for candidate cell is colliding with SSB from other neighbor cells or serving cell, we don’t observe the outstanding advantage of prioritizing one or the other. From network perspective, we think network would monitor multiple PRACH occasions even for PDCCH ordered RACH, as UE may choose the PRACH occasion after one measurement gap.

Proposal 4: In R18 LTM, UE does not transmit PRACH and receive SS/PBCH associated with different cells in a same slot when the different cells are in the same frequency band or in different frequency band.

Conclusions

This contribution provides further analysis on maintenance requirements on L1/L2 based inter-cell mobility. The following proposals are provided:

Proposal 1: In FR1, for UE not supporting/configured with L1 measurement, one SSB occasion is needed from RAN4 requirement point of view for T/F fine tracking, if

- For intra-frequency case: the time gap between completion of TCI activation and cell switch command is larger than 480ms.

  - During the 480ms after the TCI state activation, longer L3 measurement delay may be expected.

- For inter-frequency case: the time gap between completion of TCI activation and cell switch command is larger than 160ms.

Proposal 2: Not pursue the discussion on the further optimization on TLTM_RRC-processing =0 when the total number of candidates triggered early TCI activation or early PDCCH order RACH exceed UE capability (option 3).

Observation 1: The new test for EMR based UE reporting valid measurement within X could not verify whether UE correctly executed validity check correctly.

Proposal 3: Suggest not to introduce new test for Idle mode fast CA/DC eEMR measurement.

Observation 2: In legacy RAN4 specifies that for intra-frequency measurement without gap, UE is not expected to transmit PUCCH/PUSCH/SRS when these signals collide with SSB. There is no requirement for how to handle PRACH in the case.

Observation 3: In legacy RAN2 specifies that for inter-frequency measurement with gap, it is up to UE implementation to transmit PRACH or perform SSB measurement within gap occasion.

Proposal 4: In R18 LTM, UE does not transmit PRACH and receive SS/PBCH associated with different cells in a same slot when the different cells are in the same frequency band or in different frequency band.

Reference

- R4-2420234, WF on Further NR mobility enhancements, MediaTek

- R1-2410816, Moderator summary 2 on handling collision of SSB and RA occasion for LTM, Google

- R1-2410901, LS on collision between SSB and RA occasion for LTM, Google

Annex- Draft LS reply

Title:	Reply LS on collision between SSB and RA occasion for LTM

Response to:	R1-2410901

Release:	Rel-18

Work Item:	NR_Mob_enh2-Core

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Han Jing

hw.hanjing@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

- Overall description

RAN4 would like to thank RAN1 for the LS in (R1-2410901). RAN4 discussed the questions in the LS, and concluded that in R18 LTM, UE does not transmit PRACH and receive SS/PBCH associated with different cells in a same slot when the different cells are in the same frequency band or in different frequency band.

Actions

To RAN2

ACTION: 	RAN4 respectfully asks RAN1 to take above information into account.

Dates of next TSG RAN WG 4 meetings

TSG RAN WG2 Meeting #114bis	7 – 11 April 2025	China

TSG RAN WG4 Meeting #115	     19 – 23 May 2025	Malta, MT