3GPP TSG-RAN WG4 Meeting #114	R4-2501271

Athens, Greece, 17 – 21 February, 2025

Title: 	Initial discussion on RRM requirements for NTN in Ku band

Source: 	Huawei, HiSilicon

Agenda item:	7.11.6

Document for:	Discussion

Introduction

The WID for R19 WI “Introduction of Ku bands for NR NTN” was updated in RAN#106, with following RRM related objective being added.

- Specify RRM requirements including timing requirements for mobile VSAT served by NGSO - NOTE: RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only.

In this paper, we will provide our initial views on RRM requirements for NTN in Ku band.

Discussion

In R18, the RRM requirements for FR2-NTN are defined for the following cases.

- Case-1: Stationary UE for GSO;

- Case-2: Stationary UE for LEO;

- Case-3: Mobile UE for GSO.

The stationary and mobile UE are defined in RF spec and based on the UE FG 40-1.

Support of fixed or mobile VSAT (Very Small Aperture Terminal) UE type - Type 1: a fixed VSAT, which can only be fixed. - Type 2: a mobile VSAT, which is capable to move. A VSAT (Very Small Aperture Terminal) UE as defined in TS 38.101-5 shall indicate support of this capability with only one type.

One clear missing part for mobile VSAT served by NGSO in RRM requirements is timing, which is defined separately for different cases (requirements for case 1 and case 2 are same), as in Table 7.1C.2-2 and Table 7.1C.2-3 of 38.133 as copied below.

Table 7.1C.2-2: Te_NTN Timing Error Limit for fixed VSAT is served by GSO and fixed VSAT is served by NGSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

Table 7.1C.2-3: Te_NTN Timing Error Limit for mobile VSAT is served by GSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] Note 2: The value of Te_NTN for 120 kHz UL SCS assumes LOS, and GNSS signal power level above [TBD] dBm [and max acceleration of [TBD] m/s2] FFS other side conditions if needed. |  |  |  |

To define the timing requirements for mobile VSAT served by NGSO, we need to determine the assumed GNSS error and satellite location error.

- Based on the latest agreement from last meeting, GNSS error is assumed to be same for fixed and mobile UEs, but there would be some side conditions for the mobile UE to achieve this accuracy. GNSS error should be same for GSO and NGSO.

- Satellite location error may be different for GSO and NGSO. In our understanding, satellite location error should be no worse than that for GSO, provided that the ephemeris information from the satellite control system is accurate and timely updated (which has also been the assumption so far).

Based on above analysis, we suggest to re-use the timing requirements of mobile VSAT is served by GSO for mobile VSAT served by NGSO.

The above discussions are assuming FR2 numerology will be used in Ku bands, as to Ka bands. However, as agreed in [1], there will be another set of bands based on FR1 numerology.

- Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing.

The timing requirements for FR1-NTN is in Table 7.1C.2-1 of 38.133 as copied below.

Table 7.1C.2-1: Te_NTN Timing Error Limit

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR1-NTN | 15 | 15 | 29*64*Tc |
|  |  | 30 | 24*64*Tc |
|  |  | 60 | N/A |
|  | 30 | 15 | 24*64*Tc |
|  |  | 30 | 22*64*Tc |
|  |  | 60 | N/A |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] |  |  |  |

These requirements are defined without any restriction on UE movement or satellite type. As VSAT is more capable than typical FR1-NTN device e.g. smartphone, VSAT should be able to meet these requirements in Ku bands. On the other hand, for 30kHz SCS CP is 72 Ts, and the existing accuracy of 24 or 22 Ts is well below CP/2, so we also do not see a strong need to tighten the requirements for Ku bands.

Based on all above discussion, we have the following proposals.

Proposal 1: RAN4 to define timing accuracy requirements for mobile VSAT served by NGSO.

- For FR1 numerology, existing requirements in Table 7.1C.2-1 of 38.133 (FR1-NTN) are re-used.

- For FR2 numerology, existing requirements in Table 7.1C.2-3 of 38.133 (FR2-NTN mobile VSAT served by GSO) are re-used.

As the requirements for mobile VSAT served by NGSO are introduced as part of Ku bands WI, they should apply to Ku bands only. Besides, there is note in the WID that “RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only”. We understand it means a UE is only required to meet one of the requirements for FR1 and FR2 numerology. Which requirement UE should meet depends on which of the FR1-NTN operation and FR2-NTN operation UE is supporting.

Proposal 2: The timing accuracy requirements for mobile VSAT served by NGSO are applicable for operation on Ku bands only. A UE is required to meet requirements for FR1-NTN operation only, or for FR2-NTN operation only.

Conclusions

In this paper we provided provide our initial views on RRM requirements for NTN in Ku band.

Proposal 1: RAN4 to define timing accuracy requirements for mobile VSAT served by NGSO.

- For FR1 numerology, existing requirements in Table 7.1C.2-1 of 38.133 (FR1-NTN) are re-used.

- For FR2 numerology, existing requirements in Table 7.1C.2-3 of 38.133 (FR2-NTN mobile VSAT served by GSO) are re-used.

Proposal 2: The timing accuracy requirements for mobile VSAT served by NGSO are applicable for operation on Ku bands only. A UE is required to meet requirements for FR1-NTN operation only, or for FR2-NTN operation only.

Reference

- RP-243314, Revised WID: Introduction of Ku bands for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, Airbus, Sharp, Mitre, JSAT