# 3GPP TSG-RAN WG4 Meeting #114 　　　　　　　　　　　　　　　　　	R4-2501310

Athens, GR, 17th  – 21st  February, 2025

Source: 	NTT DOCOMO INC.

Title:	Discussion on BS indication for MPR reduction

Agenda item:	7.1.1.2.1

Document for:	Discussion

- Introduction

In Rel-19, in the WI “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”[1], MPR reduction is included as one of the objectives. RAN4 has discussed approaches to enable MPR reduction, but the detail of the BS indication have not yet been agreed in the WF[2]. This paper provides our views on the signaling aspects. Please note that this is similar to the paper at the last meeting, as it was not discussed in detail in terms of BS signaling at the last meeting.

- Discussion

Background:

According to the WID for this Rel-19 WI, ACLR/SEM/SE can be relaxed under particular conditions with BS indication as follows[1]:

Objective in [1] Power boosting and/or MPR reduction - Specify power domain enhancement, e.g., MPR reduction for NR single carrier and NR intra-band UL CA - Study the scenarios, and if feasible, specify the power domain enhancement, e.g., MPR reduction, for PC2 and PC3 with applicable ACLR/SEM/spurious emission modification with BS indication for NR FR1 on a single UL carrier - Include the following scenarios: - when there is no adjacent in-band/out-of-band co-existence issue - when a UE uses a narrower channel bandwidth within a wider BS bandwidth - Include both (e)RedCap UE (only PC3) and non-RedCap UE - Limited to QSPK and 16QAM

However, the details of the BS indication have not yet been agreed upon[2].

Discussion:

While the approved WF mentioned that the discussion on signaling aspects should be after sufficient evaluation of the work, there seemed some confusion among companies in last meeting regarding how to control this feature, especially for region where relaxation on emission requirement is not allowed, and thus let us provide our understanding for clarification.

According to the WID[1], the applicability of the ACLR/SEM/spurious emission modification is determined by BS indication. It means that this new feature can be under NW control. In other words, UE can enable this feature after UE receives related BS indication from NW. Therefore, even if the regulation in some regions does not allow relevant emission requirement relaxation, there is no problem as long as the NW does not give the indication. This aspect should be clarified to avoid any misunderstanding.

Observation 1: BS indication is needed before UE enable the MPR reduction by relaxing emission requirements according to the description in objective in the WID.

In addition to the principle mentioned above, if the BS indication signals a uniform modification for ACLR/SEM/spurious emission, in our view, the conditions under which this feature can be enabled are limited.

In light of regulations, interference requirements are influenced by frequency usage and coexistence with surrounding systems, therefore, legislation varies from region to region. For example, there may be regions where SEM and spurious emissions cannot be mitigated, but only ACLR modifications can. If the BS indication was designed to inform a uniform modification of ACLR/SEM/spurious emissions, this function cannot be activated in such regions. In other words, the uniform correction based on the BS indication is only valid in areas where all corrections are permitted. It is also possible that this function cannot be enabled if future changes in legislation make it impossible to relax some of the requirements.

Furthermore, valid modifications to interference requirements depend on specific scenarios. For example, an ACLR modification may be permitted in one BS but not in another. In this case, the function would only be enabled in BSs where all modifications are allowed as well.

From the above, it can be said that, considering that the variability of interference mitigation requirements varies from region to region and scenario to scenario, the BS instructions should be designed to allow BSs to choose which of the ACLR/SEM/spurious emissions can be mitigated.

Observation 2: BS indication that determines a uniform modification of ACLR/SEM/spurious emission may limit the situations in which this feature can be enabled due to variations in regulations and scenarios.

Proposal: BS indication should be designed to allow selection of which ACLR/SEM/spurious emission can be relaxed to fully enable this feature.

- Conclusion

Here we summarize our proposals:

Observation 1: BS indication is needed before UE enable the MPR reduction by relaxing emission requirements according to the description in objective in the WID.

Observation 2: BS indication that determines a uniform modification of ACLR/SEM/spurious emission may limit the situations in which this feature can be enabled due to variations in regulations and scenarios.

Proposal: BS indication should be designed to allow selection of which ACLR/SEM/spurious emission can be relaxed to fully enable this feature.

4	References

[1] RP-241656, “Revised WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, RAN4 chair (Huawei)

[2] R4-2420527, “WF on Power domain enhancements”, Huawei, HiSilicon