3GPP TSG-RAN WG4 Meeting #114	R4-2501312

Athens, Greece, Feb 17th – Feb 21st, 2025

Source:	Nokia, Rohde&Schwarz, Keysight

Title:	(NR_newRAT-Perf) 38.101-4 Information on FRC changes to maximum throughput.

Agenda item:	4.6

Document for:	Information

Introduction

In RAN4#113 Nokia contributed 1 discussion paper [1] and 4 CRs [2][3][4][5] for streamlining the way the maximum throughput in the FRCs in 38.101-4 [6] are rounded, as 3 different approaches were used throughout the FRCs.

The feedback from the discussions in RAN4#113 were that some companies did not mind to implement the changes while others would like to avoid changing the existing specification as they saw no need to do so.
Based on the feedback, the CRs were postponed and Nokia took the responsibility to discuss with the users of the information in the FRCs (i.e. TE vendors) before RAN4#114, what should be the next step (see[7]):

R4-2417968 (NR_newRAT-Perf) CR for 38.101-4 on FRC corrections Type: CR For: Agreement 38.101-4 v15.23.0 CR-0651 rev Cat: F (Rel-15) Source: Nokia Decision: Postponed Huawei: We don’t think the changes are needed. Nokia: We will discuss with TE vendors for next meeting.

This contribution contains the outcome of the discussion.

Further considerations after offline discussions

Since RAN4#113 more considerations have been heard in offline exchanges, in particular from the TE vendor side.	
To summarize, the following points have been made:

- Even though in the latest version of 38.101-4 both flooring/rounding to 3 decimals, as well as, no rounding (which requires 4 decimals) is used. It is acceptable for the older and current version of 38.101-4 to keep the existing Max throughput values.

- Still, it is strongly recommended for potentially new FRCs added in RAN4#114 and future meetings to agree on how to derive the Max. throughput values. Following two solutions could be acceptable, assuming to keep 3 digits as target:

  - Use floor to 3 digits always.

  - Change from Mbps to Kbps which would secure no rounding is needed as the last 2 digits of the 3 would be 0 always.

Changing from Mbps to Kbps is preferred however changing from Mbps to Kbps, cannot be done in existing tables in 38.101-4 and it would be misaligned between the tables if a new table is using Kbps instead of Mbps, hence it is seen as the best approach for 38.101-4 to keep the values in Mbps and always used floor function for new FRCs.
For future specifications there is no need to keep Mbps so Kbps can be used instead, which will secure no rounding required.

Stating the discussion and potential future treatment of throughput calculations for FRCs in observation form:

There is no strong need to update the existing FRCs in 38.101-4, however it is strongly recommended to align on using floor function to Mbps with 3 decimals for any future FRCs added to 38.101-4. Based on that, the postponed CRs from RAN4#113 will not be further pursued.

For future specifications it is recommended to use Kbps instead of Mbps to avoid the need to use the floor function. Such approach will secure the Max. throughput values provided in the FRCs of future specifications are exact.

It is not possible to make agreements in maintenances as there will be no WF to capture them, hence agreements on how to proceed will have to be made at the point in time where a new FRC is added to 38.101-4 and as well when a new FRC is added to a future specification.

Conclusion

In this contribution we have provided the outcome of the offline discussion after RAN4#113 related to the postponed CRs contributed in RAN4#113 about possible FRC changes to maximum throughput.

Following are the observations made:

- There is no strong need to update the existing FRCs in 38.101-4, however it is strongly recommended to align on using floor function to Mbps with 3 decimals for any future FRCs added to 38.101-4. Based on that, the postponed CRs from RAN4#113 will not be further pursued.

For future specifications it is recommended to use Kbps instead of Mbps to avoid the need to use the floor function. Such approach will secure the Max. throughput values provided in the FRCs of future specifications are exact.

It is not possible to make agreements in maintenances as there will be no WF to capture them, hence agreements on how to proceed will have to be made at the point in time where a new FRC is added to 38.101-4 and as well when a new FRC is added to a future specification.

- References

- R4-2417972 - (NR_newRAT-Perf) Discussion on FRC corrections to maximum throughput

- R4-2417968 - (NR_newRAT-Perf) CR for 38101-4 on FRC corrections

- R4-2417969 - (NR_newRAT-Perf) CR for 38101-4 on FRC corrections

- R4-2417970 - (NR_newRAT-Perf) CR for 38101-4 on FRC corrections

- R4-2417971 - (NR_newRAT-Perf) CR for 38101-4 on FRC corrections

- TS38.101-4 - User Equipment (UE) radio transmission and reception; Part 4: Performance requirements (Release 18)

- RAN4_113_BDaT_Session_report_v7_PostMeeting