3GPP TSG-RAN WG4 Meeting #114	R4-2501350

Athens, Greece, 17th – 21st February 2025

Source:	Ericsson

Title:	Discussion on Rel-19 MMSE-IRC BS demodulation requirements

Agenda item:	7.18.3

Document for:	Discussion

# Introduction

In previous RAN4#113 meeting, companies discussed the interference modelling, test parameters and simulation assumptions, and manufacturer declarations and applicability rules for Rel-19 NR MMSE-IRC BS requirement. The open issues on general issues are captured in our other contribution [1].

In this contribution, the open issues on interference modeling and test parameters will be furtherly discussed as per the WF [2].

# Discussion

## 2.1	Interference Modelling

Open Issue:

Number of interferers for 2Rx

  - Option 1: Define BS MMSE-IRC requirements for 2Rx

- Option 1A: 1 interferer

- Option 1B: 2 interferers

- Option 1C: Consider 2 interferers for HomNet and 1 interfere for HetNet

  - Option 2: Do not need to specify requirements for 2Rx

  - Companies are encouraged to provide simulation results for 1 or 2 interferers for 2Rx, to check the performance gain.

Agreements:

Interference profile

- On how to account for HPUE in existing LTE profiles:

  - For interference profile not to consider HPUE scenario for BS MMSE-IRC requirements for this Rel-19 WI

## Fig. 1: Performance gain of MMSE-IRC over MMSE for 15 KHz SCS and 5 MHz CBW

## As it was mentioned in the previous meeting, the additional impairment margin around 3 dB can offset the performance gain of MMSE-IRC over MMSE in ideal simulation results. From Fig. 1, we observe with 3dB additional impairment to SNR@70% of maximum throughput, the performance gain of ~2 dB in homogenous networks and 3~4 dB in heterogenous networks remains valid. Further, following the PUSCH demodulation requirements in LTE with synchronous interference [3], we should test 2, 4 and 8Rx to have appropriate test coverage. However, after aligning and averaging the results from the companies, the gain should be sufficient for 2Rx configuration to create the demodulation requirements.

From our simulations [4], no significant variation in gain of MMSE-IRC over the baseline MMSE receiver is observed in both homogenous and heterogenous scenarios when the number of interferers increases from 1 to 2.

Proposal 1: Consider 1 interferer to define BS MMSE-IRC requirements for 2Rx, provided that the gain of MMSE-IRC over MMSE is deemed sufficient after alignment and averaging among the companies.

## 2.2	Test parameters and simulation assumptions

Open Issue:

Waveform – whether to cover DFT-s-OFDM

- Candidate options:

  - Option 1: Cover both CP-OFDM and DFT-s-OFDM waveform

  - Option 2: Cover CP-OFDM only

In the previous discussions, companies have noted that DFT-s-OFDM is rarely used in real deployments. Additionally, our previous simulations [5] did not show a significant performance difference between CP-OFDM and DFT-s-OFDM. Therefore, defining the requirement based solely on CP-OFDM should be sufficient.

Proposal 2: Only consider CP-OFDM for NR MMSE-IRC PUSCH requirements.

Open Issue:

Channel bandwidth

- Proposals on the CHBW for requirement definition for CP-OFDM:

  - Option 1:

- For 15kHz SCS, cover 5MHz, 10MHz, 20MHz

- For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz

  - Option 2:

- For 15kHz SCS, cover 5MHz, 10MHz

- For 30kHz SCS, cover 10MHz, 40MHz

  - Option 3: minimum CBW

- For 15kHz SCS, 5MHz

- For 30kHz SCS, 10MHz

  - Option 4: 5/50MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz SCS

- Proposals for DFT-s-OFDM:

  - Option 1:

- For 15kHz SCS, 5MHz

- For 30kHz SCS, 10MHz

For channel bandwidth per SCS, the minimum channel bandwidth should be considered as high priority regarding existing applicability rule in TS 38.141-1 and in many Rel-18 requirements. As highlighted in our previous contribution [6] and pointed by some companies, there is no significant performance difference of MMSE-IRC over MMSE for a larger channel bandwidth with CP-OFDM. According to our previous simulation results [7], the performance difference between minimum and maximum channel bandwidth is <1dB. Thus, to reduce the simulation and test effort, we propose only to consider option 3.

Proposal 3: Only consider minimum CBW for CP-OFDM, i.e. 5 MHz for 15KHz SCS and 10 MHz for 30 KHz SCS to define BS MMSE-IRC requirements.

Open Issue:

Propagation condition

- For serving cell, consider the following options for performance evaluation and as candidates for further down-selection:

o    Option 1: TDLA30-10 HetNet and TDLB100-400 for HomNet

o    Option 2: Consider different channel for different MCS

ü  Option 2A: MCS 2 with TDLB100-400, MCS 16 with TDLC300-100, MCS 20 with TDLA30-10

Agreements:

- For the interference cell, use TDLC300-100.

In LTE study of MMSE-IRC receiver [8], same propagation model was used for serving and interfering cell (Table 7.1-2) in link-level simulations while different models were used to define the BS requirements (Sec. 8.2.6 in [9]). Following the same assumption in this WI, we agreed to introduce two sets of requirements for homogenous and heterogenous scenarios separately, so different channel model for two deployments might be more suitable.

Observation 1: Following the BS requirements in LTE, it would be more suitable to use different channel model for different BS deployment scenarios.

To compare and down-select from options 1 and 2, we performed detailed simulations in [4][5]. Here we provide a brief overview for 15 KHz SCS 5 MHz CBW case in Table 1 and 2. Similar observations were captured for 30 KHz SCS 10 MHz CBW case and with two interferers.

Table 1: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 5 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Disabled with 1 Interferer

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | MCS In Table 1 | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 |  |  | MMSE-IRC | MMSE | Gain |
| 1 | 2 | TDLA30-10 | TDLC300-100 | 9.83 | 2 | -1.17 | 2.57 | 3.74 |
|  |  | TDLB100-400 |  | 5.36 | 2 | -1.52 | 0.28 | 1.8 |
|  | 4 | TDLA30-10 | TDLC300-100 | 9.83 | 16 | 5.73 | 11.13 | 5.4 |
|  |  | TDLB100-400 |  | 5.36 | 16 | 7.61 | 10.22 | 2.61 |
|  | 8 | TDLA30-10 | TDLC300-100 | 9.83 | 20 | 5.55 | 10.60 | 5.05 |
|  |  | TDLB100-400 |  | 5.36 | 20 | 7.32 | 9.65 | 2.33 |

Table 2: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 5 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Disabled with 1 Interferer

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | MCS In Table 1 | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 |  |  | MMSE-IRC | MMSE | Gain |
| 1 | 2 | TDLB100-400 | TDLC300-100 | 9.83 | 2 | 0.007 | 3.47 | 3.47 |
|  |  |  |  | 5.36 | 2 | -1.52 | 0.28 | 1.8 |
|  | 4 | TDLC300-100 | TDLC300-100 | 9.83 | 16 | 7.65 | 12.53 | 4.88 |
|  |  |  |  | 5.36 | 16 | 6.70 | 9.39 | 2.69 |
|  | 8 | TDLA30-10 | TDLC300-100 | 9.83 | 20 | 5.56 | 10.60 | 5.04 |
|  |  |  |  | 5.36 | 20 | 5.14 | 7.70 | 2.56 |

We use the simulation parameters in Table 1, following the LTE BS IRC requirements outlined in TS 36.104 as suggested in option 1, while Table 2 adheres to the legacy NR BS requirements specified in TS 38.104, as suggested in option 2.

Observation 2: The gain of MMSE-IRC over MMSE is almost similar while considering:

- Different channel models for different MCS (MCS 2 with TDLB100-400, MCS 16 with TDLC300-100, MCS 20 with TDLA30-10) in both deployments.

- TDLA30-10 for heterogenous and TDLB100-400 for homogenous scenarios.

Further, heterogenous scenario should prioritize low Doppler configurations due to the smaller cell radius and reduced relative mobility of users, while homogenous scenario should consider larger Doppler configurations to account for their wider cell coverage and potentially higher relative user mobility. Therefore, the BS capability may need to be tailored to account for deployment-specific channel conditions.

Considering observation 1 and 2, we think it would be better to consider different channel models for different BS deployment, i.e. option 1.

Proposal 4: Use TDLA30-10 for heterogenous and TDLB100-400 for homogenous deployment scenarios for serving cell.

Open Issue:

MCS

- Consider the following options for performance evaluation and as candidates for further down-selection:

o    For 2Rx:

ü  Option 1: MCS 2

ü  Option 2: MCS 16

o    For 4Rx:

ü  Option 1: MCS 16

o    For 8Rx:

ü  Option 1: MCS 20

ü  Option 2: MCS 16

- Check whether the existing test applicability rule can be reused if different MCS is configured for different Rx configuration.

Agreements:

- Reuse the same FRC A.3, A.4 and A.5 with Additional DM-RS position = pos1 and 1 transmission layer defined in TS38.104 for MCS 2, MCS 16 and MCS 20 respectively.

While using the same MCS across different number of Rx antennas could indeed simplify the test setup, it may not be the most realistic representation of different deployment scenarios. As the number of Rx antennas increases, the link quality improves due to spatial diversity. In such cases, higher MCS values can be used to achieve higher throughput without compromising reliability. Further, we think it would be better to test QPSK, 16QAM and 64QAM modulations, like the legacy BS requirements in TS 38.104.

Observation 3: Following the BS-IRC requirements for LTE (Sec. 8.2.6 in [9]), different MCS have been configured for different Rx antennas for both homogenous and heterogenous deployments.

Since adjusting the MCS based on the number of Rx antennas provides a clearer understanding of how the system scales with diversity and performs in terms of throughput and reliability as channel conditions improve, using a single MCS for different numbers of Rx antennas may not be feasible.

Proposal 5: Avoid relying solely on a single MCS to define BS MMSE-IRC requirements for multiple Rx configurations.

The IRC performance results are expected to be worse than the Rel-15 normal PUSCH due to the presence of interferers. However, since the results in TS 38.104 are averaged based on impairment values provided by different companies, alignment among companies is necessary before proceeding.

Observation 4: Our simulations indicate that the target SNR for IRC (as shown in Table 2) for each MCS is achievable and aligns with the Rel-15 Normal PUSCH requirements specified in TS 38.104.

Table 3: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 5 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Disabled with 1 Interferer

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | MCS In Table 1 | SNR [dB] |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 |  |  | MMSE-IRC | MMSE | Gain |
| 1 | 2 | TDLA30-10 | TDLC300-100 | 9.83 | 16 | 11.51 | 15.26 | 3.75 |
|  |  | TDLB100-400 |  | 5.36 | 16 | 14.36 | 15.56 | 1.2 |

For 8Rx, both MCS 16 and MCS 20 can be feasible. But MCS 20 would be a better choice to define the requirement since higher modulation (64QAM) can be tested which would require higher target SNR for 70% normalized throughput requirement. As agreed for 4Rx, with MCS 16, 16QAM will be tested. Similarly, for 2Rx both MCS 2 and 16 can be feasible, but it would be better to also test QPSK modulation. As shown in Table 3, the MMSE-IRC gain over MMSE remains the same for a heterogeneous network with MCS 2 and MCS 16. However, a slightly lower gain is observed for a homogeneous deployment with MCS 16. Given this, MCS 2 appears to be the better choice for the 2Rx configuration.

Proposal 6: It may be feasible to consider MCS index 2, 16, 20 in MCS table 1 for 2, 4 and 8Rx requirements.

Further, MCS Table 1 is mandatory and must be supported by the product. Additionally, the Rel-15 requirement is mandatory and has been tested with existing MCS values and channel models across different Rx antenna configurations. Since this is not directly related to product capability, the existing test applicability rules can be reused if different MCS values are configured for different Rx configurations.

Open Issue:

DMRS configuration

- FFS whether to include DMRS configuration type 2.

- FFS DMRS sequence generation configuration for DFT-s-OFDM

Agreements:

- Cover DMRS configuration type 1.

- Use DMRS port #0 for both serving and interfering PUSCH.

- On DMRS sequence generation, use different DMRS sequence for each gNB:

- For CP-OFDM: nSCID =0 for all cells and NID0=0, 1 and 2 for cell #0 (serving), cell #1 and cell#2 (interfering) respectively

Observation 5: It is not required to introduce new applicability rule for different MCS being configured for different Rx configurations.

DMRS Configuration Type defines different patterns for placing DMRS within a Physical Resource Block. Type 2 is typically used in high-mobility scenarios, MU-MIMO configurations, etc. where DMRS symbols are mapped to a denser set of subcarriers and support more orthogonal DMRS ports, compared to type 1. This enables improved channel estimation, especially in scenarios with higher frequency selectivity. For normal PUSCH requirement in TS 38.104, RAN4 only specified the requirement with type 1. Therefore, for BS MMSE-IRC, it is reasonable to follow the legacy normal PUSCH requirements.

Proposal 7: Only consider DMRS configuration type 1 for BS MMSE-IRC PUSCH requirements.

Open Issue:

Test requirement definition

- Candidate options:

  - Option 1: FFS whether SINR based definition is needed

  - Option 2: Use the SNR based requirement definition

As given in Sec 8.1 of TS 36.104 for LTE, both SNR and SINR based test requirements are defined because DIP-based interference modeling is used in LTE. For NR, we can follow INR-based interference modeling approach, which also aligns with the UE MMSE-IRC demodulation requirement. Therefore, we consider SNR based requirement as currently existing in Sec 8.1 of TS 38.104.

Proposal 8: Use SNR based requirement as in TS 38.104 (Sec. 8.1).

# Conclusion

Based on the discussion, we made the following observations:

Observation 1: Following the BS requirements in LTE, it would be more suitable to use different channel model for different BS deployment scenarios.

Observation 2: The gain of MMSE-IRC over MMSE is almost similar while considering:

- Different channel models for different MCS (MCS 2 with TDLB100-400, MCS 16 with TDLC300-100, MCS 20 with TDLA30-10) in both deployments.

- TDLA30-10 for heterogenous and TDLB100-400 for homogenous scenarios.

Observation 3: Following the BS-IRC requirements for LTE (Sec. 8.2.6 in [9]), different MCS have been configured for different Rx antennas for both homogenous and heterogenous deployments.

Observation 4: Our simulations indicate that the target SNR for IRC (as shown in Table 2) for each MCS is achievable and aligns with the Rel-15 Normal PUSCH requirements specified in TS 38.104.

Observation 5: It is not required to introduce new applicability rule for different MCS being configured for different Rx configurations.

Based on the discussion, we made the following proposals:

Proposal 1: Consider 1 interferer to define BS MMSE-IRC requirements for 2Rx, provided that the gain of MMSE-IRC over MMSE is deemed sufficient after alignment and averaging among the companies.

Proposal 2: Only consider CP-OFDM for NR MMSE-IRC PUSCH requirements.

Proposal 3: Only consider minimum CBW for CP-OFDM, i.e. 5 MHz for 15KHz SCS and 10 MHz for 30 KHz SCS to define BS MMSE-IRC requirements.

Proposal 4: Use TDLA30-10 for heterogenous and TDLB100-400 for homogenous deployment scenarios for serving cell.

Proposal 5: Avoid relying solely on a single MCS to define BS MMSE-IRC requirements.

Proposal 6: It may be feasible to consider MCS index 2, 16, 20 in MCS table 1 for 2, 4 and 8Rx requirements.

Proposal 7: Only consider DMRS configuration type 1 for BS MMSE-IRC PUSCH requirements.

Proposal 8: Use SNR based requirement as in TS 38.104 (Sec. 8.1).

# References

[1] R4-2501352, “On general issues for MMSE-IRC receiver under interference”, Ericsson, RAN4#114, Athens, Greece.

[2] R4-2419780, “Way Forward for [113] [321] NR_demod_Ph5_Part1_General_BS”, China Telecom, MediaTek, RAN4#113, Orlando, USA.

[3] 3GPP TS 36.104, Base Station (BS) radio transmission and reception (Release 18)

[4] R4-2501351, “Simulation results for Rel-19 MMSE-IRC BS demodulation requirements”, Ericsson, RAN4#114, Athens. Greece.

[5] R4-2419250, “Simulation results for Rel-19 MMSE-IRC BS demodulation requirements”, Ericsson, RAN4#113, Orlando, USA.

[6] R4-2419249, “Discussion on Rel-19 MMSE-IRC BS demodulation requirements”, Ericsson, RAN4#113, Orlando, USA.

[7] R4-2416092, “Simulation results for Rel-19 MMSE-IRC BS demodulation requirements”, Ericsson, RAN4#112bis, Hefei, Anhui, China.

[8] 3GPP TR 36.884, Performance requirements of MMSE-IRC receiver for LTE BS (Release 13)

[9] 3GPP TS 36.104, Base Station (BS) radio transmission and reception (Release 18)