3GPP TSG-RAN WG4 Meeting #114	R4-2501382

Athens, Greece, Feb 17th – Feb 21st, 2025

Source:	Nokia

Title:	RedCap NTN UEs

Agenda item:	7.29.2.1

Document for:	Discussion

Introduction

In this paper, we provide our views on the support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands. RedCap specific objectives mentioned in the WID [1] are copied below for ease of reading.

Objectives:

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, check whether any essential changes are needed for their support (i.e. focusing on HD collision rules) by end of Q2/2024 [RAN1] - Depending on feasibility assessment above, define the RF and RRM requirements [RAN4] - Notes for this objective: GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

In the last RAN4 meeting, the following agreements were made [1].

## Issue 1-1: PC3 output power for (e)RedCap Agreement: - FFS whether and how a potential new scheme to increase output power is addressed in specifications - Alt 1:Further confirm whether rel-18 power boost is already applicable for NR NTN including (e)RedCap - If confirmed, no further work is needed. - Alt 2: Power increase by network controlled optional new signalling (e.g capability) - Alt 3: Change in power tolerances - E.g. Upper tolerance exception to UE in specific regions via table note. - FFS whether solution is common for HD-FDD and FD-FDD - If no solution agreed, PC3 tolerance will remain unchanged to +/- 2 dB. ## Issue 1-2: HD-FDD refsens for 2 Rx Agreement: Further discussion on potential improvement beyond TN HD-FDD 2 RX REFSENS level is limited to maximum 0.2 dB improvement based on average of inputs to RAN4. ## Issue 1-3: Simultaneous operation with GNSS Agreement: Simultaneous GNSS and NR-NTN operation for (e)Redcap cannot be guaranteed in some occasions for some NTN bands for some specific UE implementations. Send LS to RAN2 requesting common IDC solution for (e)RedCap NR NTN and non-RedCap NR NTN which further improves the existing IDC framework to be more suitable for NR NTN – GNSS co-existence. Final solution is up to RAN2. ## Issue 2-1: UL capacity enhancements Agreement: No RF impact on inter-slot OCC over 2 slots.

Discussion

PC3 output power for (e)RedCap

There has been an interest in increasing the output power for a HD-FDD PC3 UE because of lower front-end intersection loss. However, no agreement has reached how to do that. Generally, with increased output power comes increased emissions and thus UE get an MPR margin in order to meet emission requirements. In this particular case, although we are sympathetic to  the goal of increasing the output power for the default power class, we do not want to increase in the MPR relaxations because of this extra output power.

MPR should not be changed because of increase in the output power.

HD-FDD REFSENS for two RX

Companies have proposed to use 0.5dB for n254 and 0dB for n255 based on the tightening rules that were used for TN RedCap HD-FDD REFSENS [3]. The reasoning for 0.5 dB goes back to almost 10 years old limited survey on insertion loss difference between a SAW RF filter vs SAW duplex filter. As SAW RF filters have less insertion loss compared to SAW duplex filters and since HD-FDD implementations do not need duplex filters, REFSENS can be tightened by 0.5 dB. Further, the reasoning for 0dB was based on REFSENS of TDD bands.

Reasoning for 0.5 dB improvement goes back to almost 10 years old survey on insertion loss differences.

Reasoning for 0 dB tightening was related to REFSESN of TDD bands.

We did a limited survey of duplexers, RF filters, and TRX switches for n255 and n254, and noticed that the insertion loss difference ranges from 1.2 dB to 2 dB. This observation supports the 0.8 dB number that was proposed under more recent discussions done under Release 17 RedCap for HD-FDD case. Given that lot of improvement in device technology has happened over last 10 years, it is possible to leverage that improvement for improving the REFSENS.

Recent survey on RF components showed that insertion loss difference ranges from 1.2dB to 2dB.

In addition, there are no TDD NTN bands, so utilizing TDD bands as an excuse for inaction hinders overall system efficiency and improvements.

There are no TDD bands in NTN, so sticking to 0dB tightening is unjustified.

It was argued that coverage is limited by UEs uplink power and improving REFSENS will not help. However, the views expressed during the meeting by satellite service providers were contrary to this claim. According to them, even downlink is limited because of limited transmit power at the satellite and thus any improvement in the device performance is helpful.

Signal margin in downlink is also limited due to limited available satellite power.

In the last meeting, it was agreed to keep the potential improvement beyond TN HD-FDD 2 RX REFSENS level to a maximum of 0.2 dB based on average of inputs to RAN4. Based on the averaged numbers discussed in previous meetings, we propose the following REFSENS values for n255 and n254.

Table 1 REFSENS values for n254 and n255.

| Operating Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
|  | 15 | -100.2 | -97 | -95.2 | -94.0 |
| n255 | 30 |  | -97.3 | -95.3 | -94.2 |
|  | 60 |  | -97.7 | -95.6 | -94.4 |
|  | 15 | -100.1 | -96.9 | -95.1 |  |
| n254 | 30 |  | -97.2 | -95.2 |  |
|  | 60 |  | -97.6 | -95.5 |  |

Agree to the following REFSESN values for n254 and n255

| Operating Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
|  | 15 | -100.2 | -97 | -95.2 | -94.0 |
| n255 | 30 |  | -97.3 | -95.3 | -94.2 |
|  | 60 |  | -97.7 | -95.6 | -94.4 |
|  | 15 | -100.1 | -96.9 | -95.1 |  |
| n254 | 30 |  | -97.2 | -95.2 |  |
|  | 60 |  | -97.6 | -95.5 |  |

Conclusion

Proposal 1: MPR should not be changed because of increase in the output power.

Observation 1: Reasoning for 0.5 dB improvement goes back to almost 10 years old survey on insertion loss differences.

Observation 2: Reasoning for 0 dB tightening was related to REFSESN of TDD bands.

Observation 3: Recent survey on RF components showed that insertion loss difference ranges from 1.2dB to 2dB.

Observation 4: There are no TDD bands in NTN, so sticking to 0dB tightening is unjustified.

Observation 5: Signal margin in downlink is also limited due to limited available satellite power.

Proposal 2: Agree to the following REFSESN values for n254 and n255

| Operating Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
|  | 15 | -100.2 | -97 | -95.2 | -94.0 |
| n255 | 30 |  | -97.3 | -95.3 | -94.2 |
|  | 60 |  | -97.7 | -95.6 | -94.4 |
|  | 15 | -100.1 | -96.9 | -95.1 |  |
| n254 | 30 |  | -97.2 | -95.2 |  |
|  | 60 |  | -97.6 | -95.5 |  |

- References

- R4-2419897, Way Forward for [113][310] NR_NTN_Ph3_UE_RF, Qualcomm Inc., RAN4#113, Orlando, United States, November 18 – November 22, 2024.

- R4-2416462, “UE RF requirements for NR NTN RedCap”, Qualcomm Inc., ., RAN4#112bis, Hefei, China, October 14 – October 18, 2024.