TSG-RAN Working Group 4 (Radio) meeting #114	R4-2501647

Athens, EU, 17th to 21th February 2025

Source:	Ericsson

Title:	TS 38.141-2 Annex I: Discussion on improvement of TRP measurement methods

Agenda item:	7.13.4.3

Document for:	Approval

# Introduction

One of the work areas of the work item on NR base station (BS) RF requirement evolution for FR1/FR2 and testing simplification [1], targets the BS TRP estimation methods in Annex I in TS 38.141-2. The goal is to improve applicability and description. As this annex is normative, we need to be very clear how the manufacturer performs the TRP measurement procedures based on the grids described in this annex, in which order and what are the criteria for deciding compliance.

In a previous contribution [2] we highlighted the most important issues, some being received as part of feedback from regulators during drafting of the harmonised standard in Europe and IEEE/ANSI C63.26 in US.

At many places in Annex I of TS 38.141-2 there are alternatives of measurement grids that are supposed to produce similar outcomes (at least according to the MU analysis in the TR 37.941). There are probably even more alternatives which are not included at this moment. This range of choices is not appreciated by regulators, as it is not clear if they produce or not the same pass/fail outcome. It would however be beneficial to agree on one procedure that gives an accurate TRP estimate as the reference, and then relate other methods to this.

# Discussion

#### Grid vs. method

The way Annex I is organized seem to give the impression that there are many test methods to pick from and the end result can be different, depending on the choice. Even if this is not necessarily the truth, there are risks associated with presenting the alternatives in this manner. First, we need to make a difference between “grid” and “method”. Picking the right grid is part of the method just to start with. The measurement “method” also includes the post processing of the data, which is necessary to estimate the TRP.

We propose that there should be one reference grid, preferably called the “Spherical equal angle grid” with angular steps not exceeding the “reference steps”. Any other grid can be also used as long as it can be proven that it gives either the same TRP result withing acceptable tolerances or an overestimate of the TRP, where applicable. For all unwanted emissions, an overestimate serves well, while for the BS radiated power the TRP value should be as accurate as possible.

One approach can be to describe the measurement method with flowcharts, to visualize the logic behind deciding the grid and the criteria for pass/fail.  Examples of such flowcharts are shown below.

Figure 2.1-1 Example of TRP method for spurious emissions.

Figure 2.1-2. Example of TRP method for OOB emissions.

Proposal 1: Agree on one single reference method for accurate TRP.

Proposal 2: Use a flowchart (with possibly many steps) to describe a single method for spurious emissions.

Proposal 3: Use a flowchart (with possibly many steps) to describe a single method for in-band and out-of-band TRP.

Proposal 4: Clean up Annex I of redundant grids and other information, add a note that alternative sampling grids and techniques/chambers can be used, as long as they are proved to lead to the same outcome as the reference method. Keep detailed descriptions in the TR 37.941.

#### Pre-scan

Pre-scan is the natural starting point, but is described in the last sub-clause I.13 of Annex I. The technical details of the pre-scan are missing, especially steps 1) and 2), i.e., instrument settings (frequency step, RBW, …) and angular steps.

How to set up the pre-scan by using a turntable to rotate the EUT is not described. Handheld test antenna may be out-dated, since the actual TRP measurement for a BS with significant output power calls for using positioner-controlled measurements. An interesting alternative is to define a pre-scan method based on reverberation chamber. Using beam-sweeping during a pre-scan can also bring additional benefits in term of reducing the test time and improving the result accuracy.

When it comes to polarization, there is a potential 3dB discrepancy between measuring the case linear polarizations and the total EIRP of a circularly polarized emission. The polarizability of the emission is a topic that is not described at all. If the device transmits two independent signals on two orthogonal polarizations the polarization is undefined, for example.

Proposal 5: Pre-scan needs improved description and further investigations. Investigate use of beam sweeping. Consider an alternative method based on reverberation chamber.

#### TRP uncertainty

The uncertainty of the TRP using EIRP measurements in anechoic chambers (FAC, CATR or PWS), can be divided in three categories

- EIRP errors (calibration and measurement),

- Finite angular step,

- Summation over angular grid

Note 1: The EIRP errors are already covered in the MU tables for EIRP and the corresponding chamber type. These errors can be made smaller by improving the power calibration using e.g. power sensors.

Note 2: The reference steps are decided to have negligible sampling error in the case of “full sphere – reference steps” case. Sparse sampling is taken care of by using the ΔTRP correction factor.

Note 3: The post-processing error is the effect of random errors of the EIRP values when summed to get a TRP estimate. It is well known that the effect of summing values with random errors results in a decrease of the relative error, i.e., the uncertainty in TRP is most probably lower than the uncertainty in EIRP. Hence, the Summation error should be removed. This can be further studied if desired. This value was decided without an appropriate error propagation model.

Proposal 6: Investigate the effect of random errors of EIRP values on TRP by using the sum formula in Annex I clause 1.2.1.

# Summary of proposals

Proposal 1: Agree on one single reference method for accurate TRP.

Proposal 2: Use a flowchart (with possibly many steps) to describe a single method for spurious emissions .

Proposal 3: Use a flowchart (with possibly many steps) to describe a single method for in-band and out-of-band TRP.

Proposal 4: Clean up Annex I of redundant grids and other information, add a note that alternative sampling grids and techniques/chambers can be used, as long as they are proved to lead to the same outcome as the reference method. Keep detailed descriptions in the TR 37.941.

Proposal 5: Pre-scan needs improved description and further investigations. Investigate use of beam sweeping. Consider an alternative method based on reverberation chamber.

Proposal 6: Investigate the effect of random errors of EIRP values on TRP by using the sum formula in Annex I clause 1.2.1.

# References

[1]	RP-243270, “Revised WID: NR base station (BS) RF requirement evolution for FR1/FR2 and testing”, ZTE Corporation, HiSilicon, Huawei, Ericsson, Nokia, NEC

[2]	R4-2419585, “Improvement of TRP measurement methods in TS 38.141-2, Annex I”, Ericsson