# 3GPP TSG-RAN WG4 Meeting #114	R4-2501708

# Athens, GR, 17th – 21st February, 2025

Agenda item:	7.23.2.1

Source: 	Qualcomm Incorporated

Title: 	Views on new requirements for SBFD

Document for:	Approval

# Introduction

In RAN#102, the work item on evolution of NR duplex operation was approved [1]. From RAN4 RF perspective, the objective of the work item is as follows:

The objectives are as follows: - Specify BS RF requirements for SBFD operation at gNB [RAN4]

In RAN4#113, a WF was agreed in [2]. In this contribution we share our views on existing BS RF requirements for Rel-19 SBFD.

# New BS RF requirements for SBFD operation

In [2], the following was agreed regarding the potentially new in-channel adjacent subband leakage ratio and in-channel adjacent subband blocking requirements:

- New requirement for in-channel adjacent subband leakage ratio should be defined - FFS whether the requirement can be defined similar to the ACLR requirement and use existing ACLR requirement as baseline. - New requirement for in-channel adjacent subband blocking should be defined - FFS the impact of in-channel interference source(s), with and without gNB-to-gNB CLI handling schemes, based on reasonable expected gNB-to-gNB isolation. - FFS in-channel selectivity (ICS) requirement can cover in-channel adjacent subband blocking requirement. - The requirement level shall be driven by feasibility consideration.

All the above-mentioned new requirements are related to the in-band operation, i.e., intra-operator, and are different than existing ACLR/ ACS/ blocking requirements, which are aimed to ensure feasible deployment of inter-operator adjacent operation. Historically, RAN4 did not specify any in-band emissions for legacy TDD BS, since it is implementation dependent and can be addressed via proprietary solutions.

Observation 1: RAN4 has not specified before in-band emission requirements for legacy TDD BSs as it is implementation dependent and different solutions might vary among vendors.

Additionally, during the study item phase, legacy ACLR and ACS was assumed for inter-channel leakage and inter-channel selectivity, this was mainly assumed to progress the work on the coexistence between legacy TDD and new SBFD deployments for the different simulation scenarios.

Observation 2: During study phase, RAN4 agreed to adopt legacy ACLR and ACS values for in-channel adjacent subband leakage and in-channel adjacent subband selectivity.

In our views, it is a viable way forward to follow what was done in the study item as a baseline and further agree on methodologies that could be agreed within RAN4 to confirm the feasibility of such way forward. One challenge for the in-channel new requirements is the dependency on the self-interference capability yields the definition of a new inter-subband requirement difficult to agree between different companies. As observed in Section 9 in TR 38.853, different implementations can be assumed among vendors, leading to different self-interference, co-sector interference, and inter-site interference mitigation capabilities. Moreover, employed technologies and mitigation techniques in the SBFD-capable gNB vary within RAN4, and thus, room for implementation differences should be considered when defining the new inter-subband leakage and blocking requirements.

Observation 3: The in-channel subband sub-band Tx leakage and blocking depend on the self-interference, inter-site and inter-sector interference considerations; thus, they are implementation dependent.

Proposal 1: RAN4 to agree as a starting point on reusing legacy ACLR for the new in-channel adjacent subband leakage and further discuss a methodology to confirm its feasibility.

One of the main differences between the adjacent blocking requirement that are being discussed for SBFD-capable gNB and the in-channel adjacent subband blocking is the impact of the inter-gNB CLI mitigation/ handling techniques. Since in-channel emissions are being investigated, CLI handling techniques are a cornerstone of enabling intra-operator deployment of SBFD, as they offer solutions to mitigate intra-operator inter-gNB CLI. Accordingly, it is recommended that RAN4 consider the impact of the CLI handling schemes when analysing feasible in-channel adjacent subband blocking requirement.

Proposal 2: RAN4 consider the impact of inter-CLI handling schemes when analysing feasible in-channel adjacent subband blocking requirement.

# Conclusion

In this contribution, we provided our views on new BS RF requirements for SBFD with the following observations and proposals:

Observation 1: RAN4 has not specified before in-band emission requirements for legacy TDD BSs as it is implementation dependent and different solutions might vary among vendors.

Observation 2: During study phase, RAN4 agreed to adopt legacy ACLR and ACS values for in-channel adjacent subband leakage and in-channel adjacent subband selectivity.

Observation 3: The in-channel subband sub-band Tx leakage and blocking depend on the self-interference, inter-site and inter-sector interference considerations; thus, they are implementation dependent.

Proposal 1: RAN4 to agree as a starting point on reusing legacy ACLR for the new in-channel adjacent subband leakage and further discuss a methodology to confirm its feasibility.

Proposal 2: RAN4 consider the impact of inter-CLI handling schemes when analysing feasible in-channel adjacent subband blocking requirement.

# References

- RP-240789 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, HiSilicon, 3GPP TSG RAN#103.

- R4-2419907, “Way Forward for [113][306] NR_duplex_evo_General”, RAN4#113, Samsung.