3GPP TSG-RAN WG4 Meeting #114	R4-2501785

# Athens, Greece, February 17th – 21st 2025

Source:	Ericsson

Title:	Discussion on general aspects of NR SCM for demodulation

Agenda item:	7.16.1

Document for:	Discussion

# Introduction

In RAN4#113 meeting, companies have agreement on simulation parameters of interested test cases for comparison between different methodologies and further alignment [1].

In this contribution, the general aspects, such as TR content and structure, are furtherly discussed and draft TR will be delivered based on the discussion.

# Discussion

Content of Technical Report 38.753

Agreement:

1 Scope 2 References 3 Definitions of terms, symbols and abbreviations 3.1 Terms 3.2 Symbols 3.3 Abbreviations 4 Background 4.1 General 4.2 Study Item Objectives 5 Spatial Channel Modelling Approaches 5.1 CDL Approaches 5.2 TDL Approaches 6 Comparison of Spatial Channel Models 6.1 SU-MIMO (note: 2 companies for the work split) 6.2 MU-MIMO 7 [Implementation/Simulation] Alignment of Spatial Channel Models 8 Conclusions Annex A : Channel Measurements Annex <B>: [Channel Model XYZ] Annex <C>: [Informative: related considerations]

## 2.1	Scalable CDL model

In WF from RAN4#113 meeting [1], a general flow chart of channel model derivation for TR38.753 is captured for furtherly discussion, see below.

For demodulation perspective, all randomized parameters should be fixed. But the problem is the CDL model would be quite different according to different test cases under specific scenarios, especially BS and UE antenna configuration, Doppler vectors etc. But the propagation modeling could be general which only depends on deployment, such as Uma, Umi etc. Similar as method in TR38.901, general CDL models could be derived for TR38.753 with partially fixed parameters on propagation model part but leave other parts to be scalable for specific test cases.

Following procedures could be considered and furtherly studied:

- Derive scalable deterministic CDL models based on 38.901 model for MIMO demodulation:

  - Reuse relative delay and power per cluster defined in CDL models in 38.901.

  - Set UE is static as 38.901.

  - Assume one isotropic antenna element for both BS and UE side.

  - Set desired angles, desired angel spread, coupling patterns and initial phase of polarization matrix etc., as 38.827 have done.

  - Derive channel model profiles (for example, call them “CDLM-A/B/C”) which can be considered as a common model for a frequency range.

- Derive specific CDL models for demodulation requirements:

  - Set desired delay spread, desired UE velocity vector (Doppler shift and directions) and proper BS and UE antenna configurations according to a typical feature scenario.

  - Derive specific model profiles on top of scalable channel model profiles from step 1 above.

Proposal 1	Consider deriving scalable CDL model with partially deterministic parameters for TR38.753 which could be furtherly derived to full deterministic model for different test cases.

## 2.2	Content of Chapter 6 and 7 in TR 38.753

The chapter 7 of TR38.753 in WF [1] is tentatively agreed as “[Implementation/Simulation] Alignment of Spatial Channel Models”. The content of this chapter is not quite clear to companies, then here we would like to discuss it a bit, especially the difference from chapter 6.

Agreed title of chapter 6 is “Comparison of Spatial Channel Models”, so it should focus on the comparison between different models, such as CDL and TDL approaches, to have better understanding on the pros and cons of models. Of course, results from different companies also needs to be compared but full performance curves might be proper to see the whole picture. It would be helpful to make conclusion on which SCM approach is preferred.

As for chapter 7, the purpose of alignment after comparison is to get alignment on channel model implementation to have agreed channel profile at the first step, and then to get alignment on simulation results for specific test cases especially for the potential requirement purpose. In that case, chapter 7 could have two subclauses:

- The first subclause could describe for the preferred/aligned models (not all compared models in chapter 6) and give some derivation guide if necessary (e.g., scalable deterministic CDL models and full deterministic CDL models).

- The second subclause could capture performance value from companies with preferred model at a certain metric per test case (e.g., SNR at 70% normalized throughput for SU-MIMO PDSCH with CDL model X). The gap could be furtherly analyzed and give some observations.

To summarize, the content of the chapter 7 could be as following:

7	Alignment of Spatial Channel Models

7.1	Channel model approach alignment

7.1.1	CDL approach

7.1.1.1 	Scalable deterministic model description

7.1.1.2	Full deterministic model description

7.1.2	TDL approach

7.1.2.1	Scalable deterministic model description

7.1.2.2	Full deterministic model description

7.2	Simulation results alignment

7.2.1	CDL model X (full model version)

7.2.1.1	SU-MIMO PDSCH with 8 layers

7.2.1.2	SU-MIMO PMI with 4 layers

7.2.2	TDL model Y (full model version)

….

Proposal 2	Capture preferred and aligned model description in chapter 7.

Proposal 3	Capture performance value from companies with preferred model at a certain metric per test case in chapter 7.

Proposal 4 	Use sub-clause heading above for chapter 7.

# Conclusions

In the previous sections we made the following observations:

No table of figures entries found.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Consider deriving scalable CDL model with partially deterministic parameters for TR38.753 which could be furtherly derived to full deterministic model for different test cases.

Proposal 2	Capture preferred and aligned model description in chapter 7.

Proposal 3	Capture performance value from companies with preferred model at a certain metric per test case in chapter 7.

Proposal 4 	Use sub-clause heading above for chapter 7.

# References

[1] 	R4-2419782, Way Forward for [113][320] FS_NR_demod_SCM, Nokia

[2]