3GPP TSG-RAN WG4 Meeting #114									                        R4- 2501942

Athens, Greece, Feb. 17 – 21, 2025

Agenda item:			7.12.2

Source:	Ericsson

Title:	Discussion on TAE requirement of ATG BS DL MIMO

Document for:	Approval

# Introduction

In 3GPP release 18, RAN4 has completed the standardization work for the air-to-ground (ATG) network WI, and the main work focused on the FR1 single carrier scenario. To enhance the throughput to meet growing ATG capacity demand and fully utilize operators’ spectrum, the RAN plenary meeting #103 has approved a new WI on enhancements for the air-to-ground network for NR [1].

# Discussion

In the last RAN meeting, additional objectives (DL MIMO) were agreed [2]. The updated objectives are shown below:

The core part of the work item includes:

- Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4]:

  - FR1 intra-band contiguous CA

    - Example band combination: CA_n79C

  - FR1+FR1 inter-band CA

    - Example band combination: CA_n3A-n39A

- Specify the RF requirements for support of UL MIMO with 2TX for single CC for UE. [RAN4]

- Specify the TAE requirements for support of DL MIMO for BS. [RAN4].

- Specify signaling and UE capabilities if necessary [RAN2].

The following session will discuss the TAE requirement for ATG BS DL MIMO.

## RF requirements for ATG BS DL MIMO

As presented in [3], carrier aggregation is a feature primarily relevant to UE and, in most cases, does not directly impact BS. The feature is quite often confused with multi-carrier. In a multi-carrier scenario, a BS can transmit data to various UEs. At the same time, a non-multi-carrier BS can also support CA. Therefore, the multi-carriers BS does not necessarily require specific requirements.

In the Rel-18 discussion, the ATG BS reuses the majority of the NR BS RF requirements except NB-IoT, MIMO, and CA. The current NR FR1 BS TAE requirements in TS38.104 are listed below:

#### 6.5.3.2	Minimum requirement for BS type 1-C and BS type 1-H

For MIMO transmission, at each carrier frequency, TAE shall not exceed 65 ns.

For intra-band contiguous carrier aggregation, with or without MIMO, TAE shall not exceed 260ns.

For intra-band non-contiguous carrier aggregation, with or without MIMO, TAE shall not exceed 3µs.

For inter-band carrier aggregation, with or without MIMO, TAE shall not exceed 3µs.

The time alignment error requirements for NB-IoT are specified in TS 36.104 [13] clause 6.5.3.

#### 9.6.3.2	Minimum requirement for BS type 1-O

For MIMO transmission, at each carrier frequency, OTA TAE shall not exceed 65 ns.

For intra-band contiguous carrier aggregation, with or without MIMO, OTA TAE shall not exceed 260 ns.

For intra-band non-contiguous carrier aggregation, with or without MIMO, OTA TAE shall not exceed 3 µs.

For inter-band carrier aggregation, with or without MIMO, OTA TAE shall not exceed 3 µs.

Since the BS DL MIMO was added to the objective and no other technical issues exist, it is natural to reuse the MIMO TAE requirement for the ATG BS as well.

- Reuse the current TS38.104 TAE requirements for ATG BS support DL MIMO.

- Remove the sentence “There is no TAE requirement for ATG BS” in 6.5.3.1 and 9.6.3 in TS38.104.

# Conclusion

Based on the discussion in the previous sections, we propose the following:

Proposal 1	Reuse the current TS38.104 TAE requirements for ATG BS support DL MIMO.

Proposal 2	Remove the sentence “There is no TAE requirement for ATG BS” in 6.5.3.1 and 9.6.3 in TS38.104.

# References

- RP-240804, New WID on Enhancements for Air-to-ground network for NR.

- RP-243285, Revised WID on Enhancements for Air-to-ground network for NR. CMCC, ZTE, CATT, Huawei, HiSilicon, Verizon, China Telecom, China Unicom.

- R4-2405522, Discussion on RF considerations of ATG BS CA, Ericsson.