3GPP TSG-RAN WG4 Meeting #114	R4-2501948

Athens, Greece, Feb 17th – Feb 21st, 2025

Source:	Nokia, Anterix

Title:	Support of 3MHz channel bandwidth for (e)RedCap devices

Agenda item:	5.33.2

Document for:	Discussion

Introduction

In the last RAN plenary meeting, Anterix raised the request for supporting 3 MHz channel bandwidth for (e)RedCap devices. In their contribution, they outlined the justification for the request [1]. In RAN#106 no new WI for this request was agreed. However, it was discussed that this work may be pursued in the TEI path.

In this document, we present our views on what technical changes in various specifications are required and the feasibility of this work in the TEI path. CR associated with this paper is R4-2501947

Discussion

Additional channel bandwidth

The basic assumption is that 3 MHz will be the additional channel bandwidth supported by the (e)RedCap devices. In the current specification, for bandwidth restricted eRedCap devices, requirements are based on 5MHz channel bandwidth and that shall not change by the addition of 3 MHz channel bandwidth.

Introduction of this is currently being discussed in RF session.

Required RRM changes

If support of 3MHz channel bandwidth for (e)RedCap devices is agreed and introduced by RF session, there is a need for introducing the related RRM requirements.

The basic premise of the work is to bring support for 3 MHz channel bandwidth for low cost (e)RedCap devices in n106. Currently, (e)RedCap devices may besides 2Rx also support 1 RX and HD-FDD mode. However, introducing support of 3 MHz channel bandwidth with 1 RX and HD-FDD under TEI may not be feasible.

However, with a limited scope, as discussed in RF session, only focusing on 2Rx Redcap devices, the work is feasible under TEI.

Support for 1Rx and HDFDD Redcap would need a new WI due to the increased scope.

Hence, with restricting the scope to band n106 with (e)RedCap devices with 2 RX only support, work can be finished under TEI.

With such restriction it is also feasible to finalize the necessary RRM work within TEI. Assuming only considering 2Rx devices the requirements as agreed in Rel-18 can apply also for Redcap.

Assuming only considering 2Rx devices it is also feasible to finalize the necessary RRM work within TEI.

We therefore propose, conditioned agreement in RF session, to introduce the necessary RRM requirements for 3MHz channel bandwidth for (e)RedCap devices.

Conditioned agreement in RF session, introduce RRM requirements for 3MHz channel bandwidth for (e)RedCap devices.

Conclusion

The following Observations and Proposals were made:

Observation 1: Assuming only considering 2Rx devices it is also feasible to finalize the necessary RRM work within TEI.

Proposal 1: Conditioned agreement in RF session, introduce RRM requirements for 3MHz channel bandwidth for (e)RedCap devices.

- References

- RP-242812, Motivation for new WI on less than 5 Mhz (e)RedCap Support in Rel-19, Anterix, RAN#106, Madrid, Spain, Dec 9 – Dec 12, 2024.

- R4-2501947, (TEI18) Support of 3MHz channel bandwidth for RedCap devices RRM CR [RedCap_3MHz], RAN4#114, Athens, Greece, Feb 17 – Feb 21, 2025.