# 3GPP TSG-RAN WG4 Meeting # 114                                                         R4-2501986

# Athens, Greece, 17 – 21 February 2025

# Title:	DC_(n)8AA MSD and clean-ups for DC_(n)71AA, DC_(n)5AA

# Agenda Item:	6.2.2

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document is a companion to CR [1] which introduces the missing MSD test point for DC_(n)8AA [2], simplifies the number of test points for DC_(n)71AA, DC_(n)5AA, and provides further corrections to 1UL MSD test points.

# Discussion

## Introduction of missing test point for DC_(n)8AA

At RAN4 meeting #113, the MSD test point for DC_(n)8AA was not captured for Rel-19 since only a Rel-18 was filed. We propose to correct this by introducing the test point of proposal 1 in companion CR [1].

Proposal 1: Adopt the following 1UL MSD requirements for B8 in DC_(n)8AA.

| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_(n)8AA | 8 | N/A | 10 | N/A | 935 | 19 | FDD |
|  | n8 | 905 | 20 | 20 (RBstart = 86) | 950 | N/A |  |

## DC_(n)71AA MSD test point simplification

The MSD test points for DC_(n)71AA are summarized in Table 1. Four test points were specified in Release 15 to reflect the BCS0 configuration set, four test points were added in Release 16 for BCS1 specificities, and in Release 17 another set of four tests points were added to capture 1UL MSD for BCS0, 1 and 2.

Table 1: MSD test points for DC_(n)71AA

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Test point # | EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| Rel-15 2UL BCS0 test points | #1 | DC_(n)71AA | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | N/A | FDD |
|  |  |  | n71 | 675.5 | 15 | 15 (RBstart = 0) | 629.5 | 1.8 |  |
|  | #2 | DC_(n)71AA | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | N/A |  |
|  |  |  | n71 | 680.5 | 5 | 5 (RBstart = 0) | 634.5 | 1.6 |  |
|  | #3 | DC_(n)71AA | 71 | 668 | 10 | 10 (RBend = 49) | 622 | N/A |  |
|  |  |  | n71 | 678 | 10 | 10 (RBstart = 0) | 632 | 1.7 |  |
|  | #4 | DC_(n)71AA | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 |  |
|  |  |  | n71 | 678 | 10 | 10 (RBend = 51) | 632 | 29.4 |  |
| Rel-16 2UL BCS1 test points | #5 | DC_(n)71AA | 71 | 665.5 | 5 | 5 (RBend =24) | 619.5 | N/A | FDD |
|  |  |  | n71 | 675.5 | 151 | 15 (RBstart = 0) | 6321 | 2.5 |  |
|  | #6 | DC_(n)71AA | 71 | 670.5 | 15 | 15 (RBend = 74) | 624.5 | N/A |  |
|  |  |  | n71 | 680.5 | 51 | 5 (RBstart = 0) | 6371 | 2.2 |  |
|  | #7 | DC_(n)71AA | 71 | 668 | 10 | 10 (RBend = 49) | 622 | N/A |  |
|  |  |  | n71 | 678 | 101 | 10 (RBstart = 0) | 634.51 | 2.5 |  |
|  | #8 | DC_(n)71AA | 71 | 668 | 10 | 10 (RBstart = 0) | 622 | 17.2 |  |
|  |  |  | n71 | 678 | 101 | 10 (RBend = 51) | 634.51 | 29.1 |  |
| Rel-17 1UL test points | #9 | DC_(n)71AA3 | 71 | N/A | 10 | N/A | 642.0 | 21.3 | FDD |
|  |  |  | n71 | 673.0 | 20 | 5 (RBstart = 0) | 627.0 | N/A |  |
|  | #10 | DC_(n)71AA3 | 71 | N/A | 15 | N/A | 639.5 | 5.4 |  |
|  |  |  | n71 | 670.5 | 15 | 5 (RBstart = 0) | 624.5 | N/A |  |
|  | #11 | DC_(n)71AA4 | 71 | N/A | 5 | N/A | 639.5 | 6.8 |  |
|  |  |  | n71 | 670.5 | 151 | 5 (RBstart = 2) | 627.0 | N/A |  |
|  | #12 | DC_(n)71AA5 | 71 | N/A | 5 | N/A | 634.5 | 6.4 |  |
|  |  |  | n71 | 670.5 | 15 | 5 (RBstart = 0) | 624.5 | N/A |  |
| NOTE 1: In accordance to BCS1, the NR uplink bandwidth is specified as in this table, but the corresponding NR downlink bandwidth is 5 MHz larger. NOTE 2: The transmitters powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [4], with additional limits on configured maximum output power for the uplink according to clause 6.2B.4. NOTE 3: Applicable only to BCS 2. NOTE 4: Applicable only to BCS 1. NOTE 5: Applicable only to BCS 0. |  |  |  |  |  |  |  |  |  |  |

Observation 1: There are 12 MSD test points solely for DC_(n)71AA. This seems a bit excessive compared to other band combinations. Note 1 is captured twice for the Release 16 test points. It is sufficient to capture note 1 only in the Channel BW allocation column.

Observation 2:

- Test points #1/2/3/4 and #5/6/7/8 have identical UL Fc, CBW, and UL RB configuration. Only the DL CBW is 5 MHz larger for BCS1 (point #5,6,7,8) than for BCS0 (#1,2,3,4).

- MSD levels for test points #1,2,3 and #5,6,7 are very small due to near-contiguous RB allocations. The MSD may occur in some devices and do not seem necessary to verify the performance of the UE RF-FE.

- It may be sufficient to verify only the test points for which the DL bands are affected by the highest MSD levels.

Based on observation 1 and 2, we propose the adopt the following simplification.

Proposal 2: Simplify the MSD requirements for DC_(n)71AA by:

- Removing MSD test points #1,2,4,5,6,7 and #10 (Table 1).

- Deleting the “1” from the column “FC (DL) (MHz)” for test points #5,6,7,8 (Table 1).

## Simplification of DC_(n)5AA 1UL MSD test points

Table 2 shows that four 1UL MSD test points are specified for DC_(n)5AA. Unlike DC_(n)71AA in which MSD test points cover three different BCS, there is only one BCS (BCS0) for DC_(n)5AA. Therefore, having four 1UL test points to cover just one BCS seems a bit excessive. By revisiting each of these test points we observe the following opportunities for simplification:

- Test point #1verifies the B5 MSD due to 1UL IMD5, with 5.2 dB MSD

- Test point #2 and #3 verify the B5 MSD due to partial overlap of both IMD5 and IMD7, also both with 5.2 dB MSD,

- Test point #4 verifies the B5 MSD due to partial overlap of IMD7 and IMD9, with a low MSD of 1.5 dB.

Based on these observations, it seems that the verification of the UE RF-FE performance may be sufficiently well-tested using only test point #1. Test points #2 and #3 are nearly identical, and test point #4 verifies high IMD orders.

Table 2: 1UL Intra-band Contiguous MSD Test Points from Table 7.3B.2.1-1

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test point # | EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| #1 | DC_(n)5AA | 5 | 826.5 | 5 | N/A | 871.5 | 5.2 | FDD |
|  |  | n5 | 839 | 20 | 20 (RBend = 105) | 884 | N/A |  |
| #2 | DC_(n)5AA | 5 | 829 | 10 | N/A | 874 | 5.2 |  |
|  |  | n5 | 841.5 | 15 | 20 (RBend = 78) | 886.5 | N/A |  |
| #3 | DC_(n)5AA | 5 | 831.5 | 5 | N/A | 876.5 | 5.2 |  |
|  |  | n5 | 841.5 | 15 | 20 (RBend = 78) | 886.5 | N/A |  |
| #4 | DC_(n)5AA | 5 | 834 | 10 | N/A | 879 | 1.5 |  |
|  |  | n5 | 844 | 10 | 25 (RBend = 51) | 889 | N/A |  |
| NOTE 1: In accordance to BCS1, the NR uplink bandwidth is specified as in this table, but the corresponding NR downlink bandwidth is 5 MHz larger. NOTE 2: The transmitters powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [4], with additional limits on configured maximum output power for the uplink according to clause 6.2B.4. NOTE 3: Applicable only to BCS 2. NOTE 4: Applicable only to BCS 1. NOTE 5: Applicable only to BCS 0. |  |  |  |  |  |  |  |  |

Observation 3: There are four 1UL MSD test points for only one DC_(n)5AA BCS (BCS0). This seems a bit excessive compared to other band combinations. A closer inspection shows that it may be sufficient to verify a UE RF-FE performance using the first MSD test point of Table 2.

Proposal 3: Simplify the MSD requirements for DC_(n)5AA by:

- Removing MSD test points #2,3,4, (Table 2).

## Remaining 1UL MSD test point corrections

Table 3 summarizes the remaining 1UL MSD test points specified in Table 7.3B.2.1-1, where we have removed the proposed DC_(n)5AA clean-up proposal and where we highlight:

- the correct use of “N/A” for non-applicable UL parameter “Fc (UL)”, “UL allocation,” and “MSD” (in green)

- the UL carrier frequencies (“Fc (UL)”) corrected to “N/A” (in yellow)

Table 3: 1UL Intra-band Contiguous MSD Test Points from Table 7.3B.2.1-1

| EN-DC configuration / channel allocations /MSD |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| DC_(n)3AA | 3 | N/A | 15 | N/A | 1842.5 | 6.2 | FDD |
|  | n3 | 1770.0 | 30 | 10 (RBstart = 150) | 1865.0 | N/A |  |
| DC_(n)5AA | 5 | 826.5 | 5 | N/A | 871.5 | 5.2 | FDD |
|  | n5 | 839 | 20 | 20 (RBend = 105) | 884 | N/A |  |
| DC_(n)12AA | 12 | 703.5 | 5 | N/A | 733.5 | 4.5 | FDD |
|  | n12 | 711 | 10 | 20 (RBend = 51) | 741 | N/A |  |
| DC_(n)71AA3 | 71 | N/A | 10 | N/A | 642.0 | 21.3 | FDD |
|  | n71 | 673.0 | 20 | 5 (RBstart = 0) | 627.0 | N/A |  |
| DC_(n)71AA3 | 71 | N/A | 15 | N/A | 639.5 | 5.4 |  |
|  | n71 | 670.5 | 15 | 5 (RBstart = 0) | 624.5 | N/A |  |
| DC_(n)71AA4 | 71 | 680.5 | 5 | N/A | 639.5 | 6.8 |  |
|  | n71 | 670.5 | 151 | 5 (RBstart = 2) | 627.0 | N/A |  |
| DC_(n)71AA5 | 71 | 680.5 | 5 | N/A | 634.5 | 6.4 |  |
|  | n71 | 670.5 | 15 | 5 (RBstart = 0) | 624.5 | N/A |  |
| NOTE 1: In accordance to BCS1, the NR uplink bandwidth is specified as in this table, but the corresponding NR downlink bandwidth is 5 MHz larger. NOTE 2: The transmitters powers shall be set to PUMAX, as defined in TS 38.101-1 [2], TS 38.101-2 [3], and TS 36.101 [4], with additional limits on configured maximum output power for the uplink according to clause 6.2B.4. NOTE 3: Applicable only to BCS 2. NOTE 4: Applicable only to BCS 1. NOTE 5: Applicable only to BCS 0. |  |  |  |  |  |  |  |

Observation 4: Several E-UTRA UL carrier frequencies (“Fc (UL)”) remain specified for 1UL MSD test points in Table 7.3B.2.1-1. They should be changed to “N/A” for consistency.

Proposal 4: Change “Fc (UL)” to “N/A” for:

- B5 for the remaining 1UL DC_(n)5AA MSD test point,

- B12 for the 1UL DC_(n)12AA MSD test point,

- B71 for the last two 1UL DC_(n)71AA MSD test points.

# Conclusion

This paper captures our views on simplifying the DC_(n)71AA MSD test points and on introducing the DC_(n)8AA MSD test point.

Proposal 1: Adopt the following 1UL MSD requirements for B8 in DC_(n)8AA.

| EN-DC configuration | E-UTRA/NR band | FC (UL) (MHz) | Channel bandwidth (MHz) | UL allocation (LCRB) | FC (DL) (MHz) | MSD (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DC_(n)8AA | 8 | N/A | 10 | N/A | 935 | 19 | FDD |
|  | n8 | 905 | 20 | 20 (RBstart = 86) | 950 | N/A |  |

Observation 1: There are 12 MSD test points solely for DC_(n)71AA. This seems a bit excessive compared to other band combinations. Note 1 is captured twice for the Release 16 test points. It is sufficient to capture Note 1 only in the Channel BW allocation column.

Observation 2:

- Test points #1/2/3/4 and #5/6/7/8 have identical UL Fc, CBW, and UL RB configuration. Only the DL CBW is 5 MHz larger for BCS1 (point #5,6,7,8) than for BCS0 (#1,2,3,4).

- MSD levels for test points #1,2,3 and #5,6,7 are very small due to near-contiguous RB allocations. The MSD may occur in some devices and do not seem necessary to verify the performance of the UE RF-FE.

- It may be sufficient to verify only the test points for which the DL bands are affected by the highest MSD levels.

Based on observations 1 and 2, we propose the adopt the following simplification:

Proposal 2: Simplify the MSD requirements for DC_(n)71AA by:

- Removing MSD test points #1,2,4,5,6,7, and #10 (Table 1).

- Deleting the “1” from the column “FC (DL) (MHz)” for test points #5,6,7, and 8 (Table 1).

Observation 3: There are four 1UL MSD test points for only one DC_(n)5AA BCS (BCS0). This seems a bit excessive compared to other band combinations. A closer inspection shows that it may be sufficient to verify UE RF-FE performance using the first MSD test point of Table 2.

Proposal 3: Simplify the MSD requirements for DC_(n)5AA by:

- Removing MSD test points #2,3, and 4 (Table 2).

Observation 4: Several E-UTRA UL carrier frequencies (“Fc (UL)”) remain specified for 1UL MSD test points in Table 7.3B.2.1-1. They should be changed to “N/A” for consistency.

Proposal 4: Change “Fc (UL)” to “N/A” for:

- B5 for the remaining 1UL DC_(n)5AA MSD test point,

- B12 for the 1UL DC_(n)12AA MSD test point,

- B71 for the last two 1UL DC_(n)71AA MSD test points.

# References

- R4-2501932 (NR_CADC_SUL_R19-Core) Cat F CR for TS 38.101-3-j00 DC_(n)8AA MSD and MSD clean-ups, 3GPP TSG-RAN WG4 Meeting #114, Athens, Greece, Skyworks Solutions, Inc, Murata Corporation Ltd.

- R4-2420526 CR for TS 38.101-3-i70 Corrections to intra-band MSD requirements, 3GPP TSG-RAN WG4 Meeting #113, Orlando, USA, Skyworks Solutions, Inc, Murata Corporation Ltd.