3GPP TSG-RAN4 Meeting #114	R4-2502165

# Athens, Greece 17th – 21st February, 2025

Source: 	Qualcomm Incorporated

Title: 	RRM Core Requirements for AI/ML Beam Management

Agenda item:	7.19.3

Document for:	Discussion

# Introduction

This paper focuses on RRM core requirements for AI/ML based beam management.

RAN4 made the following agreements during the last meeting [7][8][9]

Issue 2-3: RSRP Absolute accuracy Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP]. Issue 2-5: Requirements for beam predictions Agreement: - Requirements to be defined: - Measurement period for prediction (how long will set B be measured before making a prediction) - FFS on whether the legacy measurement period can be reused or not. - Prediction accuracy - FFS how/what to define depending on the chosen KPI (e.g. predicted RSRP, beam ID, Top-K/1 or Top 1/K, etc) - TCI state known/unknown - FFS on whether the other requirements are needed - Issue 2-9: Test setup requirements Agreement: - For CDL or simplified CDL channel model, RAN4 to study the relationship between AoD range [and granularity], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For TDL channel model, RAN4 to study the relationship between [dynamic power range], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For single AoA, FSS how to derive Set A Issue 2-6: Measurement error impact evaluation RAN4 to perform study to assess the impact of measurement errors on prediction accuracy Perform LLS to evaluate the RSRP prediction accuracy and other KPIs - UE Model trained using ideal measurements Simulation assumptions in R4-2417211 can be used as starting point. Issue 3-5: Prediction delay requirements Agreement: - Prediction delay includes: - Measurement for prediction - FFS on whether the legacy measurement delay can be reused - Inference delay - Other components are FFS Test setup for Beam prediction testing Companies are invited to bring further analysis on the test setups with single AoA and multiple AoAs. Issue 3-2: UE Rx beam knowledge Agreement: - Whether the TCI state associated with a predicted Tx beam is known or unknown is FFS - RAN4 to further discuss whether to mandate that the UE has to predict the Rx beam paired with the predicted Tx beam or not. - Other way to know UE Rx beams is not precluded - UE capability is one of the solutions

RAN1 made the following agreements in [5][6]:

- RAN1#117: Working Assumption - Regarding the associated ID for Rel-19, the UE assumes that NW-side additional conditions with the same associated ID are consistent at least within a cell. - FFS: whether/how UE assumption can be applicable for multiple cells (including the feasibility study). - RAN1#118 agreement: For UE sided model in beam management, support associated ID - [Working Assumption] - The associated ID at least can be configured within CSI framework - FFS on details - FFS on whether/how to configure/indicate the associated ID via other signal(s) and/or in other procedure(s)/framework(s) - UE may assume the similar properties of a DL Tx beam or beam set/list are associated with the same associated ID - FFS: whether/how to define similar properties of a DL Tx beam or beam set/list

# Discussion

## 2.1 Selection of Beam Prediction Metrics

3GPP focused on the following prediction metrics in [3]

Both spatial-domain DL beam prediction and temporal DL beam prediction are considered. For metrics for beam management requirements/tests, the following test metrics/KPIs are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" - Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" - Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, - Related measurement accuracy can be considered to determine x - Option 4: combinations of above options The overhead/latency reduction should be considered for the requirements as the side condition.

RAN1 discussed this issue during the last meeting and made the following agreement [9]:

Agreement For BM-Case1 and BM-Case2 with a UE-sided AI/ML model, for Option 2 (UE-assisted performance monitoring), - At least support Alt 1: Top 1 or Top K beam prediction accuracy (with or without margin) by comparing the prediction results and the Top 1 or Top K beam based on the measurements from a resource set/ resources for monitoring - FFS on detail definition of the metric, including whether/how to configure or define a window for calculation - FFS: on other details including how to configure the resource set/resources for monitoring, including - E.g. whether/how to use full set of Set A for measurement. If the full set A is not configured, whether/how to define the metric - FFS other alternatives

RAN1’s agreed Alt 1 (without margin) simply translates to RAN4’s option 2 (beam prediction accuracy). On the other hand, RAN1’s agreed Alt 1 (with margin) simply translates to RAN4’s option 3 (beam prediction accuracy with X dB margin between measured RSRP of predicted top K beams and actual strongest beam).

RAN1 has not yet agreed to support RAN4’s option 1 (“L1-RSRP accuracy”) yet. Hence, RAN4 should postpone defining requirement for L1-RSRP accuracy until RAN1 reaches a conclusion.

Observation 1: RAN1’s agreed beam prediction metric without or with margin simply translates to option 2 and option 3 of RAN4’s beam prediction metric.

Observation 2: Support for option 1 of beam prediction metric is still TBD in RAN1.

Technically, RAN4 can define performance requirements for both option 2 and option 3. However, option 3 provides more flexibility than option 2 for RAN4 to define a metric. Option 2’s drawback lies in the fact that option 2 (top-1 (%) version of option 2) does not distinguish between following two scenarios:

- UE’s top-1 predicted beam is adjacent to, but not same as, the top-1 strongest beam

- UE’s top-1 predicted beam is far from the top-1 strongest beam.

This drawback of option 2 is addressed in option 3. UE can pass option 3 if the maximum RSRP among UE’s top-K predicted beams lies within X dB of the RSRP of the strongest beam. Hence, option 3 allows UE to pass a test if UE’s top-1 predicted beam is adjacent to the top-1 strongest beam.

RAN1 and RAN2 have not yet decided the maximum number of beams that UE can convey in its inferred reporting. The maximum number of measured beams that TE can report is also upper bounded by legacy spec. If TE configures the setB and setA beams to be too close to each other and the actual measured RSRP of topK beams of setA lies within the range of UE’s measurement error, UE will not be able to pass option 2 focused test.

Observation 3: Option 2’s drawback lies in the fact that its pass/fail criteria don’t depend on whether the top predicted beams are adjacent to the strongest beams or not.

- If TE configures the setB and setA beams to be too close to each other and the actual measured RSRP of more than topK beams of setA lies within the range of UE’s measurement error, UE may not be able to pass option 2 focused test.

- Note: The maximum value of K depends on the maximum number of beams that UE can report for measurement and inference. The max value of K is decided by RAN1/RAN2 and RAN4 cannot control it.

Option 3 is a more refined version of option 2. It does not suffer from the constraints of option 2 and it retains the same usefulness of option 2. Even if more than topK beams of setA lies within the range of UE’s measurement error, UE will be able to pass option 3 tests as long as the measured RSRP of its predicted beam lies within X dB of the strongest beam. RAN4 can control the value of X dB and ensure that UE does not fail a test due to RAN4 allowed measurement error.

Observation 4: Option 3 does not suffer from the constraints of option 2 that got mentioned in observation 4. Even if more than topK beams of setA lies within the range of UE’s measurement error, UE will be able to pass option 3 tests as long as the measured RSRP of its predicted beam lies within X dB of the strongest beam.

- RAN4 can control the value of X dB and ensure that UE does not fail a test due to RAN4 allowed measurement error.

Proposal 1: RAN4 defines AI-ML BM requirement for option 3.

Proposal 2: RAN4 waits for simulation results and RAN1 progress before defining AI-ML BM requirement for option 2 and option 1 respectively.

## 2.2 TCI State Status of Predicted Tx Beam

During the last meeting, RAN4 discussed whether the TCI state of predicted Tx beam is known or unknown. RAN4 also discussed whether UE knows the RX beam paired with the predicted TX beam.

3GPP is considering L1-RSRP accuracy as a potential prediction metric for AI-ML based beam management. For both temporal and spatial beam prediction, UE must know its RX beam to predict L1-RSRP accuracy of a particular gNB TX beam. If UE does not know its RX beam corresponding to the gNB Tx beam, UE’s predicted L1-RSRP RSRP will be drastically different from the actual L1-RSRP.

Observation 5: 3GPP is considering L1-RSRP accuracy as a potential prediction metric for AI-ML based beam management. UE has to know its RX beam to predict L1-RSRP accuracy of a particular gNB TX beam.

3GPP is also considering top-1 (%) beam or top-K (%) beam as a prediction metric. UE has to predict the top beams of setA based on what UE would see at its baseband. To compare between gNB beams, UE needs to have some idea of the Rx gain associated with them. Hence, UE has to know the Rx beam of the predicted gNB beam even if it only has to predict top-1 (%) or top-K (%) beams.

Observation 6: To compare between gNB beams of setA, UE needs to have some idea of the Rx gain associated with them. UE has to know the Rx beam of the predicted gNB beam even if it only has to predict top-1 (%) or top-K (%) beams.

Figure 1: Spatial Beam Prediction

RAN4 needs to further discuss whether the TCI state corresponding to predicted TX beam is known or unknown.

RAN4 spec differentiates between known and unknown condition for TCI state in the following way [7]:

The TCI state is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target TCI state to the completion of active TCI state switch, where the RS resource for L1-RSRP measurement is the RS in target TCI state or QCLed to the target TCI state - TCI state switch command is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 L1-RSRP report for the target TCI state before the TCI state switch command - The TCI state remains detectable during the TCI state switching period - The SSB associated with the TCI state remain detectable during the TCI switching period - SNR of the TCI state ≥ -3dB Otherwise, the TCI state is unknown.

As above text shows, the SNR of the SSB associated with the TCI state needs to be >= -3 dB for TCI state to be known. These types of side conditions need to revisited to determine if the TCI state corresponding to the strongest beam is known or unknown.

Proposal 3: UE knows the RX beam paired with the predicted Tx beam from setA.

- FFS: TCI state of predicted TX beam.

## 2.3 Measurement Error and Beam Prediction Accuracy

### 2.3.1 Beam Prediction Accuracy without Measurement Error

We first show our system simulation results for 8 beam based setB and 64 beam based setA without any kind of measurement error. We will show simulation results for 32 beam based setA by the next meeting. We also displayed these results in [10]

#### Simulation Assumptions

we predict the best gNB beam ID in Set A based on the measured RSRPs of Set B. In simulations, we focus on the UMa scenario, and follow the agreed SLS simulation assumptions so far as summarized in Table 1. We assume there is no UE mobility. The dataset is generated from random UE location drops in a cell with three sectors. Spatial consistency procedure is performed based on 38.901 [2], to ensure the channel characteristics between neighbouring UEs are appropriately correlated. Key assumptions for the simulations are summarized in Table 1. The datasets used for training and testing correspond to different sets of UE locations. The gNB array has 32 antennas (64 per sector), 8 on azimuth and 4 on elevation. We assume 192 DFT beams in Set A in the cell, and 24 beams are down-selected for Set B (8 per sector). In Figure 2, we illustrate the beam pointing angles of Set A and Set B beams in a sector. We select the Set B in a way that the measured beams cover as much space in both elevation and azimuth directions as possible.

Table 1 Simulation assumptions

| Parameters | Value |
| --- | --- |
| Scenario | Uma, outdoor |
| Carrier frequency | 30 GHz |
| ISD | 200 m |
| BS antenna cfg | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ |
| gNB codebook (Set A) | 16 (azimuth) by 4 (elevation) beams per sector, in total Set A has 192 beams per cell |
| gNB codebook (Set B) | 8 beams down-selected from Set A per sector, as shown in Figure 42 (or in Figure 43 for Set B1), in total set B has 24 beams per cell |
| gNB antenna gain | 8 dBi |
| BS Tx power | 30 dBm for 20 RB bandwidth |
| SCS | 120KHz |
| Car penetration loss | Included |
| UE mobility and rotation | Not assumed, only consider a single time shot |

For the AI/ML based approach, we apply a fully connected (FC) layer based NN to predict the best beam in Set A. We also consider the following 2 baseline approaches to predict the best beam ID, in addition to a NN-based AI/ML model:

- Baseline-1 (linear interpolation-based approach): For the non-measured beams in Set A, we estimate their RSRPs by performing a 2D linear interpolation (and extrapolation, when applicable) based on the measured RSRPs and the pointing angle of the beams. We select the top K predicted beams based on the interpolated RSRPs.

- Baseline-2 (empirical PMF-based approach): We derive an empirical probability distribution of the best beam ID in Set A, given the best beam ID in Set B, based on the training dataset. For testing, we select the top K predicted beams based on the derived empirical distribution.

Figure 2 Illustration of pointing angles of Set A and Set B beams in a sector of the cell.

#### Evaluation Results

We present the beam prediction performance for use case1 in Table 2. For Top-K beam selection accuracy, we follow the KPIs for beam prediction accuracy. Compared with the baseline approach, our results show that NN based AI/ML model provide a significant performance gain, in terms of both beam selection accuracy and L1 RSRP difference. The linear interpolation-based approach (Baseline-1) provides the worst performance, as Set B beams are sparsely sampled from Set A, and the RSRPs in general cannot be approximated by a linear fit.

Table 2 Evaluation results for BM-Case1 (Use case 1) without model generalization for DL Tx beam prediction at UE side (Results for Set B, Set B subset of Set A)

| Qualcomm |  |  |  |  |
| --- | --- | --- | --- | --- |
| Assumptions |  | Number of beams [beams/beam pairs] in Set A | 192 | 192 |
|  |  | Number of beams [beams/beam pairs] in Set B | 24 | 24 |
|  |  | Baseline scheme | Empirical PMF-based approach | linear interpolation-based approach |
| AI/ML model input/output |  | Model input | RSRPs of Set B beams | RSRPs of Set B beams |
|  |  | Model output | Best Tx beam ID | Best Tx beam ID |
| Data size |  | Training | 4000 | 4000 |
|  |  | Testing | 600 | 600 |
| AI/ML model |  | Model description | FC layer based NN | FC layer based NN |
|  |  | Model complexity | 4.6K parameters | 4.6K parameters |
|  |  | Computational complexity | 9.8K FLOPs | 9.8K FLOPs |
| Evaluation results [with AI/ML/baseline (Baseline-2)] | Beam prediction accuracy (%) | Top-1 (%) | 63.5 / 28.3 | 63.5 / 10.7 |
|  |  | Top-2/1 (%) | 80.0 / 46.1 | 80.0 / 16.3 |
|  |  | Top-5/1 (%) | 92.5 / 79.2 | 92.5 / 31.8 |
|  |  | 1-dB marginal accuracy (%) | 90.4 / 59.0 | 90.4 / 32.8 |
|  | L1-RSRP diff (otion 3 with K = 1) | Avg. L1-RSRP difference in dB | 0.36 / 1.27 | 0.36 / 4.22 |
|  | System performance | RS overhead Reduction (%) | 87.5 | 87.5 |

Observation 7: The evaluation results for 8 beam based setB and 64 beam based setA without any measurement error are shown in Table 2.

### 2.3.2 Simulation Framework

RAN4 agreed to the following simulation framework during the last meeting with approach.

- Companies to generate ideal L1-RSRP dataset from the SLS assumption defined in TR 38.843 Table 6.3.1-1. - Use subset samples of the ideal dataset generated from the 1st step for training models. Approach a: - Filter UEs in the other subset samples of the ideal dataset (none overlap samples with the subset dataset in 2nd step) according to SNR defined in Table 4 with a range to derive the ideal dataset for inference. and to evaluate the performance metric without considering measurement errors. Companies are encouraged to report the range for UEs filtering. - Use the LLS simulation assumptions defined in Table 1-5 to generate L1-RSRP difference as the baseband errors. - Add the baseband errors generated from LLS into the ideal SLS dataset to derive the dataset with measurement errors. - If RF errors are modelled, then they should be added into the ideal SLS dataset in addition to baseband errors. - Use the dataset with measurement errors for inference and to evaluate the performance metric with considering measurement errors.

The ultimate goal of this simulation framework is to investigate the impact of measurement error on beam prediction metrics. Hence, above simulation framework suggests companies to “Use the dataset with measurement errors for inference and to evaluate the performance metric with considering measurement errors”. However, the consideration of measurement error can be in three forms.

- Case 1:

  - Training data: No error

  - Testing data: Includes measurement errors

  - Ground truth for training and testing: No error

- Case 2:

  - Training data: Includes measurement error

  - Testing data: Includes measurement error

  - Ground truth for training and testing: No error

- Case 3:

  - Training data: Includes measurement error

  - Testing data: Includes measurement error

  - Ground truth for training and testing: Includes measurement error

If testing data includes measurement error, UE should train its model based on a dataset that includes measurement error, too. Hence, Case 2 is clearly expected to perform better than case 1.

In a real FR2 AI-ML BM test, ground truth will be decided based on UE’s report. RAN4 already agreed to this concept in the context of ground truth for predicted RSRP in [14]

- Agreement - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded

Hence, it makes sense for RAN4 to consider case 3, i.e., ground truth for training and testing should include measurement error, as well.

Proposal 4: RAN4 discusses both of following approaches to investigate the impact of measurement error on AI-ML BM metric.

  - Case 1: Training data, testing data and ground truth for training and testing all include measurement error.

  - Case 2: Training and testing data include measurement error. Ground truth for training and testing data does not include measurement error.

### 2.3.2 Baseband Error Modelling

We have generated baseband error results at -3 dB with both AWGN and the TDL-C channel assumptions of [11]. We show the distribution of baseband error results in Figure [3] and [4].

Figure 3: Baseband absolute error distribution of SSB L1-RSRP at AWGN (-3 dB SNR, curve fitted with zero mean normal distribution and sigma = 0.6 dB)

Figure 4: Baseband absolute error distribution of SSB L1-RSRP (TDL-C, -3 dB SNR, curve fitted with normal distribution and mean = -0.12 dB and sigma = 0.85 dB)

Observation 8: At -3 dB SNR, the distribution of absolute baseband measurement error can be fitted with following Gaussian distributions:

- AWGN: Gaussian with zero mean and sigma = 0.6 dB

- TDL-CD: Gaussian with mean = -0.12 dB and sigma = 0.85 dB

The distribution of relative baseband measurement error is shown in Figure 5.

Figure 5: Baseband relative error distribution of SSB L1-RSRP at AWGN (both SSBs at -3 dB SNR, curve fitted with zero mean normal distribution and sigma = 0.85 dB)

Please note that, 0.85^2 ~= 0.6^2 + 0.6^2. In other words, the variance of baseband relative measurement error across two SSBs is approximately equal to twice the variance of baseband absolute measurement error. It is well known that the variance of the summation of two IID Gaussian random variables is the summation of the variances of the individual Gaussian random variable. Hence, this observation suggests that baseband absolute measurement error is an IID Gaussian random variable across SSBs. Baseband absolute measurement error’s independence across time samples is well.

Observation 9: The variance of the Gaussian distribution fitting the baseband relative error is approximately twice the variance of the Gaussian distribution fitting the baseband absolute error. This can only be possible if baseband absolute measurement error is independent across SSB samples.

Proposal 5: RAN4 models baseband absolute measurement error as an IID Gaussian random variable across SSBs and time samples with zero mean and SD standard deviation.

- Note: Value of SD may vary among companies.

### 2.3.3 RF Error Modelling

[13] mentioned the following regarding RF error.

The impact of RF errors should also be considered. ±4.5dB RF error is considered. Following options can be used to generate RF errors. - Option 1: Independent for each TX beam - Option 2: Same for each Tx beam - Option 3: Independent for each module/panel - Option 4: Same for each module/panel - Other options are not precluded - Companies are requested to provide details of other options. Following options about distributions can be used: - Option 1: Uniform distribution - Option 2: Gaussian distribution - Option 3: A fixed value - Other options are not precluded - Companies are requested to provide details of other options.

Existing L1-RSRP absolute and relative measurement accuracy requirements can shed light whether RF error is independent or correlated across different TX beams.

Table 3: FR2 relative L1-RSRP accuracy ranges in NR

| Accuracy |  | Conditions |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot | Io Note 1 range |  |  |  |
|  |  |  | Minimum Io |  |  | Maximum Io |
| dB | dB | dB | dBm / SCSSSB Note 2 |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  | SCSSSB = 120kHz | SCSSSB = 240kHz |  |  |
| 8.5 | 11.5 | ≥-3 | N/A |  | -70 | -50 |
| 6.5 | 9.5 | ≥-3 | Same value as SSB_RP in Table B.2.4.1-2, according to UE Power class, operating band and angle of arrival |  |  | -50 |
| NOTE 1: Io specified at the Reference point, and assumed to have constant EPRE across the bandwidth. NOTE 2: Values based on Refsens and EIS spherical coverage as defined in clauses 7.3.2 and 7.3.4 of TS 38.101-2 [19]. Applicable side condition selected depending on angle of arrival. NOTE 3: In the test cases, the SSB Ês/Iot and related parameters may need to be adjusted to ensure Ês/Iot at UE baseband is above the value defined in this table. |  |  |  |  |  |  |

RAN4 RRM spec defines relative L1-RSRP accuracy in the following way, “The relative accuracy of SSB based L1-RSRP is defined as the L1-RSRP measured from one SSB compared to the largest measured value of L1-RSRP among all SSBs of the serving cell.” Table 3 shows that RAN4 spec allows +-6.5 dB relative L1-RSRP accuracy range in FR2.

Figure 6: CDF of baseband relative L1-RSRP inaccuracy at -3 dB AWGN

Figure 6 shows that the 90 percentile baseband relative inaccuracy ranges within +-1.5 dB. Even if we assume 1-2 dB additional margin, baseband relative inaccuracy is less than the +- 6.5 dB L1-RSRP relative inaccuracy margin provided in RAN4 RRM spec. Hence, the remaining portion of relative inaccuracy range comes from RF error. Based on the +-6.5 dB total relative L1-RSRP error range mentioned in RAN4 spec, relative RF error can lie in +-3dB to +-4 dB range. However, the relative L1-RSRP error may vary across companies.

Observation 10: RAN4 spec allows +-6.5 dB relative L1-RSRP accuracy range in FR2. Internal simulations suggest that 90 percentile baseband relative inaccuracy ranges within +-1.5 dB.

Observation 11: Even if we assume 1-2 dB additional margin, baseband relative inaccuracy is less than the +- 6.5 dB L1-RSRP relative inaccuracy margin provided in RAN4 RRM spec. The remaining portion of relative inaccuracy range comes from RF error.

Unlike baseband error, we could not derive an exact distribution of RF error. Hence, we propose the absolute RF error of each SSB to be uniformly distributed within +-4.5 dB and the relative RF error of each SSB to lie within +-X dB within the absolute RF error of the strongest SSB.

RF error primarily stems from calibration error and it depends on several factors including UE’s gain state, angle of arrival, RX beam pattern, temperature, band, etc. For a particular UE, RF error hardly changes in the same band if TX power, path loss and angle of arrival don’t change.

Observation 12: Unlike baseband error, it is hard to derive an exact distribution of RF error. Hence, it is suitable to model RF error as a uniform random variable.

Proposal 6: RAN4 models RF error in the following way:

- Absolute RF error of strongest SSB lies uniformly in [-4.5, +4.5] dB range

- Assume that, in one instance, absolute RF error of strongest SSB is Y dB. Absolute RF error of all other SSBs (except strongest SSB) in that instances lie uniformly in [max(Y-X,-4.5 dB), min(Y+X, 4.5 dB)] range

  - Note: X is TBD.

- RF error does not change in time in the same band if TX power, path loss and angle of arrival don’t change.

# Conclusions

Observation 1: RAN1’s agreed beam prediction metric without or with margin simply translates to option 2 and option 3 of RAN4’s beam prediction metric.

Observation 2: Support for option 1 of beam prediction metric is still TBD in RAN1.

Observation 3: Option 2’s drawback lies in the fact that its pass/fail criteria don’t depend on whether the top predicted beams are adjacent to the strongest beams or not.

- If TE configures the setB and setA beams to be too close to each other and the actual measured RSRP of more than topK beams of setA lies within the range of UE’s measurement error, UE will not be able to pass option 2 focused test.

- Note: The maximum value of K depends the maximum number of beams that UE can report for measurement and inference. The max value of K is decided by RAN1/RAN2 and RAN4 cannot control it.

Observation 4: Option 3 does not suffer from the constraints of option 2 that got mentioned in observation 4. Even if more than topK beams of setA lies within the range of UE’s measurement error, UE will be able to pass option 3 tests as long as the measured RSRP of its predicted beam lies within X dB of the strongest beam.

- RAN4 can control the value of X dB and ensure that UE does not fail a test due to RAN4 allowed measurement error.

Observation 5: 3GPP is considering L1-RSRP accuracy as a potential prediction metric for AI-ML based beam management. UE has to know its RX beam to predict L1-RSRP accuracy of a particular gNB TX beam.

Observation 6: To compare between gNB beams of setA, UE needs to have some idea of the Rx gain associated with them. UE has to know the Rx beam of the predicted gNB beam even if it only has to predict top-1 (%) or top-K (%) beams.

Observation 7: The evaluation results for 8 beam based setB and 64 beam based setA without any measurement error are shown in Table 2.

Table 2 Evaluation results for BM-Case1 (Use case 1) without model generalization for DL Tx beam prediction at UE side (Results for Set B, Set B subset of Set A)

| Qualcomm |  |  |  |  |
| --- | --- | --- | --- | --- |
| Assumptions |  | Number of beams [beams/beam pairs] in Set A | 192 | 192 |
|  |  | Number of beams [beams/beam pairs] in Set B | 24 | 24 |
|  |  | Baseline scheme | Empirical PMF-based approach | linear interpolation-based approach |
| AI/ML model input/output |  | Model input | RSRPs of Set B beams | RSRPs of Set B beams |
|  |  | Model output | Best Tx beam ID | Best Tx beam ID |
| Data size |  | Training | 4000 | 4000 |
|  |  | Testing | 600 | 600 |
| AI/ML model |  | Model description | FC layer based NN | FC layer based NN |
|  |  | Model complexity | 4.6K parameters | 4.6K parameters |
|  |  | Computational complexity | 9.8K FLOPs | 9.8K FLOPs |
| Evaluation results [with AI/ML/baseline (Baseline-2)] | Beam prediction accuracy (%) | Top-1 (%) | 63.5 / 28.3 | 63.5 / 10.7 |
|  |  | Top-2/1 (%) | 80.0 / 46.1 | 80.0 / 16.3 |
|  |  | Top-5/1 (%) | 92.5 / 79.2 | 92.5 / 31.8 |
|  |  | 1-dB marginal accuracy (%) | 90.4 / 59.0 | 90.4 / 32.8 |
|  | L1-RSRP diff (otion 3 with K = 1) | Avg. L1-RSRP difference in dB | 0.36 / 1.27 | 0.36 / 4.22 |
|  | System performance | RS overhead Reduction (%) | 87.5 | 87.5 |

Observation 8: At -3 dB SNR, the distribution of absolute baseband measurement error can be fitted with following Gaussian distributions:

- AWGN: Gaussian with zero mean and sigma = 0.6 dB

- TDL-CD: Gaussian with mean = -0.12 dB and sigma = 0.85 dB

Observation 9: The variance of the Gaussian distribution fitting the baseband relative error is approximately twice the variance of the Gaussian distribution fitting the baseband absolute error. This can only be possible if baseband absolute measurement error is independent across SSB samples.

Observation 10: RAN4 spec allows +-6.5 dB relative L1-RSRP accuracy range in FR2. Internal simulations suggest that 90 percentile baseband relative inaccuracy ranges within +-1.5 dB.

Observation 11: Even if we assume 1-2 dB additional margin, baseband relative inaccuracy is less than the +- 6.5 dB L1-RSRP relative inaccuracy margin provided in RAN4 RRM spec. The remaining portion of relative inaccuracy range comes from RF error.

Observation 12: Unlike baseband error, it is hard to derive an exact distribution of RF error. Hence, it is suitable to model RF error as a uniform random variable.

Proposal 1: RAN4 defines AI-ML BM requirement for option 3.

Proposal 2: RAN4 waits for simulation results and RAN1 progress before defining AI-ML BM requirement for option 2 and option 1 respectively.

Proposal 3: UE knows the RX beam paired with the predicted Tx beam from setA.

- FFS: TCI state of predicted TX beam.

Proposal 4: RAN4 discusses both of following approaches to investigate the impact of measurement error on AI-ML BM metric.

  - Case 1: Training data, testing data and ground truth for training and testing all include measurement error.

  - Case 2: Training and testing data include measurement error. Ground truth for training and testing data does not include measurement error.

Proposal 5: RAN4 models baseband absolute measurement error as an IID Gaussian random variable across SSBs and time samples with zero mean and SD standard deviation.

- Note: SD may vary among companies.

Proposal 6: RAN4 models RF error in the following way:

- Absolute RF error of strongest SSB lies uniformly in [-4.5, +4.5] dB range

- Assume that, in one instance, absolute RF error of strongest SSB is Y dB. Absolute RF error of all other SSBs (except strongest SSB) in that instances lie uniformly in [max(Y-X,-4.5 dB), min(Y+X, 4.5 dB)] range

  - Note: X is TBD.

- RF error does not change in time in the same band if TX power, path loss and angle of arrival don’t change

# References

- R4-2414446, ‘AI/ML ad-hoc meeting minutes’, Moderator (Qualcomm)

- R4-2414323, ‘WF on AI/ML’, Qualcomm

- 3GPP TR 38.843 V18.0.0, Technical Report (TR): Study on artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.

- RP-242399, “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, 3GPP TSG RAN #105.

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #117.

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 #118.

- 38.133, Requirements for support of radio resource management, V 17.11.0.

- R4-2417212, WF on Requirements for AI/ML air interface, 3GPP TSG-RAN WG4 #112bis.

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 #118bis.

- R1-2308259, “Evaluations on AI/ML for Beam Management”, Qualcomm Incorporated, 3GPP TSG-RAN WG1 #114.

- R4-2417211, “Simulation assumptions for beam prediction”, 3GPP TSG-RAN WG4 #112bis

- R4-2420513, WF on AI/ML air interface part 1, RAN4 #113.

- R4-2420480, Updated Simulation Assumptions for Beam Prediction, RAN4 #113.

- 3GPP TSG RAN4 Chairman’s Notes.