3GPP TSG-RAN WG4 Meeting # 114										       R4-2502584

Athens, GR, 17 – 21 Feb., 2025

Title: 	WF on RRM requirements for NR_Mob_enh2

Agenda Item:	5.1.2

Source: 	MediaTek Inc.

Document for:	Approval

# 0 Notes:

In this document,

- < Agreement > represents the decisions made by in this meeting

- < Way forward > represents the next step in later meetings

  - “FFS” does not mean RAN4 will make a down-selection for the item. More other options can be proposed.

# 1 Topic #1: LTM - Core part

## Sub-topic 1-1 Early TCI state activation

Issue 1-1-1: Clarify in the early TCI state activation delay requirement that the activation applies for both UL and DL TCI state

Discuss the following proposal in the CR directly.

- Proposals

  - Proposal 1 (Nokia):

    - Clarify in the early TCI state activation delay requirement that the activation applies for both UL and DL TCI state (joint or separate), without changing the agreed delay.

## Sub-topic 1-2 RAN1 LS on collision between SSB and RA occasion for LTM

Issue 1-2-1: whether/how to handle the collision between an SSB and a valid RO for LTM

Agreement on reply LS

- RACH shall be prioritized over SSB, if the collision is outside MG.

- If collision happens within MG, it is up to UE implementation, i.e., UE may transmit or drop RACH when the RACH occasion colliding with the MG occasion.

< Agreement >

- Further discuss the definition of RACH occasion colliding with the MG occasion.

## Sub-topic 1-3 PDCCH-order PRACH to a candidate cell

Issue 1-3-1: PDCCH-order PRACH delay

Table 8.6.2-1: BWP switch delay

|  | NR Slot length | BWP switch delay TBWPswitchDelay (slots) |  |
| --- | --- | --- | --- |
|  | (ms) | Type 1Note 1 | Type 2Note 1 |
| 0 | 1 | 1 | 3 |
| 1 | 0.5 | 2 | 5 |
| 2 | 0.25 | 3 | 9 |
| 3 | 0.125 | 6 | 18 |
| 5 | 0.03125 | 20 | 65 |
| 6 | 0.015625 | 39 | 129 |
| NOTE 1: Depends on UE capability. NOTE 2: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the smaller SCS between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |  |

< Way forward > Further discuss the following proposals

  - Proposal 1 (Ericsson):

    - If PRACH is outside UL BWP of PCell, but withing UL BWP of any active SCell, for PRACH transmission using 1.25kHz or 5 kHz SCS, UE determines BWP switch delay assuming µ=0.

  - Proposal 2 (QC):

    - Update the value of T_SSB-proc defined for SSB processing in PDCCH-order PRACH delay requirement to 7 ms from 2 ms

## Sub-topic 1-4 L1-RSRP Measurement

Issue 1-4-1: L1-RSRP Measurement for joint ICBM/mTRP & LTM

< Agreement >

  - Clarify in TS 38.133 that RRM requirements are not applicable to the case when L1 measurement on non-serving cell and LTM L1 measurement are both configured simultaneously.

## Sub-topic 1-5 Cell switch delay requirements for Pcell/PSCell

#### 1.5.1 T/F fine tracking: Tfirst-RS and Tmargin

Issue 1-5-1-1: In FR1 for UE not supporting/configured with L1 measurement, conditions for Tfirst-RS =0 in cell switch delay

< Agreement >

- In FR1 for UE not supporting/configured with L1 measurement, Tfirst-RS =0 in cell switch delay on the condition that the time gap between UE activating the TCI state and receiving cell switch command is

  - no more than 480ms for intra-frequency

  - no more than 160 ms for inter-frequency

#### 1.5.2 Fast RRC decoding

Issue 1-5-2-1: Which cell(s) TLTM-RRC-processing = 0 apply to when the total number of candidates triggered early TCI activation or early PDCCH order RACH exceed UE capability?

< Agreement >

  - Not pursue the discussion on the further optimization on TLTM_RRC-processing =0 when the total number of candidates triggered early TCI activation or early PDCCH order RACH exceed UE capability.

Issue 1-5-2-2: Which cell(s) TLTM-RRC-processing = 0 apply to if the total number of candidates exceeds UE capability when neither early TCI state activation nor early PDCCH order RACH is triggered on any candidate?

< Way forward >

    - If the total number of pre-configured candidates exceeds UE capability when neither early TCI state activation nor early PDCCH order RACH is triggered on any candidate, TLTM-RRC-processing ≠ 0 for all the LTM candidates.

not an essential issue and the issue is closed.

#### 1.5.3 PL-RS

Issue 1-5-3-1: Clarification on agreed “Conditions of no extra time for PL-RS measurement in cell switch delay” in RAN4#112

Common understanding:

  - The following agreement is to make sure L3-RSRP or L1-RSRP on the SSB associated with PL-RS has been measured/reported recently before cell switch command.

  - The sub-bullet in the following agreement is to make sure “SSB associated to the target TCI state is the same as the SSB associated with PL-RS”.

RAN4#112 Issue 1-4-2-1: Conditions of no extra time for PL-RS measurement in cell switch delay <Agreement>: - No additional PL-RS measurement time is needed, provided L3-RSRP or L1-RSRP on the SSB associated with PL-RS has been measured/reported. - PL-RS is associated with TCI state indicated by LTM cell switch command in terms of QCL chain. - Further discuss the wording in the CR next meeting.

#### 1.5.4 Others

Issue 1-5-4-1: Applicable condition of cell switch delay for CFRA cell switch

<Agreement>

  - Make it clear: for CFRA cell switch, the requirements are only applicable to the case that the TCI state indicated in cell switch command is associated with the SSB for CFRA.

## 1.6 Sub-topic 1-6 UE feature

RAN4#113 < Agreement > For RAN4 39-3-6: - ‘SSB resources of serving cells and neighbouring cells’ includes both LTM candidate cell(s) and all SSBs from serving cells, including the serving cells that are not configured with LTM L1 measurements, i.e. SSB resources configured by CSI-MeasConfig are also counted.

Issue 1-6-1: RAN4 FG 39-3-1

| Index | Feature group | Components |
| --- | --- | --- |
| 39-3-1 | Number of frequency layers for L1-RSRP measurement | - The max number of frequency layers UE can measure for intra- and inter-frequency without measurement gaps L1-RSRP measurement 2. The max number of frequency layers UE can measure for inter-frequency L1-RSRP measurement with measurement gaps |

< Agreement >

  - The 1st component of RAN4 FG 39-3-1 does not include the frequency layers for which SSB L1-RSRP measurement is not configured on any neighbor cell on.

Issue 1-6-2: RAN4 FG 39-3-3

| Index | Feature group | Components |
| --- | --- | --- |
| 39-3-3 | Number of total cells to be measured | The max number of total cells of serving cells and neighboring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. |

< Agreement >

  - RAN4 FG 39-3-3 supportedMaxCellsWithoutGapsL1-Meas-r18, ‘total cells of serving cells and neighbouring cells’ counts both LTM candidate cell(s) and serving cells for SSB L1-RSRP measurement, no matter SSB L1-RSRP measurement on serving cell is configured in LTM-CSI-ResourceConfig-r18 or not.

Issue 1-6-3: RAN4 FG 39-3-5

| 39-3-5 | Number of SSB resources for L1-RSRP measurement per frequency layer | - The max number of SSB resources UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps 2. The max number of SSB resources UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps | Candidate value of Component 1: {1,2,3,4,5,6,7,8} Candidate value of Component 2: {1,2,3,4,5,6,7,8} |
| --- | --- | --- | --- |

< Agreement >

  - The first component of RAN4 FG 39-3-5 maxSSB-PerFreqLayerL1-Meas-r18 counts both LTM candidate cell(s) and serving cells for SSB L1-RSRP measurement, no matter SSB L1-RSRP measurement on serving cell is configured in LTM-CSI-ResourceConfig-r18 or not.

  - For RAN4 FG 39-3-5, include the candidate values of 12, 16, 20, 24 to existing candidate values.

    - FFS: include the candidate values of 32, 40, 48, 64.

Issue 1-6-4: RAN4 FG 39-3-6

| 39-3-6 | Number of total SSB resources to be measured | The max number of total SSB resources of serving cells and neighboring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. | PerBC | Candidate values: {2,4,8,12,16,32,64} Note: the value should be not smaller than UE capability of beamManagementSSB-CSI-RS (Component 2 of 2-24) |
| --- | --- | --- | --- | --- |

| 2-24 | SSB/CSI-RS for beam measurement | 1) The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP within a slot shall not exceed MB_1 2) The max number of CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP shall not exceed MC_1 3) The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP within a slot shall not exceed MB_2 4) Supported density of CSI-RS 5) The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-RSRP shall not exceed MD_1 | 2-21, 2-22 or 2-23, 2-23a | beamManagementSSB-CSI-RS { 1. maxNumberSSB-CSI-RS-ResourceOneTx 2. maxNumberCSI-RS-Resource 3. maxNumberCSI-RS-ResourceTwoTx 4. supportedCSI-RS-Density 5. maxNumberAperiodicCSI-RS-Resource } |
| --- | --- | --- | --- | --- |

< Way forward >

  - FFS Update the 39-3-6 FG candidate values description from Component 2 of 2-24 to component 1 of 2-24.

Issue 1-6-5: A new UE capability for early-PRACH based RACH-less LTM applications

The issue is closed.

Issue 1-6-6: LS to RAN2

< Agreement >

  - Send RAN2 LS capturing the agreements on RAN4 FG 39-3-1, 39-3-3, 39-3-5.

# 2 Topic #2: Improvement on SCell/SCG setup delay - Core part

Issue 2-1-1: Mis-alignment between RAN2 and RAN4 spec when measReselectionCarrierListNR is not configured

< Agreement >

  - Not to update RAN4 specification.

# 3. Topic #3: Performance Part

## 3.1 Sub-topic 3-1 LTM

Issue 3-1-1: Test case for PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 (A.6.3.2.4.2)

< Agreement >

  - In the test case for PDCCH-ordered RACH to an inter-frequency candidate cell in FR1 (A.6.3.2.4.2), add a limitation that PDCCH order is not expected on the frame with SFN mod 8 =3.

Issue 3-1-2: Test cases for SSB based L1-RSRP measurement in FR1 – intra-f & inter-f without gap (A.6.6.26.1 & A.6.6.28.1.3)

Issue is closed

- Proposals

  - Proposal 1 (MTK): In LTM L1-RSRP test cases in FR1 (A.6.6.26.1 & A.6.6.28.1.3), intra-f and inter-f without gap, the test requirements should be measurement delay + report delay considering the worst case, i.e., 80slots+80slots.

## 3.2 Sub-topic 3-2 Improvement on SCell/SCG setup delay

Issue 3-2-1: More test for Improvement on SCell/SCG setup delay

< Agreement >

  - Not to introduce an additional test for Idle mode fast CA/DC eEMR measurement to verify a scenario where validityDuration-r18 is configured in rel-18.