3GPP TSG-RAN WG4 Meeting # 114					       			               R4-2502585

Athens, GR, Feb. 17 - Feb. 21, 2025

Agenda item:			5.1.2

Source:	Huawei, HiSilicon

Title:	WF on RRM requirements for Netw_Energy_NR

Document for:	Approval

- Topic #1: Core requirements maintenance

Issue 1-1-1: Neighbour cells on carrier of SSB-less SCell

Agreement:

- From RAN4 understanding, the definition of intra-frequency/inter-frequency, whether MG is needed and which requirements apply depends on whether servingCellMO is present or not.

- Based on the agreements in RAN4#113 meeting, RAN4 provide analysis for following cases:

  - If the servingCellMO indication is configured:

    - Intra/inter: The neighbor cell measurement is defined as intra-frequency when centre frequency of the SSB of the neighbour cell is same as that of SSB frequency configured in servingCellMO; otherwise, it is defined as inter-frequency

    - Requirements:

Add clause numbers for corresponding requirements (i.e. intra/inter and with/without MG)

- Intra-frequency without measurement gap: Clause 9.2.5 TS 38.133

- Intra-frequency with measurement gap: Clause 9.2.6 TS 38.133

- Inter-frequency with measurement gap: Clause 9.3.4 TS 38.133

- Inter-frequency without measurement gap: Clause 9.3.9 TS 38.133

  - If the servingCellMO indication is not configured:

    - Intra/inter: Based current RAN4 framework on intra-f/inter-f definition, the neighbor cell measurement is defined as inter-frequency

Requirements: Add clause numbers for corresponding requirements (i.e. inter and with/without MG)

- Inter-frequency with measurement gap: Clause 9.3.4 TS 38.133

- Inter-frequency without measurement gap: Clause 9.3.9 TS 38.133

- Other information to RAN2:

  - Based on above information, it is up to RAN2 to decide whether servingCellMO can be present or not and detailed signalling design.

  - The potential conflict (if any) when servingCellMO is present is up to RAN2 to handle.

Note: servingCellMO is based on the current RAN2 specification, it does not preclude any RAN2 discussion or signalling design for the same purpose.

Issue 1-1-2: EPRE conditions updating

Agreement: Remove the [] on [30] for Y.