3GPP TSG-RAN WG4 Meeting #114							                                      	R4-2502602

Athens, GR, 17th-21st February 2025

Agenda item:			7.27.3

Source:	China Telecom

Title:	WF on RRM requirements for NR_Mob_Ph4_Part2

Document for:	Approval

Notes

In this document,

- < Agreement > represents the decisions made by in this meeting

- < Way forward > represents the next step in later meetings

  - “FFS” does not mean RAN4 will make a down-selection for the item. More other proposals can be proposed.

Topic #1: RRM discussion

General aspects on Conditional LTM

Issue 1-1-1: The applicable scenarios for conditional LTM

RAN4#113 < Agreement > ad hoc - FFS: RAN4 requirements of CLTM which is triggered by L1 measurement (e.g. event LTM3 and LTM5) does not apply to candidate cells on deactivated SCC, including both serving cell and neighbor cells. - Note: This can be revisited if RAN4 agreed to introduce measurement requirements for L1 measurement on deactivated SCC.

< Agreement >

  - For the case which CLTM is triggered by L3 measurement, deactivated SCell and the cells with same frequency as deactivated Scell are included.

  - For the case which CLTM is triggered by L1 measurement, FFS deactivated SCell and the cells with same frequency as deactivated Scell are included.

Issue 1-1-2: Measurement requirements on conditional LTM

RAN4#113 < Way forward > FFS the following proposal: - Support L3 measurement based conditional LTM for FR1 from RAN4 perspective. - Note: QC need further check. Continue to discuss in the next meeting. RAN2#128 1. The triggering condition of conditional LTM can be based on L3 measurement. 4. For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell. 6. The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

< Agreement >

  - For FR1, define the requirements for L3 measurement based conditional LTM.

  - For FR2, not define the requirements for L3 measurement based conditional LTM.

Early synchronization and TCI state activation

Issue 1-2-1: UE based TA measurement

RAN4#113 < Way forward > FFS the following proposals: - Define RRM requirements for UE based TA measurement. (CTC, CMCC, vivo, QC, CATT) - Do not define RRM requirements for UE based TA measurement. (Apple, MTK)

< Way forward > FFS the following options:

  - Option 1: Define requirements for L1 measurement on deactivated Scell.

  - Option 2: Define requirements for UE based TA measurement.

    - Proponent: The feature is essential for C-LTM. No cell switch command carrying TA for R19 C-LTM. For C-LTM, PRACH based on PDCCH order is very complicated.

    - Concern: Network cannot trust the TA value from UE. Difficult to define proper requirements. There is other command carrying TA.

  - Option 3: Not define requirement for any of them.

Issue 1-2-2: PDCCH ordered RACH

RAN4#113 < Agreement > - PDCCH ordered RACH is supported for conditional intra-CU LTM from RAN4 perspective. - Note: If no new PDCCH ordered RACH procedure will be introduced in RAN2, the related R18 RAN4 requirements are applicable.

< Agreement >

  - No need to change existing PDCCH ordered RACH delay requirements based on current RAN2 agreements, but RAN4 to monitor and take any new RAN1/2 agreements into account, if needed.

Issue 1-2-3: Early TCI state activation

RAN4#113 < Agreement > - Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM from RAN4 perspective. - Note: If no new procedure will be introduced in RAN2, the related R18 RAN4 requirements are applicable.

< Agreement >

- No need to change the existing TCI state activation delay requirements based on the current RAN2 agreements, but RAN4 to monitor and take any new RAN1/2 agreements into account, if needed.

Conditional LTM delay

Issue 1-3-1: General aspects of Conditional LTM delay

< Way forward > FFS the following proposals:

- Proposal 1: CHO requirements framework can be used as baseline for Conditional LTM.

  - Proposal 1a: Update Tinterrupt according to TLTM-interrupt defined in R18 LTM cell switch delay.

- Proposal 2: Before RAN4 reaches a conclusion on whether to define the delay requirements for conditional LTM and subsequent conditional LTM together, RAN4 should prioritize discussing the delay requirements for conditional LTM here.

- Proposal 3: RAN4 specifies the 1st time CLTM and subsequent CLTM delay separately.

- Proposal 4: Support performing T/F fine tracking (TΔ) if needed at first and then Tprocessing, to reduce the interruption time during conditional LTM.

Issue 1-3-2: The starting point of conditional LTM delay

< Agreement > ad hoc

  - The starting point of conditional LTM is the end of the last TTI containing the RRC command for conditional LTM, [under assumption that UE will start evaluating the CLTM event after reception of the RRC for CLTM].

  - Revisit if RAN2 has more progress.

Issue 1-3-3: The components of conditional LTM delay

< Agreement > FFS the following options ad hoc

- Option 1: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt + TLTM-RRC-processing + Tfirst-RS + TRS-proc

  - TCLTM_interrupt = TLTM-processing + TLTM-IU ms

- Option 2: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt + Tfirst-RS + TRS-proc

  - TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU ms

- Option 3: DCLTM = [TRRC] + TEvent_DU + Tmeasure + TCLTM_interrupt

  - TCLTM_interrupt = TLTM-RRC-processing + TLTM-processing + TLTM-IU + Tfirst-RS + TRS-proc ms

Issue 1-3-4: TRRC

< Way forward > FFS the following proposals:

- Proposal 1: TRRC is the RRC procedure delay.

- Proposal 2: Too early to agree whether TRRC is needed.

Issue 1-3-5: TEvent_DU

< Way forward > FFS the following proposals:

- Proposal 1: TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional LTM command until a condition exists at the measurement reference point which will trigger the conditional intra-CU LTM.

    - Proposal 1a: It can cover both cases:

- UE first completes early TCI state activation and/or PDCCH order PRACH and then performs execution condition evaluation

- Neither early TCI state activation nor PDCCH order PRACH is successfully performed before condition is met.

- Proposal 2: If certain process can guarantee that UE first completes DL sync and UL sync and then performs execution condition evaluation, the delay for DL sync and UL sync can be contained in TEvent_DU.

- Proposal 3: Wait for more RAN2 agreements.

Issue 1-3-6: Tmeasure

< Way forward > FFS the following proposals:

- Proposal 1: Tmeasure is measurements time. It is defined from the end of TEvent_DU until UE executes LTM to a target cell and interruption time starts.

    - Proposal 1a:

  - If SSB based L1-RSRP measurement is used in the event, Tmeasure is SSB based L1-RSRP measurement period in R18 LTM.

  - If SSB based L3-RSRP measurement is used in the event, Tmeasure is SSB based L3-RSRP measurement period in legacy.

  - If CSI-RS based L1-RSRP measurement is used in the event, Tmeasure is CSI-RS based L1-RSRP measurement period which is to introduced in R19.

  - FFS on CSI-RS based L3-RSRP measurement.

    - Proposal 1b:

      - When the condition is L1-based: Use the Rel-18 L1 measurement period for serving cell and candidate cells as the starting point to define Tmeasure.

      - When the condition is L3-based: As the starting point, Tmeasure to follow the definition of Tmeasure for CHO.

Issue 1-3-7: TCLTM_execution / TLTM_execution

< Way forward > FFS the following proposals:

- Proposal 1: TCLTM_execution is the UE execution preparation time for conditional intra-CU LTM and starts after UE realizes the condition of LTM is met and identity of the target cell is determined.

    - Proposal 1a: TLTM_execution can be up to 10ms.

- Proposal 2: TLTM_RRC-processing covers the delay related to processing of the LTM candidate cell configuration. There is no need to add an additional execution/preparation delay to the cell switch delay.

Issue 1-3-8: Tinterrupt

< Way forward > FFS the following proposals:

- Proposal 1: Tinterrupt is the time between when the UE starts to execute the conditional intra-CU LTM to the target cell and the time the UE starts to transmit the first UL transmission on the target cell.

- Proposal 2: The term Tinterrupt  may induce ambiguity when define conditional LTM delay, as interruptions may also happen in early PDCCH RACH procedure.

- Proposal 3: Wait for more RAN2 agreements.

Issue 1-3-9: The components of Tinterrupt

< Way forward > FFS the following proposals:

  - Proposal 1: Tinterrupt = TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU.

- TLTM-processing is the time for UE processing, consisting of applying the target cell parameters and L1/L2 change.

- Tfirst-RS is the time for fine time tracking and acquiring full timing information of the target cell.

- TRS-proc is the time for SSB processing.

- TLTM-IU is the interruption uncertainty during LTM cell switch.

  - Proposal 2: Excluding TLTM-RRC-processing, other components defined in the interruption time for Rel-18 LTM Cell Switch delay can be a baseline, including TLTM-processing, Tfirst-RS, TRS-proc and TLTM-IU.

  - Proposal 3: ASN.1 decoding and validity check and fine DL synchronization (first SSB) is not part of Tinterrupt but may be part of DCLTM if not done before the starting point of the cell switch delay.

Subsequent Conditional LTM

Issue 1-4-1: General aspects of Subsequent Conditional LTM delay

RAN4#112bis < Agreement> - RAN4 to define cell switch delay requirements for conditional LTM - RAN4 to define subsequent conditional LTM requirement unless there is any technical issue identified. - Wait for other WGs input to further discuss detailed RRM requirements.

< Way forward > FFS the following proposals:

- Proposal 1: Subsequent conditional LTM cell switch delay can be discussed based on the conclusion of conditional LTM delay.

- Proposal 2: Wait for more RAN2 agreements before defining the requirements for subsequent CLTM.

Issue 1-4-2: The starting point of Subsequent conditional LTM delay

< Agreement > ad hoc

  - The starting point of the subsequent conditional LTM delay is when UE transmits RRCReconfigurationcomplete message for the previous conditional LTM, [under assumption that UE will continue evaluating the CLTM event after the completion of previous CLTM].

  - Revisit if RAN2 has more progress.

Issue 1-4-3: The components of subsequent conditional LTM delay

< Way forward > FFS the following proposals:

- Proposal 1: DLTM_Conditional_subsequent = TEvent_DU + Tmeasure + TLTM_execution + TLTM-interrupt.

- TEvent_DU is the delay uncertainty which is the time from when the UE successfully transmits an RRCReconfigurationcomplete message for the previous cell switch until a condition exists at the measurement reference point which will trigger the conditional LTM.

- Tmeasure is measurements time.

- TLTM_execution is the UE execution preparation time for conditional LTM, and starts after UE realizes the condition of conditional LTM is met and identity of the target cell is determined. TLTM_execution can be up to 10ms.

- TLTM-interrupt is the interruption time as specified in Rel-18.

- Proposal 2: DCLTM_sub = TEvent_DU + Tmeasure + TCLTM_execution + Tinterrupt +TEarly_process.