3GPP TSG-RAN WG4 Meeting #114	R4-2502850

# Athens, Greece, February 17th – 21st, 2025

Title: 	WF on NR and IoT NTN bands

Agenda Item:	6.18, 6.19, 6.20

Source: 	Moderator (Aalyria)

Document for:	Approval

# Topic #1: NR-NTN S-band

### Sub-topic 1-1: General Requirements and Coexistence Aspects

#### Issue 1-1-1: UE to UE Coexistence with band n2/n25

Agreement:

- Agree to two different additional spurious emission requirements, each associated to an NS value, and corresponding A-MPR, for protection of band b25/n25/b2/n2

  - -30 dBm/MHz (NS_XX)

  - -40 dBm/MHz (NS_YY)

- For the above requirements, the following statement will be captured in a normative way:

  - “The requirement applies based on coordination between operators and subject to regional/national regulation”

    - Apply the requirements to UE based on NS signal

- Agree to specify asymmetric channel bandwidths with 5, 10 and 15 MHz UL CHBW for band n252, with BCS0 including all permutation based on UL channel bandwidth

Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n252 | 5 | 10,15, 20 | 0 |
|  | 10 | 15, 20 |  |
|  | 15 | 20 |  |

#### Issue 1-1-2: UE NS For Additional Spurious emissions

6.5.3.3.X	Requirement for network signalling value "NS_XX"

When "NS_XX" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.2-X. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.2-X: Additional requirements for "NS_XX"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| 1930 ≤ f <1995 | -30 | 1MHz | XX |

Agreement:

- the note will be moved to NS mapping table, including the contents below

  - “The requirement applies based on coordination between operators and subject to regional/national regulation”

  - Apply the requirements to UE based on NS signal

6.5.3.3.Y	Requirement for network signalling value "NS_YY"

When "NS_YY" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.2-Y. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.2-Y: Additional requirements for "NS_YY"

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| 1930 ≤ f < 1995 | -40 | 1MHz | YY |

Agreement:

- the note will be moved to NS mapping table, including the contents below

  - “The requirement applies based on coordination between operators and subject to regional/national regulation”

  - Apply the requirements to UE based on NS signal

#### Issue 1-1-3: UE A-MPR for corresponding NS values

- Agreement:

  - Specify different A-MPR values for each NS depending on the UL CHBW, RB start and LcRB.

    - FFS on exact values

  - Adopt the following table skeleton

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR satellite Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_XX | 6.5.3.3.X | n252 | 5, 10, 15, 20 |  | Clause 6.2.3.X |
| NS_YY | 6.5.3.3.Y | n252 | 5, 10, 15, 20 |  | Clause 6.2.3.Y |
| NOTE 4: The NS is signalled based on coordination between operators and subject to regional/national regulation |  |  |  |  |  |

6.2.3.2	A-MPR for NS_XX

Table 6.2.3.X-X: A-MPR regions for NS_XX

| UL Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 5MHz |  |  |  |  |
| 10MHz |  |  |  |  |
| 15MHz |  |  |  |  |
| 20MHz |  |  |  |  |

Table 6.2.3.X-X: A-MPR for NS_XX

|  | Modulation | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK |  |  |  |  |  |  |
|  | QPSK |  |  |  |  |  |  |
|  | 16QAM |  |  |  |  |  |  |
|  | 64QAM |  |  |  |  |  |  |
|  | 256QAM |  |  |  |  |  |  |
| CP-OFDM | QPSK |  |  |  |  |  |  |
|  | 16QAM |  |  |  |  |  |  |
|  | 64QAM |  |  |  |  |  |  |
|  | 256QAM |  |  |  |  |  |  |

#### Issue 1-1-4: Protection of aeronautical radionavigation-satellite service (i.e. GNSS)

- Agreement:

  - Specify the following additional spurious requirements for protection of the aeronautical radionavigation satellite service:

Additional spurious emission requirements for band n252

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |
| 1559≤ f < 1610 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1559 ≤ f < 1610 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

### Sub-topic 1-2: UE RF

Sub-topic description: General UE RF Requirements

#### Issue 1-2-2: UE Asymmetric Channel Bandwidths

- Agreement:

  - Specify Asymmetric channel bandwidths as follows (see Draft CR)

Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n252 | 5 | 10,15, 20 | 0 |
|  | 10 | 15, 20 |  |
|  | 15 | 20 |  |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. NOTE 2: As indicated in TS38.306 [11], it is mandatory for UEs to support asymmetric channel BCS0 if there is an asymmetric BCS0 defined for the band. |  |  |  |

#### Issue 1-2-3: UE REFSENS

- Agreement:

  - Specify n252 UE REFSENS based on

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
|  | 15 | [-99.5] | [-96.3] | [-94.5] | [-93.3] |
| n252 | 30 |  | [-96.6] | [-94.6] | [-93.5] |
|  | 60 |  | [-97.0] | [-94.9] | [-93.7] |

#### Issue 1-2-4: UE Out-of-band Blocking

- Agreement:

  - Further discuss the Out-of-Band Blocking requirements for band n252 based on the options following

    - Option 1, with no co-banding assumption with n256 and n65

Table 2.2-1: Out of-band blocking for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n252 | Finterferer (CW) | MHz | -70 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -95 < f – FDL_low ≤ -70 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 95 or FDL_high + 85 ≤ f ≤ 12750 |
| n2542 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations NOTE 2: Band n254 power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. NOTE 3: void NOTE 4: void |  |  |  |  |  |

  - Option 2 with assumption of n252 co-banding with n256 and n65

Table 1: Out of-band blocking for n252

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n2521 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 and n252 lower frequency ranges are modified to enable specific implementations |  |  |  |  |  |

  - Option3: reuse the OOB requirements of band B23

# Topic #2 IoT-NTN S-band

### Sub-topic 2-1: UE RF

Sub-topic description: UE RF requirements

#### Issue 2-1-1: UE TX-RX Separation

- Agreement:

    - Specify B252 variable TX-RX frequency separation as follows for Cat M1

Table 5.4A.3-1: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 252 | 80 MHz1 161.4 to 198.6 MHz2 |
| NOTE 1: Default TX-RX separation. NOTE 2: The verification of flexible TX-RX frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3A. |  |

Table 7.3A-4: TX – RX carrier centre frequency separation for REFSENS verification

| E-UTRA Operating Band | Channel bandwidth | TX – RX carrier centre frequency separation for REFSENS verification |
| --- | --- | --- |
| 252 | 1.4 MHz | 161.4 MHz, 198.6 MHz |

    - Specify B252 variable TX-RX frequency separation as follows for Cat NB1 and NB2

Table 5.4B.3-1: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 252 | 80 MHz1 160.2 to -199.8 MHz2 |
| NOTE 1: Default Tx-Rx separation. NOTE 2: The verification of flexible TX-RX frequency separation within this range is limited to reference sensitivity. Further details are specified in clause 7.3B. |  |

Table 7.3B-2: TX – RX carrier centre frequency separation for REFSENS verification

| E-UTRA Operating Band | Channel bandwidth | TX – RX carrier centre frequency separation for REFSENS verification |
| --- | --- | --- |
| 252 | 0.2 MHz | 160.2 MHz, 199.8 MHz |

#### Issue 2-1-2: UE Out of band Blocking for Cat M1

- Agreement

  - Aligned with the assumptions agreed for NR.

#### Issue 2-1-3: UE Out of band Blocking for Cat NB1/NB2

- Agreement:

  - Aligned with the assumptions agreed for NR.

### Sub-topic 2-3: TR 36.764 Update

#### Issue 2-3-1: TR Section Skeleton

- Agreement:

  - Update TR 36.764 adding the following section

# 8 FDD band B252 (IoT-NTN FDD S_band) ## 8.1 Regulation ### 8.1.1 Regional Applicability ## 8.2 UE requirements ### 8.2.1 Channel arrangement ### 8.2.2 UE transmitter characteristics #### 8.2.2.1 Maximum output power #### 8.2.2.2 Emission requirements and NS values ##### 8.2.2.2.1 Spurious emission ### 8.2.3 UE receiver characteristics #### 8.2.3.1 Reference sensitivity #### 8.2.3.2 Blocking requirements ### 8.2.4 A-MPR Evaluation ## 8.3 BS requirements

# Topic #3: NR-NTN L-bands

### Sub-topic 3-1: General Aspects and Regulatory Input

Sub-topic description: Regulatory aspects

#### Issue 3-1-1: Capturing of ETSI EN Requirements

- Agreement:

  - Capture existing in-band and out-of-band emission requirements for L-bands n251, n250 and n253 as defined by ETSI EN 301 681 in a new table under a new sub-clause under Clause 6.5.2 (Conducted transmitter characteristics, Out-of-Band emissions) as Additional Regional Emission Requirements.

    - Introduce a NOTE specifying that the requirements are converted from radiated requirements (i.e. EIRP) in ETSI EN 301 681 to conducted requirements, assuming a 0dBi antenna with flat frequency response integral to the UE.

      - Detailed reference with specific version of EN 301 681 spec being referenced shall be included.

    - FFS for UE with high gain antenna or UE with non-integral antenna.

    - Come back in next meeting with specific NS values to be specified

#### Issue 3-1-2: Mechanisms for Protection of Radioastronomy

- Agreement:

  - For n251, n250, n253 protection of Radio Astronomy, consider the following two mechanisms as a baseline:

    - Network-controlled RB scheduling is considered as a baseline based on operator’s control

    - FFS whether a new NS value should be specified to signal the additional emission limit at the edge of the band, to signal cells in proximity or overlap to RA stations

      - Clarify the specific emission limits required and the frequency range applicability, and whether a specific value of geographical distance for the limit is considered, when defining proximity to a RA station

### Sub-topic 3-2: UE RF

Sub-topic description: System parameters and UE RF aspects

#### Issue 3-2-1: UE Out-of-band blocking

  - Agreement:

    - Consider the following out-of-band blocking requirements for band n253, n251 and n250 as a starting point for further verification (come back in next meeting):

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n251, n250 | Finterferer (CW) | MHz | [-60 < f – FDL_low < -15 or 15 < f – FDL_high < 60] | [-85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85] | [1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750] |
| n253 | Finterferer (CW) | MHz | [-60 < f – FDL_low < -15 or 15 < f – FDL_high < 94] | [-85 < f – FDL_low ≤ -60 or 94 ≤ f – FDL_high < 119] | [1 ≤ f ≤ FDL_low – 85 or FDL_high + 119 ≤ f ≤ 12750] |