3GPP TSG-RAN WG4 Meeting #114bis							         		R4-2503169

# Wuhan, Hubei, China, 07th – 11th April, 2025

Title: 	Simplification of HPUE 2UL DeltaMSD

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.1.1.1.1	General aspects (incl. duty cycle solutions, MSD rules) [NR_ENDC_RF_Ph4-Core]

Document for:	Approval

# 1	Background

In RAN#114, together with other contributors [2], we discussed in [1] the different MSD cases for HPUE inter-band combinations for 2UL IMDs and made some early proposals for simplification of the existing Rel-17/18/19 cases with DeltaMSD. Separately, another contribution captured in way forward [3] listed all the potential 2DL 2/3Tx HPUE combinations including cases where one band uses NR-U. In this contribution, we provide our analysis for all 2UL HPUE cases (and sub-cases) including those with NRU in at least one band. Then further simplification compared to [1] is proposed for DeltaMSD for two-band and three-band DL combinations but excluding cases with NR-U used in at least one band for Release-19.

# Discussion

## 2.1 Detailed description of the cases and high-level assumptions

To analyze the different cases, it is essential to get the details of how the interference level increases and how it compares with the PA linearity (e.g., is the linearity staying the same or increasing with the interference level and is it the same for each UL band or not).

Table 1 provides the details of the HPUE cases when NR-U is ignored with each PA in each band and the associated 2UL power:

- The first two columns provide a case number and which release introduced such combinations.

- The next two columns provide the inter-band 2UL power class and the associated total number of Tx.

- Then, parameters are provided per each UL band that are essential for the analysis of the IMD increases, as follows:

  - The power class per band

  - The number of Tx per band

  - The 1UL max power per band

  - The 2UL power per band (sum is limited to inter-band power class)

  - Associated PA back-off per band in 2UL mode.

- Finally, the last two columns provide the potential maximum power with both bands at their maximum and whether this opens for the increased power option. Note that increased power above PC1.5 may need further discussion, but this is not in the scope of this contribution.

Table 1: Detailed description of the different inter-band HPUE 2UL combinations without NR-U

| case | Rel | ULCA PC | #Tx | FDD/TDD Types (R19 only) | /Bd PC |  | /Bd #Tx |  | /Bd 1UL Pwr |  | /Bd 2UL power |  | /Bd 2UL BO |  | Max Tot Pwr | Inc Pwr option |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Bd1 | Bd2 | Bd1 | Bd2 | Bd1 | Bd2 | Bd1 | Bd2 | Bd1 | Bd2 |  |  |
| 1 | R15 | PC3 | 2 | F+T, T+T, F+F | PC3 | PC3 | 1 | 1 | 23 | 23 | 20 | 20 | -3 | -3 | 26 | N(+3dB) |
| 2 | R17 | PC2 | 2 |  | PC3 | PC3 | 1 | 1 | 23 | 23 | 23 | 23 | 0 | 0 | 26 | N |
| 3 | R17 | PC2 | 2 | F+T, T+T, F+F | PC3 | PC2 | 1 | 1 | 23 | 26 | 23 | 23 | 0 | -3 | 27.8 | Y(+1.8dB) |
| 4 | R18 | PC2 | 3 |  | PC3 | PC2 | 1 | 2 | 23 | 2x23 | 23 | 2x20 | 0 | -3 | 27.8 | Y(+1.8dB) |
| 5 | R18 | PC2 | 2 | F+T, T+T, F+F | PC2 | PC2 | 1 | 1 | 26 | 26 | 23 | 23 | -3 | -3 | 29 | N(+3dB) |
| 6 | R19 | PC2 | 3 |  | PC2 | PC2 | 1 | 2 | 26 | 2x23 | 23 | 2x20 | -3 | -3 | 29 | N(+3dB) |
| 7 | R19 | PC1.5 | 2 |  | PC2 | PC2 | 1 | 1 | 26 | 26 | 26 | 26 | 0 | 0 | 29 | N |
| 8 | R19 | PC1.5 | 3 |  | PC2 | PC2 | 1 | 2 | 26 | 2x23 | 26 | 2x23 | 0 | 0 | 29 | N |
| 9 | R18 | PC1.5 | 3 | F+T, T+T | PC3 | PC1.5 | 1 | 2 | 23 | 2x26 | 23 | 2x24.7 | 0 | -1.3 | 30 | Y(+1dB) |
| 10 | R19 | PC1.5 | 3 |  | PC2 | PC1.5 | 1 | 2 | 26 | 2x26 | 26 | 2x23 | 0 | -3 | 30.8 | Y(+1.8dB) |

Observation: One first observation is that the cases in yellow (including the baseline PC3 case) would allow the implementation of the next power class of inter-band HPUE provided no other UE limitation applies:

- PC3 case 1 implementation can support PC2 case 2.

- PC2 case 5 can support PC1.5 case 7.

- PC2 case 6 can support PC1.5 case 8.

- There is one case where one of the PA back-off is different from 0 or 3dB.

- This may be a later option for simplification to merge case 5/6 with 7/8 respectively.

Until the last meeting, the potential inter-band HPUE cases discussed were ignoring that one band or both could be an NR-U band which can be implemented with 1TX PC5, 1TX PC3 or 2Tx PC3. In way forward [3] however, several inter-bands NRCA cases using NR-U were listed in the following two tables for inter-band UL CA with 2Tx or 3Tx PC2 and PC1.5 respectively:

| 1st band | 2nd band | 2Tx or 3Tx |
| --- | --- | --- |
| PC3 TDD 1Tx | PC2 FDD 1Tx | 2Tx |
| PC3 TDD 1Tx | PC2 FDD 2Tx | 3Tx |
| PC3 FDD 1Tx | PC2 FDD 1Tx | 2Tx |
| PC3 FDD 1Tx | PC2 FDD 2Tx | 3Tx |
| PC5 TDD 1Tx | PC2 TDD 1Tx | 2Tx |
| PC5 TDD 1Tx | PC2 TDD 2Tx | 3Tx |
| PC5 TDD 1Tx | PC2 FDD 1Tx | 2Tx |
| PC5 TDD 1Tx | PC2 TDD 2Tx | 3Tx |

| 1st band | 2nd band | 2Tx or 3Tx |
| --- | --- | --- |
| PC2 TDD 1Tx | PC2 TDD 1Tx | 2Tx |
| PC2 TDD 1Tx | PC2 TDD 2Tx | 3Tx |
| PC3 TDD 1Tx | PC1.5 TDD 2Tx | 3Tx |
| PC2 TDD 1Tx | PC1.5 TDD 2Tx | 3Tx |
| PC2 TDD 1Tx | PC2 FDD 1Tx | 2Tx |
| PC2 TDD 1Tx | PC2 FDD 2Tx | 3Tx |
| PC2 FDD 1Tx | PC2 TDD 2Tx | 3Tx |
| PC2 FDD 1Tx | PC2 FDD 1Tx | 2Tx |
| PC2 FDD 1Tx | PC2 FDD 2Tx | 3Tx |
| PC2 FDD 1Tx | PC1.5 TDD 2Tx | 3Tx |
| PC5 TDD 1Tx | PC1.5 TDD 2Tx | 3Tx |

It should be noted that these tables still ignore the NR-U PC3 2Tx case, some PC3 baseline cases, and some PC3+PC3 cases.

Table 2 provides the details of the HPUE cases specific to NR-U with each PA in each band and the associated 2UL power. The table is constructed in the same way as Table 1 where band 2 is the NR-U band.

| case | Rel | UL CA PC | #Tx | FDD/TDD/NR-U Types (R19 only) | /Bd PC Bd2 NR-U |  | /Bd #Tx Bd2 NR-U |  | /Bd 1UL Pwr |  | /Bd 2UL power |  | /Bd 2UL BO |  | Max Tot Pwr | Inc Pwr option |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Bd1 | Bd2 | Bd1 | Bd2 | Bd1 | Bd2 | Bd1 | Bd2 | Bd1 | Bd2 |  |  |
| A | R17 | PC3 | 2 | U+U | PC5 | PC5 | 1 | 1 | 20 | 20 | 20 | 20 | 0 | 0 | 23 | N |
| B | ? | PC3 | 2 | U+U | PC3 | PC5 | 1 | 1 | 23 | 20 | 20 | 20 | -3 | 0 | 24.8 | Y(+1.8dB) |
| C | ? | PC3 | 3 | U+U | PC3 | PC5 | 2 | 1 | 2X20 | 20 | 2x17 | 20 | -3 | 0 | 24.8 | Y(+1.8dB) |
| D | ? | PC3 | 2 | U+U | PC3 | PC3 | 1 | 1 | 23 | 23 | 20 | 20 | -3 | -3 | 26 | N(+3dB) |
| E | ? | PC3 | 3 | U+U | PC3 | PC3 | 2 | 1 | 2X20 | 23 | 2x17 | 20 | -3 | -3 | 26 | N(+3dB) |
| D | ? | PC2 | 2 | U+U | PC3 | PC3 | 1 | 1 | 23 | 23 | 23 | 23 | 0 | 0 | 26 | N |
| F | ? | PC2 | 3 | U+U | PC3 | PC3 | 2 | 1 | 2X20 | 23 | 2x20 | 23 | 0 | 0 | 26 | N |
| 1 | R17 | PC3 | 2 | TorF+U | PC3 | PC5 | 1 | 1 | 23 | 20 | 20 | 20 | -3 | 0 | 24.8 | Y(+1.8dB) |
| 2 | ? | PC3 | 2 | TorF+U | PC3 | PC3 | 1 | 1 | 23 | 23 | 20 | 20 | -3 | -3 | 26 | N(+3dB) |
| 3 | ? | PC3 | 3 | TorF+U | PC3 | PC3 | 1 | 2 | 23 | 2x20 | 20 | 2x17 | -3 | -3 | 26 | N(+3dB) |
| 4 | ? | PC2 | 2 | TorF+U | PC3 | PC3 | 1 | 1 | 23 | 23 | 23 | 23 | 0 | 0 | 26 | N |
| 5 | ? | PC2 | 3 | TorF+U | PC3 | PC3 | 1 | 2 | 23 | 2x20 | 23 | 2x20 | 0 | 0 | 26 | N |
| 6 | ? | PC2 | 2 | TorF+U | PC2 | PC5 | 1 | 1 | 26 | 20 | 24.8 | 20 | -1.2 | 0 | 27 | Y(+1dB) |
| 7 | ? | PC2 | 3 | TorF+U | PC2 | PC5 | 2 | 1 | 2x23 | 20 | 2x21.8 | 20 | -1.2 | 0 | 27 | Y(+1dB) |
| 8 | ? | PC2 | 2 | TorF+U | PC2 | PC3 | 1 | 1 | 26 | 23 | 23 | 23 | -3 | 0 | 27.8 | Y(+1.8dB) |
| 9 | ? | PC2 | 3 | TorF+U | PC2 | PC3 | 1 | 2 | 26 | 2x20 | 23 | 2x20 | -3 | 0 | 27.8 | Y(+1.8dB) |
| 10 | ? | PC1.5 | 3 | T+U | PC1.5 | PC5 | 2 | 1 | 2x26 | 20 | 28.4 | 20 | -0.6 | 0 | 29.5 | Y(+0.5dB) |
| 11 | ? | PC1.5 | 3 | T+U | PC1.5 | PC3 | 2 | 1 | 2x26 | 23 | 27.8 | 23 | -1.2 | 0 | 30 | Y(+1dB) |

Observation:

- Case A to F are NR-U/NR-U cases with case A being the reference default power class.

- Case 1 is the reference default power class with PC3 NR + PC5 NR-U.

- Cases 2 and 3 could already support the next higher inter-band power class.

- There are four cases where one of the PA back-off is different from 0 or 3dB.

- There are more cases (18) compared to the NR+NR or LTE+NR cases (10).

- To our knowledge there is no HPUE inter-band combination requested with NR-U bands n46, n96 or n102 as one of the band.

It should be noted that for the MSD aspects, the NR-U cases have a specific treatment: In the case where the NR-U band is the victim, rather than an MSD value, a REFSENS exclusion range is specified which is then valid independently from the power class, thus:

- NR-U+NR-U cases (A to F) do not have MSD thus do not require an HPUE delta MSD.

- Most harmonic related cases have NR-U band as the victim and thus do not have MSD and do not require an HPUE delta MSD.

- Only 1UL harmonic mixing, cross-band and 2UL IMD cases may have NR band as the victim and thus would require a ∆MSD.

With all the above observations, we do not see an urgent need to derive delta MSD for band combination including at least one NR-U band. This will allow to focus on the key HPUE combinations and avoid additional complexity in the discussion.

Proposal: HPUE MSD for inter-band combinations including at least one NR-U band is out of scope of the simplification work for Release 19.

- This work may be done in a further release provided there are HPUE request for inter-band combinations including at least one NR-U band.

- It should be noted that cases where the NR-U band is the victim is covered by REFSENS exclusion and thus no ∆MSD is needed.

## 2.2 High-level assumptions to derive IMD interference level

To derive the IMD increase, after the details are known, one must make some assumptions in term of the dominating path for the IMD. Based on experience and existing analysis, we propose to use the assumptions in Table 3 for MSD to own band or to third band depending on the FDD/TDD types of the two bands and the number of Tx per band:

- T+T means 2UL is TDD+TDD => only the third band may be affected by MSD.

- F+T means 2UL is FDD+TDD => only the FDD band can be affected by MSD for two bands

- F+F means 2UL is FDD+FDD => both bands may be affected by MSD for two bands

- #DL denotes the number of DL bands in the inter-band combination

Proposal: The following table is proposed as assumptions for IMD increase calculations

Table 3: Proposed assumptions on dominating IMD generator versus 2/3 band DL, TDD/FDD and 2/3Tx

| #DLB | 2UL | Assumptions on dominating IMD | 3Tx assumption |
| --- | --- | --- | --- |
| 2 | F+T | Only FDD band is victim => dominated by FDD PA | If 2Tx on TDD => Only one TDD PA dominates coupling to FDD PA |
|  |  |  | If 2Tx on FDD => Only one FDD PA dominates IMD |
|  | F+F | Dominated by FDD PA associated with victim DL band | if 2Tx on victim FDD => Only one PA dominates IMD |
|  |  |  | if 1Tx on victim FDD => Only one of 2Tx PA dominates coupling to FDD PA |
| 3 | F/T+F/T | Dominated by victim band LNA | Only one LNA IMD is dominated by both UL |

Beyond these assumptions on the dominating IMD generator, there are other aspects where assumptions and possibly short cuts are needed:

- For the PA and the LNA even if the power levels are the same at the two PA outputs, all IMDs will be asymmetric in level because:

  - A PA sees only reverse IMD, thus its own signal and a small, coupled signal from the other PA

  - The LNA sees each PA with a different coupling loss.

- Asymmetric IMD is ignored as there is no way to set a generic imbalance and, even if that were the case, one would then have to look at which side IMD is the interfering one. Currently, even with a thorough analysis, there is no chance that the assumptions and levels would be the same than the one used for the PC3 analysis. In most cases, this issue is ignored regardless.

- When there is asymmetry in the level in each PA, this is considered in the IMD increase calculation by retaining the worst-case side.

- For a thorough analysis beyond the IMD order, one would have to account for the IMD index, especially when there is asymmetry in the two UL band levels.

- We also propose to ignore the IMD index since the asymmetry is not known or generic. Thus, we assume that the IMD increase is only proportional to the IMD order. Only the asymmetry of the power per band is considered and the worst-case IMD retained.

Proposal on IMD asymmetry and indexes: Since there is no way to adopt a generic approach in terms of UL band level asymmetry, we propose to ignore asymmetric levels and IMD indexes in the IMD increase calculations. However, when the level in each UL band is asymmetric, the worst IMD side is considered.

Finally, there could be as many architectures and antenna sharing schemes as one would like. The assumption here is the most common and, in most cases, the worst-case: the two UL bands share the main path antenna and thus the main antenna PA (1Tx or 2Tx) associated with the victim band dominates the IMD generation. Also, the levels at the victim LNA input are larger in the main path.

Proposal on architecture assumption: The main path of each UL band shares the same antenna and IMD interference dominates in the main DL path and is generated by the main PA associated with the victim band.

## 2.3 IMD increase calculations for 2B DL/2B UL

Using the assumptions described in the previous section, we have calculated the IMD increase for all nine HPUE cases and sub-cases by comparing step by step between the different cases. In this case, the IMD generator is the PA associated with the victim FDD band that shares its antenna.

These calculations are summarized in Table 4:

- The first seven columns provide the case description:

  - Column 1: Case number and related release

  - Column 2: Inter-band power class and number of Tx

  - Column 3-7: Band1 or band 2 parameters: Power class per band / Number of Tx per band/Output power per band in 2UL mode / Back-off per band in 2UL mode

- The eighth column shows the IMD increase versus the PC3 baseline in dB and in proportion of the IMD order “Nxorder” in bold. Then, it shows the increase versus another case which is often useful to check how IMD evolves iteratively.

  - In cases 3, 4, 6, and 8, some sub-cases are needed to account on which band (power class, number of Tx) dominated the IMD when the two bands are of different types.

- Finally, the last four columns provide the maximum applicable ∆MSD per IMD order.

Table 4: ∆MSD calculations summary for 2B DL/2B UL

| case /Rel | ULCA PC /#Tx | Bd | /Bd PC | /Bd #Tx | /Bd 2UL power | /Bd 2UL BO | 2DL2UL interference level inrease vs Ref / vs case(x) [dB] | ∆MSD max [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | 2 | 3 | 4 | 5 |
| 1 | PC3 | Bd1 | PC3 | 1Tx | 20 | -3 | Ref | 0 | 0 | 0 | 0 |
| R15 | 2Tx | Bd2 | PC3 | 1Tx | 20 | -3 |  |  |  |  |  |
| 2 | PC2 | Bd1 | PC3 | 1Tx | 23 | 0 | +3xorder / =(1)+3xorder | 6 | 9 | 12 | 15 |
| R17 | 2Tx | Bd2 | PC3 | 1Tx | 23 | 0 |  |  |  |  |  |
| 3 | PC2 | Bd1 | PC3 | 1Tx | 23 | 0 | 3a: if PC3 is FDD victim band: +3xorder / =(2) | 6 | 9 | 12 | 15 |
| R17 | 2Tx | Bd2 | PC2 | 1Tx | 23 | -3 | 3b: if PC2 is FDD victim band: +3 / =(1)+3 | 3 | 3 | 3 | 3 |
| 4 | PC2 | Bd1 | PC3 | 1Tx | 23 | 0 | 4a: if PC3 is FDD victim band: +3x(order-1)=+3xorder-3 / =(2)-3 in Bd2 | 3 | 6 | 9 | 12 |
| R18 | 3Tx | Bd2 | PC2 | 2Tx | 2x20 | -3 | 4b: if PC2 is FDD victim band: +3x(order-1)=+3xorder-3 / =(1)+3 in Bd1 | 3 | 6 | 9 | 12 |
| 5 | PC2 | Bd1 | PC2 | 1TX | 23 | -3 | +3 / =(1)+3 => since this could be PC1.5: | 3 | 3 | 3 | 3 |
| R19 | 2Tx | Bd2 | PC2 | 1Tx | 23 | -3 |  |  |  |  |  |
| 6 | PC2 | Bd1 | PC2 | 1TX | 23 | -3 | 6a: if 1Tx is FDD victim band: +3-3x(order-2)=-3xorder+9 / =(5)-3 in Bd2 | 3 | 0 | -3 | -6 |
| R19 | 3Tx | Bd2 | PC2 | 2Tx | 2x20 | -3 | 6b: if 2Tx is FDD victim band: +3x(order-1)=+3xorder-3 / =(1)+3 Bd1 | 3 | 6 | 9 | 12 |
| 7 | PC1.5 | Bd1 | PC2 | 1TX | 26 | 0 | +3xorder+3 / =(2)+3dB | 9 | 12 | 15 | 18 |
| R19 | 2Tx | Bd2 | PC2 | 1Tx | 26 | 0 |  |  |  |  |  |
| 8 | PC1.5 | Bd1 | PC2 | 1TX | 26 | 0 | 8a: if 1Tx is FDD victim band: +3x(order-1)+3=3xorder / =(4)+3dB | 6 | 9 | 12 | 15 |
| R19 | 3Tx | Bd2 | PC2 | 2Tx | 2x23 | 0 | 8b: if 2Tx is FDD victim band: +3*(order+order-1)=6xorder-3 / =(2)+3*(order-1) | 9 | 15 | 21 | 27 |
| 9 | PC1.5 | Bd1 | PC3 | 1Tx | 23 | 0 | +3*order+1.7*(order-1)=+4.7xorder-1.7/ =(2)+1.7*(order-1) | 7.7 | 12.4 | 17.1 | 21.8 |
| R18 | 3Tx | Bd2 | PC1.5 | 2Tx | 2x24.7 | -1.3 |  | =>9 | =>12 | =>18 | =>21 |
| 10 | PC1.5 | Bd1 | PC2 | 1TX | 26 | 0 | +3*(order-1)+3=3xorder / =(4)+3dB | 6 | 9 | 12 | 15 |
| R19 | 3Tx | Bd2 | PC1.5 | 2Tx | 2x23 | -3 |  |  |  |  |  |

Observation:

- The worst-case IMD increase versus PC3 is 27dB for IMD5 of PC1.5=PC2+PC2 2Tx when 2Tx is the victim band. This is high but one needs to consider that PC3 IMD5MSD levels are low, thus 27dB is highly unlikely.

- There are cases where the MSD would decrease (red highlight) for the PC2=PC2+PC2 2Tx case. This is because the levels are asymmetric and the PA generating the IMD has higher capability, thus higher-order IMDs would decrease. Since this is difficult to account for and the same implementation is capable of PC1.5, we propose to clamp the negative values at zero.

- Where yellow highlight appears, the UL level on one band is not increased by a 3dB step, resulting in IMD increases that are not a multiple of 3dB. Here, we suggest rounding to the closest 3dB increment.

- Overall, for any IMD increases, the maximum DeltaMSD is from 0 to 27dB in 3dB increments. The higher values are for higher IMD orders where the PC3 MSD is small, thus the maximum DeltaMSD is not reached.

- It should therefore be noted that higher order IMDs (orders 6,7..?) could be of concern for the inter-band highest power classes.

## 2.3 IMD increase calculations for 3B DL/2B UL

Using the assumptions for the third DL band MSD being dominated by the victim LNA IMD, we have calculated the IMD increase for all nine HPUE cases by comparing step by step between the different cases. These calculations are summarized in Table 5, which is ordered in the same fashion as Table 3.

Table 5: ∆MSD calculations summary for 3B DL/2B UL

| case /Rel | ULCA PC /#Tx | Bd | /Bd PC | /Bd #Tx | /Bd 2UL power | /Bd 2UL BO | 2DL2UL interference level inrease vs Ref / vs case(x) [dB] | ∆MSD max [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | 2 | 3 | 4 | 5 |
| 1 | PC3 | Bd1 | PC3 | 1Tx | 20 | -3 | Ref | 0 | 0 | 0 | 0 |
| R15 | 2Tx | Bd2 | PC3 | 1Tx | 20 | -3 |  |  |  |  |  |
| 2 | PC2 | Bd1 | PC3 | 1Tx | 23 | 0 | +3xorder / =(1)+3xorder | 6 | 9 | 12 | 15 |
| R17 | 2Tx | Bd2 | PC3 | 1Tx | 23 | 0 |  |  |  |  |  |
| 3 | PC2 | Bd1 | PC3 | 1Tx | 23 | 0 | +3xorder / =(1)+3xorder | 6 | 9 | 12 | 15 |
| R17 | 2Tx | Bd2 | PC2 | 1Tx | 23 | -3 |  |  |  |  |  |
| 4 | PC2 | Bd1 | PC3 | 1Tx | 23 | 0 | +3x(order-1)=+3xorder-3 / =(1)+3 Bd1 | 3 | 6 | 9 | 12 |
| R18 | 3Tx | Bd2 | PC2 | 2Tx | 2x20 | -3 |  |  |  |  |  |
| 5 | PC2 | Bd1 | PC2 | 1TX | 23 | -3 | +3xorder / =(1)+3xorder | 6 | 9 | 12 | 15 |
| R19 | 2Tx | Bd2 | PC2 | 1Tx | 23 | -3 |  |  |  |  |  |
| 6 | PC2 | Bd1 | PC2 | 1TX | 23 | -3 | +3x(order-1)=+3xorder-3 / =(1)+3 Bd1 | 3 | 6 | 9 | 12 |
| R19 | 3Tx | Bd2 | PC2 | 2Tx | 2x20 | -3 |  |  |  |  |  |
| 7 | PC1.5 | Bd1 | PC2 | 1TX | 26 | 0 | +6xorder / =(2)+3xorder | 12 | 18 | 24 | 30 |
| R19 | 2Tx | Bd2 | PC2 | 1Tx | 26 | 0 |  |  |  |  |  |
| 8 | PC1.5 | Bd1 | PC2 | 1TX | 26 | 0 | +3x(order)+3*(order-1)=6xorder-3 / =(2)+3 Bd1 | 9 | 15 | 21 | 27 |
| R19 | 3Tx | Bd2 | PC2 | 2Tx | 2x23 | 0 |  |  |  |  |  |
| 9 | PC1.5 | Bd1 | PC3 | 1Tx | 23 | 0 | +3*order+1.7*(order-1)=+4.7xorder-1.7/ =(2)+1.7*(order-1) | 7.7 | 12.4 | 17.1 | 21.8 |
| R18 | 3Tx | Bd2 | PC1.5 | 2Tx | 2x24.7 | -1.3 |  | =>9 | =>12 | =>18 | =>21 |
| 10 | PC1.5 | Bd1 | PC2 | 1TX | 26 | 0 | +3x(order)+3*(order-1)=6xorder-3 / =(2)+3 Bd1 | 9 | 15 | 21 | 27 |
| R19 | 3Tx | Bd2 | PC1.5 | 2Tx | 2x23 | -3 |  |  |  |  |  |

Observation:

- The worst-case IMD increase versus PC3 is 30dB for IMD5 of PC1.5=PC2+PC2. This is because the LNA power capability does not increase.

- Where yellow highlights appear, the UL level on one band is not increased by a 3dB step resulting in IMD increases that are not a multiple of 3dB. Here, we suggest rounding to the closest 3dB increment.

- Overall, for any IMD increase, the maximum ∆MSD is from 0 to 30dB in 3dB increments. The higher values are for higher IMD orders where the PC3 MSD is small, thus the maximum ∆MSD is not reached.

- It should therefore be noted that higher order IMDs could be of concern for the inter-band highest power classes similarly to the two band DL case.

## 2.4 Summary of the number of curves/equations needed for all ten cases and four IMD orders.

Since the IMD increase equation and maximum ∆MSD is applicable to multiple cases, we propose re-arranging the cases and assigning them to the related maximum ∆MSD values in 3dB increments. This is shown in Table 6.

Table 6: ∆MSD curve assignment per IMD order and 2DL/3DL 2UL cases.

| #DL band MSD | #Tx | Inte-band PC | per-band PC | Victim band PC | case | ∆MSDmax [dB] /IMD order |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 0 | 3 | 6 | 9 | 12 | 15 | 18 | 21 | 24 | 27 | 30 |
| 2DL | 2Tx | PC2 | PC3+PC3/PC3+PC2 | PC3 | 2/3a |  |  | 2 | 3 | 4 | 5 |  |  |  |  |  |
|  |  |  | PC3+PC2/PC2+PC2 | PC2 | 3b/5 |  | 2345 |  |  |  |  |  |  |  |  |  |
|  |  | PC1.5 | PC2+PC2 | PC2 | 7 |  |  |  | 2 | 3 | 4 | 5 |  |  |  |  |
|  | 3Tx | PC2 | PC3+PC2 2Tx/PC2+PC2 2Tx | PC3 1Tx/PC2 2Tx | 4/6b |  | 2 | 3 | 4 | 5 |  |  |  |  |  |  |
|  |  |  | PC2+PC2 2Tx | PC2 1Tx | 6a | 345 | 2 |  |  |  |  |  |  |  |  |  |
|  |  | PC1.5 | PC2+PC2 2Tx/PC2+PC1.5 2Tx | PC2 1Tx | 8a/10 |  |  | 2 | 3 | 4 | 5 |  |  |  |  |  |
|  |  |  |  | PC2 2Tx | 8b |  |  |  | 2 |  | 3 |  | 4 |  | 5 |  |
|  |  |  | PC3+PC1.5 2Tx | N/A | 9 |  |  |  | 2 | 3 |  | 4 | 5 |  |  |  |
| 3DL | 2Tx | PC2 |  |  | 2/3/5 |  |  | 2 | 3 | 4 | 5 |  |  |  |  |  |
|  |  | PC1.5 |  |  | 7 |  |  |  |  | 2 |  | 3 |  | 4 |  | 5 |
|  | 3Tx | PC2 |  |  | 4/6 |  | 2 | 3 | 4 | 5 |  |  |  |  |  |  |
|  |  | PC1.5 |  | PC2 band | 8/10 |  |  |  | 2 |  | 3 |  | 4 |  | 5 |  |
|  |  |  |  | PC3 band | 9 |  |  |  | 2 | 3 |  | 4 | 5 |  |  |  |

Table 6 description:

- Column 1: MSD to own DL band out of 2DL/2UL combination or third band in a 3DL / 2UL combination.

- Column 2: Inter-band power class, Column 3: Number of Tx, Column 4: Per band power class (when needed), Column 5: Power class of victim band.

- Column 6: Corresponding cases in previous Tables 4 and 5.

- Finally, to which maximum ∆MSD curve each IMD order applies per cases.

Observations:

- Multiple cases fall on the same ∆MSD curve, especially for 3DL and some are common between 2DL and 3DL.

- Although it looks like a high number of cases to be mapped, all the curves have a 3dB scaling to the others and are like the 1UL curves with 3, 6 and 9dB ∆MSDmax.

In the work above, we wanted to calculate the MSD increase covering all the cases we identified without further simplification. In the contribution [2] however, the proposal is further simplified with a ∆MSD per IMD order and PC2 or PC1.5 inter-band power class as in Table 7 and use an LUT rather that the equation model we proposed earlier (still provided in annex).

Table 7: 2UL ∆MSD LUTs proposed in [2]

Observations:

- The ∆PC2 IMD mapping in Table 7 is equivalent to the cases highlighted in yellow in Table 6.

- The ∆PC1.5 IMD mapping in Table 7 is equivalent to the cases highlighted in orange in Table 6.

- These correspond to the worst case ∆PC2 and ∆PC1.5 in table 6 respectively.

Since we recognize that the mapping in Table 6 is rather complex, we want to further simplify the number of cases. However, if it is acceptable to penalise some cases with up to 3dB higher MSD, there are several cases like the Table 6 2UL PC2 cases 3b/5/6a and 2UL PC1.5 cases 8a/10 where a worst-case approach like in Table 7 would significantly increase the MSD. We thus propose the following ∆MSDmax mapping in Table 8. The exact LUT proposal is part of a separate contribution that consolidates the 1UL and 2UL LUTs in a similar way than the tables proposed in [2] for each 3dB step ∆MSDmax and a separate table to map the different MSD types and power classes to the related ∆MSDmax LUT.

Proposal on 2UL band MSD for 2DL and 3DL bands and maximum 2UL CC ∆MSD: The mapping in table 8 is used for LUTs. It should be noted that 1UL ∆MSD can also be mapped to the same table.

Table 8: proposed ∆MSDmax mapping for 2UL MSDs

| 2UL Inter-band power class | Victim band | Cases in table 4/5 | ∆MSDmax [dB] /IMD order |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3 | 6 | 9 | 12 | 15 | 18 | 24 | 30 |
| 2DL PC2 | PC2 1Tx | 3b/5/6a | IMD2/3/4/5 |  |  |  |  |  |  |  |
| 2DL and 3DL PC2 | Other cases |  |  | IMD2 | IMD3 | IMD4 | IMD5 |  |  |  |
| 2DL PC1.5 | PC2 1Tx | 8a/10 |  | IMD2 | IMD3 | IMD4 | IMD5 |  |  |  |
| 2DL and 3DL PC1.5 | Other cases |  |  |  |  | IMD2 |  | IMD3 | IMD4 | IMD5 |

# Conclusion

In this contribution, we continued to analyze the delta MSD for the 2UL IMDs for HPUE inter-band combinations. We based our work on the following assumptions.

Proposal: HPUE MSD for inter-band combinations including at least one NR-U band is out of scope of the simplification work for Release 19.

- This work may be done in a further release provided there are HPUE request for inter-band combinations including at least one NR-U band.

- It should be noted that cases where the NR-U band is the victim is covered by REFSENS exclusion and thus no ∆MSD is needed.

Proposal: The following table is proposed as assumptions for IMD increase calculations

Table 3: Proposed assumptions on dominating IMD generator versus 2/3 band DL, TDD/FDD and 2/3Tx

| #DLB | 2UL | Assumptions on dominating IMD | 3Tx assumption |
| --- | --- | --- | --- |
| 2 | F+T | Only FDD band is victim => dominated by FDD PA | If 2Tx on TDD => Only one TDD PA dominates coupling to FDD PA |
|  |  |  | If 2Tx on FDD => Only one FDD PA dominates IMD |
|  | F+F | Dominated by FDD PA associated with victim DL band | if 2Tx on victim FDD => Only one PA dominates IMD |
|  |  |  | if 1Tx on victim FDD => Only one of 2Tx PA dominates coupling to FDD PA |
| 3 | F/T+F/T | Dominated by victim band LNA | Only one LNA IMD is dominated by both UL |

Proposal on IMD asymmetry and indexes: Since there is no way to adopt a generic approach in terms of UL band level asymmetry, we propose to ignore asymmetric levels and IMD indexes in the IMD increase calculations. However, when the level in each UL band is asymmetric, the worst IMD side is considered.

Proposal on architecture assumption: The main path of each UL band shares the same antenna and IMD interference dominates in the main DL path and is generated by the main PA associated with the victim band.

Based on those assumptions, the different ∆MSD curves were calculated, mapped, and simplified in the following proposals per maximum ∆MSD. The exact LUT for each ∆MSDmax 3dB step is part of another contribution that consolidates the 1ULO and 2UL LUTs.

Proposal on 2UL band MSD for 2DL and 3DL bands and maximum 2UL CC ∆MSD: The mapping in table 8 is used for LUTs. It should be noted that 1UL ∆MSD can also be mapped to the same table.

Table 8: proposed ∆MSDmax mapping for 2UL MSDs

| 2UL Inter-band power class | Victim band | Cases in table 6/7 | ∆MSDmax [dB] /IMD order |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3 | 6 | 9 | 12 | 15 | 18 | 24 | 30 |
| 2DL PC2 | PC2 1Tx | 3b/5/6a | IMD2/3/4/5 |  |  |  |  |  |  |  |
| 2DL and 3DL PC2 | Other cases |  |  | IMD2 | IMD3 | IMD4 | IMD5 |  |  |  |
| 2DL PC1.5 | PC2 1Tx | 8a/10 |  | IMD2 | IMD3 | IMD4 | IMD5 |  |  |  |
| 2DL and 3DL PC1.5 | Other cases |  |  |  |  | IMD2 |  | IMD3 | IMD4 | IMD5 |

Observation: We recognize that some of the analysis provided in this contribution would require more space for detailed explanations. We hope to provide more explanations online, offline, or during ad-hoc in the May meeting.

# References

[1] R4-2500218 Simplification of HPUE 2UL DeltaMSD, Skyworks Solutions Inc., RAN4#114

[2] R4-2501579 MSD beyond PC3, Qualcomm, RAN4#114

[3] R4-2502863 WF on TN HPUE, Samsung, Skyworks, OPPO, LGE, Apple, RAN4#114

[4] R4-250xxxx Combined 1UL and 2UL DeltaMSD LUT, Skyworks Solutions Inc., RAN4#114bis

# Annex: MSD curve calculation, modelling and scaling

The ∆MSD curves can be calculated by adding the IMD increase (Maximum ∆MSD value) to the PC3 interference value and recalculating the post MRC MSD assuming a 9dB imbalance between the main and diversity path. Table A.1 shows such calculations for the curves corresponding to a maximum Delta MSD of 3/6/9/12dB. The same calculation exists for all 3dB steps up to 30dB. It should be moted that the 1UL ∆MSD also follow similar curves.

Table A.1: ∆MSD curves calculations for cases with maximum delta MSD of 3/6/9/12dB versus PC3 MSD (0.1-29dB)

The ∆MSD values versus PC3 MSD for the same cases are plotted in a plain line in Figure A together with a model curve in the dashed line.

Figure A: 3/6/9/12dB ∆MSD curves vs PC3 MSD and their model (dashed)

The proposed modelling for the ∆MSD curves fits well with the calculated MSD and has several advantages versus the linear approach previously proposed or a single value:

- In general, the curve fitting is better than linear approach. The worst-case error is 0.5/0.8/0.8/1.5dB for the 3/6/9/12dB curves respectively. The error increases for the higher ∆MSD curves, but only for the very small values of PC3 MSD <0.3dB which are not used regardless.

- Compared with the linear approach where the MSD value would already be quite high for small PC3 MSDs, this model has 0dB ∆MSD for 0.1dB PC3 MSD.

- The model is pessimistic in the 0.5-6dB PC3 region but only for the low ∆MSD curves. This part can be changed by changing the assumed main to diversity interference imbalance, so it should not be a concern.

- The model directly scales with the maximum ∆MSD of a curve.

Possible ∆MSD model is as follows:

- Input parameter:

  - ΔMSDmax: Value from the last columns of Table 6 based on the case and IMD order

  - PC3MSD: PC3 MSD value for the corresponding IMD and case.

- Equation:

ΔMSD=Min(0.2*ΔMSDmax*(LN(PC3MSD)+2), ΔMSDmax)