3GPP TSG-RAN WG4 Meeting #114bis                    R4-2503320

# Wuhan, China, 7th – 11th April, 2025

Agenda item:			7.11.2

Source:	Xiaomi

Title:	Discussion the assumptions and parameters for NR NTN Ku Band coexistence study

Document for:	Approval

Background

The General requirements of the Rel-19 NR NTN Ku band work item had been discussed in RAN4#114. This document captured the Xiaomi’s opinions about the remaining issues of Ku-band NTN coexistence study parameters and assumptions.[1]

NTN Ku band Coexistence study based on ITU regulations

Co-existence simulation scenario for 11/14GHz

According to the document R4-2502315[2], RAN4 has the following 8 scenarios for coexistence study.

Table 1 Aggressor and victim of each scenario

| No | Combination | Aggressor | Victim | Frequency Band | Notes on the Ku Band | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | VSAT UL | UE UL | 14 GHz | Ku Band VSAT uplink interfering with gNodeB receiver. | The ACLR of the VSAT to be varied/defined. The ACS of the gNodeB is fixed. |
| 2 | TN with NTN | UE UL | VSAT UL | 14 GHz | UE uplink interfering with Ku Band Satellite receiver. | The ACLR of the UE is fixed. The ACS of the satellite to be varied/defined. |
| 3 | TN with NTN | VSAT UL | gNodeB DL | 14 GHz | Ku Band VSAT uplink interfering with UE receiver. | The ACLR of the VSAT to be varied/defined. The ACS of the UE is fixed. |
| 4 | TN with NTN | gNodeB DL | VSAT UL | 14 GHz | gNodeB downlink interfering with Ku Band Satellite receiver. | The ACLR of the gNodeB is fixed. The ACS of the Satellite to be varied/defined. |
| 5 | TN with NTN | gNodeB DL | Satellite DL | 11 GHz | gNodeB downlink interfering with Ku Band VSAT receiver. | The ACLR of the gNodeB is fixed. The ACS of the VSAT to be varied/defined. |
| 6 | TN with NTN | Satellite DL | gNodeB DL | 11 GHz | Ku Band Satellite downlink interfering with UE receiver. | The ACLR of the Satellite to be varied/defined. The ACS of the UE is fixed. |
| 7 | TN with NTN | Satellite DL | UE UL | 11 GHz | Ku Band Satellite downlink interfering with gNodeB receiver. | The ACLR of the Satellite to be varied/defined. The ACS of the gNodeB is fixed. |
| 8 | TN with NTN | UE UL | Satellite DL | 11 GHz | UE uplink interfering with Ku Band VSAT receiver. | The ACLR of the UE is fixed. The ACS of the VSAT to be varied/defined. |
| NOTE 1: For coexistence between Ku Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. |  |  |  |  |  |  |

However, for the coexistence study, not all of the scenarios are the worst case which can determine the ACLR and ACS for NTN system. In order to have a clear view for which scenario can be the worst case, Xiaomi summarized the following Table 2 from existing study results of TR38.863.

|  | Metrices | ACIR requirement | 2GHz | 17GHz DL | 27GHz UL |
| --- | --- | --- | --- | --- | --- |
| 1 | NTN UE ACLR | NTN UL interfering TN UL | 13 dB |  | 13 dB |
| 2 | NTN SAN ACS | TN UL interfering NTN UL | 28 dB |  | 1.5 dB |
| 3 | NTN UE ACLR | NTN UL interfering TN DL | 26 dB |  | 0 |
| 4 | NTN SAN ACS | TN DL interfering NTN UL | 37 dB |  | 16 dB |
| 5 | NTN UE ACS | TN DL interfering NTN DL | 28 dB | N/A |  |
| 6 | NTN SAN ACLR | NTN DL interfering TN DL | 23 dB | 0 |  |
| 7 | NTN SAN ACLR | NTN DL interfering TN UL |  | 0 |  |
| 8 | NTN UE ACS | TN UL interfering NTN DL |  | 2.5 dB |  |

Observation 1: For determining the ACLR and ACS of NTN SAN and UE, at least 4 scenarios are necessary for coexistence study.

Observation 2: To obtain NTN UE ACLR, NTN UL interfering TN UL and NTN UL interfering TN DL cases should be done to evaluate all the scenarios. However, it is not easy to get the conclusion which is the worst case basic on the two group of study results.

Observation 3: To obtain NTN SAN ACLR, NTN DL interfering TN DL and NTN DL interfering TN UL cases should be done to evaluate all the scenarios. But when we check the 17GHz results, both ACIR requirement are 0. It means that the interference from SAN to TN network are quite low with the 1.5km. We can only consider NTN DL interfering TN DL to make sure there is a worst result to determine the ACLR of NTN SAN.

Observation 4: To obtain NTN UE ACS, TN DL interfering NTN DL and TN UL interfering NTN DL cases should be done to evaluate all the scenarios. It is easy to know that the TN BS can generate more interference to NTN UE than TN UE, with the reason of the maximum EIRP, height and los probability. We can only consider TN DL interfering NTN DL to make sure there is a worst result to determine the ACS of NTN UE.

Observation 5: To obtain NTN SAN ACS, TN UL interfering NTN UL and TN DL interfering NTN UL cases should be done to evaluate all the scenarios. It is easy to know that the TN BS can generate more interference to NTN SAN than TN UE, with the reason of maximum EIRP. We can only consider TN DL interfering NTN UL to make sure there is a worst result to determine the ACS of NTN UE.

Proposal 1: In order to control the workload of coexistence study, RAN4 can evaluate scenarios:

- NTN UL interfering TN UL

- NTN UL interfering TN DL

- NTN DL interfering TN DL

- TN DL interfering NTN DL

- TN DL interfering NTN UL

Co-existence simulation assumption for 11/14GHz

Regarding the simulation results of 2GHz and Ka-band NTN[3], two options of 25 degrees and 90 degrees between NTN SAN and TN network were considered during the coexistence studies. 25 degrees results are always the worst case for the studies.

Proposal 2: In order to control the workload of coexistence study, RAN4 can ignore the 90 degrees cases for coexistence study.

For the Ku-band NTN UE, there are two types of antennas discussed during the offline discussion. 45*45 element phased array antenna with 0 or 3.5dBi element gain and Circular antenna with maximum 36.6dBi antenna gain. When we calculate the maximum antenna gain of phased array antenna 10log10​45×45+3.5=36.56dBi, which is very close to 36.6dBi.

Meanwhile new proposed GEO phased array antenna pattern 78*78 during the offline discussion. To cover the worst cases and minimized the cases:

Proposal 3: Suggest RAN4 to evaluate phased array antenna for GEO VSAT UE and circular VSAT UE for LEO for the first step.

Proposal 4: Suggest RAN4 to consider 3.5dB element gain for GEO phased array antenna VSAT UE.

To minimized the simulation cases, we can only consider one noise figure for GEO and LEO VSAT UE, therefore:

Proposal 5: Considering NTN VSAT noise figure 2.5 dB for GEO VSAT UE and 6 dB for LEO VSAT UE.

Conclusion

Proposal 1: In order to control the workload of coexistence study, RAN4 can evaluate scenarios:

- NTN UL interfering TN UL

- NTN UL interfering TN DL

- NTN DL interfering TN DL

- TN DL interfering NTN DL

- TN DL interfering NTN UL

Proposal 2: In order to control the workload of coexistence study, RAN4 can ignore the 90 degrees cases for coexistence study.

Proposal 3: Suggest RAN4 to evaluate phased array antenna for GEO VSAT UE and circular VSAT UE for LEO for the first step.

Proposal 4: Suggest RAN4 to consider 3.5dB element gain for GEO phased array antenna VSAT UE.

Proposal 5: Considering NTN VSAT noise figure 2.5 dB for GEO VSAT UE and 6 dB for LEO VSAT UE.

Referecne

- RP-243314, “Revised WID: Introduction of Ku bands for NR NTN”, 3GPP RAN#105

- R4-2502315 Co-existence simulation assumptions for NR NTN Ku band (Running document)

- TR 38.863