# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503348

# Wuhan, Hubei, China, 07th – 11th April, 2025

Agenda Item:	7.29.4.1

Source: 	OPPO

Title: 	On RRM requirements of (e)RedCap UEs with NR FR1-NTN

Document for:	Approval

Introduction

In last RAN4 meeting, a WF on RRM requirements for Rel-19 NR NTN phase3 was approved [1].

In this paper, we will provide our views on the remaining issues of (e)RedCap UEs with NR FR1-NTN.

Discussion on support of (e)RedCap UE with NTN

- Issue 1-2-3: SMTCs and 'in parallel' related - Issue 1-2-3-1: Feature 25-1(parallelSMTC) Online discussion <Way Forward>: Options for further discussion: If not differentiate 1Rx and 2Rx RedCap UEs, the baseline for UE capability is: - Option 1: [1 SMTC or] 2 SMTCs not in parallel - Option 2: 2 parallel SMTCs - ‘Parallel’ means the SMTCs are TDM with proximity distance. - Issue 1-2-3-1a: The proximity distance between two SMTCs Ad-hoc agreement Agreement: - RAN4 to define the proximity distance between two SMTCs for (e)RedCap UEs in NTN. - The distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion. - For whether to consider relaxation for proximity when (e)RedCap UE is configured with more than one SMTC on a SAN carrier, - Option 1: No. RAN4 to reuse the legacy collision condition. - Option 2: Yes. RAN4 to consider relaxation for proximity, i.e., - When (e)RedCap UE is configured with more than one SMTC on a SAN carrier, two SMTC occasions outside measurement gap are considered colliding if at least one of the following conditions is met: - the two SMTC occasions are fully or partially overlapping in time domain, or - the magnitude of the distance between two SMTC occasions in time domain is less than or equals to (4+x) ms. (x is the relaxation for (eRedCap UE)), x can be FFS. (1ms as starting point)

So far, normal NTN is mandatorily required to support 2 SMTCs per MO, and supporting more is an optional capability. When the two SMTCs do not overlap, there is no scaling in measurement period, which means UE has to support parallel measurement for the two SMTCs. And RAN4 also defined proximity distance for such parallel SMTCs, i.e., the magnitude of the distance between two SMTC occasions in time domain is no less than 4 ms.

However, for Redcap UE supporting FR1 NTN, it could be a bit challenging for UE with reduced capabilities and complexity. Thus, we prefer to define the minimum requirements for the worst case, that is, only 1 SMTC is used for measurement in the case of 2 SMTCs configured by network, regardless of proximity distance.

To consider higher-end UE capable of parallel SMTCs, Redcap UE with 2Rx can be a starting point.

Proposal 1: RAN4 to consider the minimum requirements for (e)RedCap UE with FR1-NTN bands, e.g., 1 SMTC is used for measurement regardless of proximity distance of 2 SMTCs configured for 1Rx Redcap UE.

RAN4 also discussed whether to relax the proximity distance between two SMTCs for (e)RedCap UEs when (e)RedCap UE is configured with more than one SMTC on a NTN carrier. The distance between two SMTC occasions is defined as the time difference between the ending point of the earlier SMTC occasion and the starting point of the later SMTC occasion. We prefer to reuse the legacy value of 4ms. If UE supports parallel processing two SMTC in TDM with proximity distance, we don’t see the big difference from legacy assumption. Otherwise, UE can report it doesn’t support the parallelSMTC capability.

Proposal 2: To consider higher-end UE capable of parallel SMTCs, Redcap UE with 2Rx can be a starting point.

Proposal 3: Prefer to reuse the legacy value of 4ms without relaxation for the proximity distance between two SMTCs for (e)RedCap UEs when (e)RedCap UE is configured with more than one SMTC on an NTN carrier.

- Issue 1-2-3-4: Feature 25-2(parallelMeasurementWithoutRestriction) Ad-hoc agreement Agreement: - For whether to define RRM requirements for Feature 25-2(parallelMeasurementWithoutRestriction-r17) for (e)RedCap UEs, - The UE capability is applicable, and whether to support this optional feature is up to UE implementation. RAN4 to follow the same principle of the existing non-RedCap requirements. - Issue 1-2-3-5: Feature 25-3(parallelMeasurementGap) <Way Forward>: - For whether to define RRM requirements for Feature 25-3(parallelMeasurementGap-r17) for (e)RedCap UEs, - Proposal 1: Support to define. - Proposal 1a: The UE capability is applicable, and the support of the optional features is left to UE implementation following the same principle of the existing non-RedCap requirements. - Proposal 1b: The capability can be optionally supported by (e)RedCap UEs with 2Rx, can not be supported by (e)RedCap UEs with 1Rx. - Deprioritize parallel measurement gap for (e)RedCap UEs in NTN.

RAN4 agreed on the principle on Feature 25-2 that the UE capability is applicable, and whether to support this optional feature is up to UE implementation. RAN4 to follow the same principle of the existing non-RedCap requirements.

For feature 25-3(parallelMeasurementGap), we prefer to deprioritize parallel measurement gap for (e)RedCap UEs in NTN.  But we can also compromise that the same principle for Feature 25-2 can apply. RAN4 do not define new requirements for Redcap UE with FR1-NTN, but follow the same of the existing non-RedCap requirements.

Proposal 4: Deprioritize parallel measurement gap for (e)RedCap UEs in NTN.

Conclusions

In this paper we provided our views on RRM requirements for support of (e)RedCap UEs with NR FR1-NTN.

Proposal 1: RAN4 to consider the minimum requirements for (e)RedCap UE with FR1-NTN bands, e.g., 1 SMTC is used for measurement regardless of proximity distance of 2 SMTCs configured for 1Rx Redcap UE.

Proposal 2: To consider higher-end UE capable of parallel SMTCs, Redcap UE with 2Rx can be a starting point.

Proposal 3: Prefer to reuse the legacy value of 4ms without relaxation for the proximity distance between two SMTCs for (e)RedCap UEs when (e)RedCap UE is configured with more than one SMTC on an NTN carrier.

Proposal 4: Deprioritize parallel measurement gap for (e)RedCap UEs in NTN.

Reference

- R4-2502604, WF on RRM requirements for Rel-19 NR NTN phase3, CATT