# 3GPP TSG-RAN WG4 Meeting #114bis	R4-2503353

# Wuhan, China, 7th – 11st April, 2025

Agenda Item:	7.21.3

Source: 	OPPO

Title: 	Study of testability and interoperability for AI mobility

Document for:	Approval

- Introduction

In RAN#102 meeting, the SID on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR is approved. In RAN#105 meeting, the related work plan in RAN4 has been further checked and updated as follows:

Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

In last meeting, the testability issues for AI mobility were not discussed due to limited time. In this contribution, we will further discuss the RAN4 impacts on testability based on RAN2 progress and the open issues captured in [2].

- Discussion

In RAN4#112bis meeting, the following three scenarios were agreed to be discussed in RAN4, and it was agreed that additional new testability aspects can be considered comparing to the work in “AI/ML for NR air interface”.

Issue 1-1-2: Scenario/sub-use-case priorities Agreement: RAN4 to start the discussion for scenario 2, 3, 4. Further clarification can be discussed for the scenarios. 2; High; FR1 to FR1 intra-frequency temporal domain case B; 1st goal; Intra-cell. 3; High; FR1 to FR1 inter-frequency (frequency domain); 1st goal; Inter-cell. 4; High; FR2 to FR2 intra-frequency temporal domain case A; 2nd goal; Intra-cell. Issue 3-1-1: New testability aspects - Agreement - According to the SID, RAN4 to leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - RAN4 will need to consider at least the following testability aspects for AI/ML-enabled mobility SI: - Testing in FR1 - Testing inter-carrier scenario, i.e., inter-frequency inter-cell prediction (for scenario 3) - Testing cell-level L3-RSRP measurements - Consideration of serving and neighbor cells Issue 3-2-1: Testing in FR1 - Agreement: - RAN4 to consider Option 2 as a baseline option for testing in FR1. - Option 2: Conducted testing

Testing for temporal domain

In legacy testing, the transmission power is consistent over time. But for temporal prediction case A and case B, to verify the prediction performance by using previous measurement results, RAN4 needs to model the time-varying characteristics considering UE mobility, channel modelling and transmission power etc. The candidate way can be modelling the time-varying RSRP value from transmitting side according to UE trajectories or changing the channel model with a fixed transmission power.

Proposal 1: RAN4 to discuss how to model the time-varying characteristics for temporal prediction (e.g., according to UE trajectories or changing the channel model with a stable transmission power).

Testing in FR2:

For FR2 prediction, in which both temporal and spatial characteristics need to be considered, proposals and conclusions that RAN4 made for AI BM can be reused, e.g., whether the chamber in OTA testing can be utilized and how to utilize/revise it. Also, questions like how to build channel modelling and channel correlation in the temporal domain for FR2 temporal domain prediction should be evaluated.

Proposal 2: For FR2 prediction, proposals and conclusions that RAN4 made for AI BM can be reused as baseline, e.g., whether the chamber in OTA testing can be utilized and how to utilize/revise it.

Testing for inter-carrier scenario:

For AI mobility, an important feature is the introduction of an inter-carrier scenario, e.g., inter-frequency inter-cell measurement prediction. And RAN2 has the following agreements for this scenario:

Agreements/Observations to be captured in TR - For the inter-frequency prediction, the evaluation results show that the higher the correlation coefficient is between two frequency layers, the higher the prediction accuracy. FFS on observations on low correlations. - For inter-frequency, cluster as input (i.e. measurements from different cells as inputs )can improve the prediction accuracy than single cell as input. For temporal domain, the gains are unclear. RAN2 will focus on frequency domain for cluster based approach. - In cluster approach the model takes measurements from more than one cell as inputs. 12 For Inter_F_C (inter-frequency inter-cell), RAN2 start evaluation from co-located scenario 13 for Inter_F_C(inter-frequency inter-cell), RAN2 should focus on the case where cell for measurement and cell for prediction are in the same sector. Agreements on inter-frequency - Companies need to report whether earlier predicted results are also used as inputs for future RRM prediction. - Companies should report with their simulation the correlation coefficient - Higher-to-lower and lower-to-higher frequency prediction is comparable - For co-located scenario, the UE speed in the inter-frequency case has minor impact on prediction accuracy

Based on RAN2 agreements, co-located scenario is used as starting point for inter-frequency prediction. And for single cell prediction, RAN2 focus on the case where cell for measurement and cell for prediction are in the same sector.

In such cases, the following issues should be considered:

- Spatial modeling:

How to ensure/mimic that the cell to be measured and the cell to be predicted are located in the same sector of either the serving site or the same neighboring site. For instance, if we're using conductive testing in FR1, how do we reflect the assumption of co-location among different cells? For example, are there any new designs and limitations needed during channel modeling?

- Temporal modeling:

Furthermore, it's important to consider how to construct and obtain different cell’s RSRP corresponding to a specific temporal relationship. For example, if we consider predicting cell B's RSRP at time T based on cell A's RSRP at time T, we need to consider how to construct and obtain both cell A's RSRP and cell B's RSRP corresponding to the same time T in the testing environment. Building a stable time-domain RSRP signal for a single test might be needed, e.g., configure DL signals of BS emulator with stable power which do not change with time.

- Channel modeling:

Based on RAN2 evaluation, the higher the correlation coefficient is between two frequency layers, the higher the prediction accuracy. To ensure the testability of measurements for inter-cell prediction, RAN4 need to discuss how to model the proper correlation between two frequency layers.

For FR1, considering the test environment is conducted testing, RAN4 should re-evaluate the selection of channel models. Option 1: UMA channel, Option 2: CDL channel

Proposal 3: For inter-carrier scenario (inter-frequency inter-cell prediction), the following aspects need to be discussed:

- how to model co-located or co-site scenario for the cell to be measured and the cell to be predicted.

- how to construct and obtain different cell’s RSRP corresponding to a specific temporal relationship.

- channel modelling used for inter-cell prediction

Testing L3-RSRP measurements instead of L1-RSRP

Regarding L3 RSRP, compared with L1 RSRP, the key difference lies in the L3 filter which may need to be included as part of the testing side conditions.

Proposal 4: RAN4 to consider including L3 filter coefficient as part of the testing side conditions.

Testing setup

For each scenario, further consideration on the measurement configurations is needed on how to define it in RAN4 tests. This includes the configuration of the observation window (e.g., window length, measurement sample configuration within the window), the configuration of the prediction window (e.g., window length, samples to be predicted within the window), and other test settings/conditions. The above aspects can be referenced based on the progress of RAN2's research, particularly the evaluation assumptions.

According to our companion paper on the requirements impact of AI mobility, the above configurations may have impacts on the core and performance requirements. How to setup these configurations can be discussed based on the requirements definition.

Proposal 5: RAN4 to consider how to setup the measurement configurations (e.g., OW, PW, MRRT, etc.) in the tests based on the sets of requirements.

For other testing setup, we think the legacy configurations can be used as baseline and can be updated if needed based on the above aspects (e.g., cell number, channel modelling, beam number, frequency correlation, OW, PW, MRRT, etc.).

Regarding the number of cells, for intra-frequency measurement, since the target is to predict the measurement on time domain for the same cell, we understand 2 cells are enough. For inter-frequency measurement, if single cell approach is considered, 2 or 3 cells are enough. And if cluster approach is used, more cells is needed which depends on the requirements definition and can be discussed in WI stage.

Proposal 6: For intra and inter frequency measurement prediction, RAN4 to use legacy testing setup as baseline and include the AI specific parameters if needed.

Proposal 7: For intra frequency measurement prediction, consider 2 cells (serving cell and one neighbour cell) in the testing setup.

Proposal 8: For inter frequency measurement prediction, more than 3 cells are needed if cluster approach is assumed and the exact number can be decided in WI stage according to the requirements discussion.

Ground Truth derivation

Regarding the definition of prediction accuracy, the following agreements were reached in RAN4#113 meeting:

Issue 2-1-9a: L3 RSRP prediction accuracy definition Agreement: - For testing, Absolute L3 Predicted RSRP Accuracy = reported predicted L3-RSRP – ground truth of L3-RSRP - FFS Ground truth definition of L3-RSRP for FR1 and FR2.

According to our companion paper on the requirements impact of AI mobility, the ground truth in accuracy requirements is defined as the actual L3-RSRP measurement results at point C based on legacy measurement procedure. And we think it should also be used in the testing.

Proposal 9: Regarding the ground truth derivation in tests, the actual measurement report based on legacy procedure can be used.

Testing goal

Regarding the testing goal, the following agreement was reached in previous meeting.

- Agreement - As baseline, the testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved.

For intra and inter measurement prediction, similar as legacy, the testing goal is to verify the measurement reported in OW meets the new requirements if defined and to verify the predicted measurement in PW meets the accuracy requirements under certain side conditions.

Proposal 10: The testing goal for measurement prediction is to verify

- the measurement reported in OW meets the new requirements if defined;

- the predicted measurement in PW meets the accuracy requirements under certain side conditions.

Testing of Generalization aspects

For the generalization of test, we understand it is a common issue for AI based use cases. The issue is still discussing in AI/ML for NR Air Interface without consensus and we think the methodologies can be reused for mobility use case. In addition, RAN2 has the following agreements on the model generalization for AI mobility:

Agreements on generalization over UE speed - For intra-frequency temporal domain prediction in FR1/FR2, generalization performs well across all UE speeds - Using a mixed dataset (GC#2) slightly improves the accuracy of the AI/ML model compared to GC#1 cases, while offering comparable accuracy as the baseline case (for the same data set size). - For GC#1, as the difference between the UE speed that an AI/ML model trained at and the UE speed that the inference is being made decreases, the AI/ML achieves a closer performance to the baseline. Agreements on generalization over frequency - Generalization using GC#2 always outperform that of GC#1 - Training and generalization using the knowledge about the input & output frequency (or even an indication) outperforms the case where the model cannot recognize which frequency at the input or output. - Considering the generalization parameter of different predicted frequencies in inter-freq prediction, the GC#1 case without any preprocessing based on the information of predicted frequency suffers from significant performance loss. - For GC#2 case, the prediction accuracy is acceptable and close to the baseline Agreements 1 Two sets of parameters (ISD, BS antenna height, BS Tx power) are used for the generalization across cell configurations study. 2 FR1 is the primary focus, companies can also submit results for FR2 (however, each set of generalization results covers either FR1 or FR2). 3 Agree on the two sets of configurations as in tables 2 and 3 (for FR1 and FR2). Parameter; Configuration #1; Configuration #2. Deployment scenario; UMi; UMa. ISD; 200m; 500m. BS antenna height; 10m; 25m. BS Tx power; 40dBm; 44dBm. Table 2: generalization parameters for FR1 Parameter; Configuration #1; Configuration #2. Deployment scenario; UMi; UMa. ISD; 200m; 500m. BS antenna height; 10m; 25m. BS Tx power; 40dBm; 44dBm. Table 3: generalization parameters for FR2

During RAN2 discussion, three aspects of generalization are considered for AI mobility, including the generalization for different UE speeds, generalization across different cell configurations ((ISD, BS antenna height, BS Tx power)) and for inter-frequency prediction, the generalization across different frequencies.

From RAN4 perspective, during WI stage, RAN4 need to discuss how to set the different configurations of UE speed, frequencies and cell configuration, e.g., dynamic configuration or fixed multiple sets of configurations. The same discussion is under AI/ML for NR Air Interface. The conclusions can be referred when WI for AI mobility starts.

Proposal 11: RAN4 to discuss the configurations for generalization in WI stage and the methodology discussed in NR_AIML_Air WI can be used as starting point.

- Conclusions

In this contribution, we discussed RAN4 impacts on testability for AI mobility and got the following proposals:

Proposal 1: RAN4 to discuss how to model the time-varying characteristics for temporal prediction (e.g., according to UE trajectories or changing the channel model with a stable transmission power).

Proposal 2: For FR2 prediction, proposals and conclusions that RAN4 made for AI BM can be reused as baseline, e.g., whether the chamber in OTA testing can be utilized and how to utilize/revise it.

Proposal 3: For inter-carrier scenario (inter-frequency inter-cell prediction), the following aspects need to be discussed:

- how to model co-located or co-site scenario for the cell to be measured and the cell to be predicted.

- how to construct and obtain different cell’s RSRP corresponding to a specific temporal relationship.

- channel modelling used for inter-cell prediction

Proposal 4: RAN4 to consider including L3 filter coefficient as part of the testing side conditions.

Proposal 5: RAN4 to consider how to setup the measurement configurations (e.g., OW, PW, MRRT, etc.) in the tests based on the sets of requirements.

Proposal 6: For intra and inter frequency measurement prediction, RAN4 to use legacy testing setup as baseline and include the AI specific parameters if needed.

Proposal 7: For intra frequency measurement prediction, consider 2 cells (serving cell and one neighbour cell) in the testing setup.

Proposal 8: For inter frequency measurement prediction, more than 3 cells are needed if cluster approach is assumed and the exact number can be decided in WI stage according to the requirements discussion.

Proposal 9: Regarding the ground truth derivation in tests, the actual measurement report based on legacy procedure can be used.

Proposal 10: The testing goal for measurement prediction is to verify

- the measurement reported in OW meets the new requirements if defined;

- the predicted measurement in PW meets the accuracy requirements under certain side conditions.

Proposal 11: RAN4 to discuss the configurations for generalization in WI stage and the methodology discussed in NR_AIML_Air WI can be used as starting point.

- References

R4-2502594 WF on FS_NR_AIML_Mob

R4-2500533 Topic summary for [114][219] FS_NR_AIML_Mob