3GPP TSG-RAN WG4 Meeting #114bis		R4-2503384

# Wuhan, China, 7th – 11th April, 2025

Agenda Item:	7.26.4.2

Source: 	CMCC

Title: 			 Discussion on RRM impact of LP-WUR

Document for:	Discussion

- Introduction

In last meeting, RAN4 further discussed detail LP-WUR requirements, MR RRM relaxation requirements and measurement accuracy related issues. The WF has been approved in [1], and the open issues are captured in the WF and topic summary [1, 2]. In this contribution, we further analyze the RRM impact of LP-WUR, and provide our proposals.

- Discussion

  - General aspects

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

Background:

Case #1 and #3 have been agreed

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

RAN4 to further discuss case #2

| RRM measurement case index | MR serving cell measurement | MR neighbouring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |

Based on our internal checking, for this issue, RAN2 is waiting for the agreement from RAN4.

The legacy cell re-selection procedure in TS 38.304 are:

- If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may not perform intra-frequency measurements.

- If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ,  the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority.

These two procedures should also be valid for LP-WUR capable UE.

- Proposal 1: The legacy cell re-selection measurement rules should also be valid for LP-WUR capable UE, including:

- If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may not perform intra-frequency measurements.

- If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ,  the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority.

Then, as long as network configures Case#3 entry condition lower than SnonIntraSearchP and/or SIntraSearchP, and Case#1 entry condition higher than SIntraSearchP, after UE entry in Case 3 and the Srelev becomes higher than SIntraSearchP and lower than the entry condition of Case#1, only MR serving cell relaxed measurement need to be performed, which is Case#2. Hence, Case#2 is a possible case, and its existence depends on network configuration.

- Proposal 2: Case#2 is a possible case, its existence depends on network configuration. No separate discussion is needed for Case#2.

Issue 1-1-7: On jointly consideration on Criteria (entry/exit conditions) for fully offloading/MR RRM measurement relaxation and LP-WUS monitoring

- Proposals

  - P1-1: Define same entry/exit conditions for LP-WUS monitoring and RRM measurement relaxed cases (case 3). (xiaomi LG Apple vivo).

  - P2: The LR-based LP-WUR measurement threshold can be less than to the LR-based LP-WUS monitoring threshold. (LG)

  - P3: RAN4 to suggest RAN2 to define same entry/exit condition for LP-WUS monitoring, serving cell RRM measurement offloading and MR RRM measurement relaxation. (Huawei)

Based on the latest RAN2 agreement, the summary of entry/exit conditions is:

| Case | Entry conditions | Exit conditions |
| --- | --- | --- |
| LP-WUS monitoring | MR threshold and optional LR threshold | LR threshold |
| RRM measurement fully offloading (Case 1) | MR threshold and optional LR threshold | LR threshold |
| RRM measurement relaxation (Case 3) | MR threshold and FFS LR threshold | FFS |

Currently, the entry/exit condition for LP-WUS monitoring and the entry/exit condition for LP-WUR measurement are discussed separately. However, from the perspective of UE power saving gain, it is meaningless that UE entry the LP-WUS monitoring while performing the measurement by MR. Two conditions should be considered jointly. To guarantee the power saving gain, the LP-WUS monitoring entry/exit threshold shall be equal to or higher than the threshold for RRM measurement relaxation (Case 3).

- Proposal 3: In order to guarantee the power saving gain, the LP-WUS monitoring entry/exit threshold shall be equal to or higher than the threshold for RRM measurement relaxation (Case 3).

Issue 1-1-8: LP-WUR operating carrier frequency

LS from RAN1#118 R1-2407559

- In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. - RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs. To RAN2 and RAN4: ACTION: RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

WF from RAN#106

- For idle/inactive mode, consider a standardized solution for intra-frequency case (e.g. solution 1, 2, and 3 in RP-242656). For Rel-19, inter-frequency solutions (e.g. solution 4) will not be considered. - Solution 1: MR is prioritized (via SIBs) to camp on the same band as LR (System information, Paging, RACH on LR band) - Solution 2: MR is prioritized to camp on the same band as LR (System information, Paging on LR band; and RACH on another MR band based on reselection criteria) - Solution 3: MR is not prioritized to camp on the same band as LR but can be redirected via dedicated signalling depending on LR band load (up to gNB implementation) - Detailed discussions and final solution selection is left up to WGs.

LS from RAN2#129 R2-2501394

- RAN2 would like to thank RAN1 for the LS on LP-WUS operation in IDLE/INACTIVE mode in R2-2407921/R1-2407559. In addition to RAN2 reply LS in R2-2410957, RAN2 further discussed the details/pros/cons of solutions based on the way forward from RAN plenary in RP-242656. The corresponding discussion report is in R2-2501443. And RAN2 achieved the following conclusions: - RAN2 understands that UE can report which band(s) is supported by LR to NW. - RAN2 understands that any potential overload issues could be addressed by current mechanism in spec.

- In last meeting, Solution 3 was finally adopted in RAN2, we don’t identify any RAN4 impact from Solution 3, RAN4 could close this issue.

- Proposal 4: No RAN4 impact from Solution 3, close the issue.

Issue 1-1-10: Considerations on higher priority frequency layer

- Proposals

  - P1: Support relaxed higher priority frequency layer search at case 1 and case 3 (xiaomi CT ZTE Huawei vivo Ericsson CT)

    - P1-1: For case 1 and case 3, when the serving cell is above threshold for inter-frequency neighbour cell measurement, existing relaxed requirements K2*Thigher_priority_search are re-used for higher priority frequency search. (Ericsson vivo)

    - P1-2: If serving cell is above threshold for inter-frequency neighbor cell measurement (Case #4), FFS whether existing requirements (60 * Nlayers) are re-used or further relaxed. If serving cell is below threshold for inter-frequency neighbor cell measurement (Case #3), UE meet the relaxed measurement requirements. (Huawei)

  - P2: Higher priority layer neighbou cell measurement shall also be OFF for case #1. (Apple vivo)

  - P3: When LR and MR operates at different band, whether to turn on higher priority measurement depends on the solution for issue 1-1-8. If Solution 1 for issue 1-1-8 is adopted, then the higher priority measurement can be OFF. If Solution 2 or 3 for issue 1-1-8 is adopted, the higher priority measurement should be ON (CMCC)

Recommendations:

Continue discussion.

The higher priority frequency measurement strategy is important for operator, to deploy the certain frequency layer in hotspot area to absorb capacity. Therefore, even UE could enter Case#1, 2, and 3 when fulfilling the criteria, the higher priority measurement still need to be ‘ON’. In order to guarantee UE power saving gain, we are ok to only apply the relaxed higher priority layer measurement requirement.

- Proposal 5: As long as network configure higher priority layer, UE should monitor it for possible cell-re selection. The relaxed higher priority layer measurement requirement can be applied, i.e., per higher priority layer within 1h.

  - Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Issue 1-2-4-1: On LR measurement filtering requirement

- Proposals

  - P1: The UE shall filter the LP-RSRP and LP-RSRQ and [SS-RSRP and SS-RSRQ] measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by either LP-SS periodicity/2 for LP-SS based LR or [LO periodicity /2] for SSB based LR. (Huawei Ericsson vivo)

In legacy, there are multiple SMTCs in one I-DRX cycle. To guarantee the filter gain, two measurements shall be spaced by DRX cycle/2.

However, for LP-SS measurement, the sampling period is LP-SS periodicity, and only one LP-SS resource in one LP-SS periodicity. Therefore, such filtering requirement is not needed.

For SSB measurement, the sampling period is LO periodicity. Similar as legacy, there can be multiple SSBs in one LO periodicity, to guarantee the filter gain, we are fine to introduce the filtering requirement that two measurements shall be spaced by LO periodicity/2.

- Proposal 6: The UE shall filter the LP-SS-RSRP and LP-SS-RSRQ and LP-SSS-RSRP and LP-SSS-RSRQ measurements of the serving cell using at least 2 measurements.

- For LP-SS-RSRP and LP-SS-RSRQ measurement, no need to define filtering space.

- For LP-SSS-RSRP and LP-SSS-RSRQ measurement, within the set of measurements used for the filtering, at least two measurements shall be spaced by LO periodicity /2.

Issue 1-2-5-2: On MR evaluation requirements

- Proposals

  - P1: For MR evaluation requirement for LP-WUR monitoring, case 1 and case 3, consider the following options (vivo):

    - Option 1: reuse Nserv ;

    - Option 2: reduce Nserv value by half;

    - Option 3: Nserv  always = 1; (LG xiaomi)

Background:

Recommendations:

Check whether option 2 is agreeable. The following table could be used for illustration purpose

Table x Nserv_WUR for MR evaluation requirement for LP-WUR monitoring, case 1 and case 3

| DRX cycle length [s] | Scaling Factor (N1) |  |  | Option 1: Nserv_WUR [number of DRX cycles] | Option 2: (vivo) Nserv_WUR [number of DRX cycles] | Option 3: (vivo LG xiaomi) Nserv_WUR [number of DRX cycles] |
| --- | --- | --- | --- | --- | --- | --- |
|  | FR1 | FR2-1 | FR2-2 |  |  |  |
| 0.32 | 1 |  |  | M1*N1*4 | M1*N1*2 | M1*N1 |
| 0.64 | 1 |  |  | M1*N1*4 | M1*N1*2 | M1*N1 |
| 1.28 | 1 |  |  | N1*2 | N1*1 | N1 |
| 2.56 | 1 |  |  | N1*2 | N1*1 | N1 |

When UE is in legacy state, we prefer to reuse Nserv for MR serving cell evaluation requirement, which is similar as R16 relaxation criteria evaluation.

When UE is in Case#2 and Case#3, in last meeting, RAN4 has agreed that relaxation factor 16 will be applied for serving cell measurement, which means Nserv_relax = 16 * Nserv. If we still use Nserv_relax, then evaluation period is too long to guarantee the timeliness of the evaluation. Therefore, we think Option 2 can be the starting point.

- Proposal 7: For MR evaluation requirement for LP-WUR monitoring/measurement, when UE is in legacy state, reuse Nserv; when UE is in Case#2 or Case#3, consider Nserv*16/2 as the starting point.

Issue 1-2-6: On MR wake up delay related requirements for LP-WUR

- Proposals

  - P1: MR wake-up time is pending on UE capability which can be 400ms and 800ms based on RAN1 agreement; MR sync time, need to be further discuss with different side conditions (LG)

    - Condition 1: MR and LR well sync, and LR sync info can be fully reused e.g.  1~2 SSB sufficient

    - Condition 2: MR and LR not well synced and MR need to apply sync procedure e.g. at least 10 SSBs required

  - P2: Wait for RAN1 further progress. (CMCC CATT oppo Huawei Ericsson)

  - P3: RAN4 only needs to define the wake-up delay requirement, if such requirement is necessary from majority companies’ view, where the wake-up delay means minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. (Apple)

  - P4: After MR is triggered to be waken up and during the waking up duration, MR should follow SSB periodicity instead of DRX periodicity to perform measurement (vivo)

  - P5: Defining MR wake up requirement for the normal case and the “wake up delay” duration for the normal is: “wake up delay” duration = Ramping duration + [5] samples * SSB periodicity; Whether to further consider a worst case where a cell reselection is needed after ME is wake up needs FFS. (vivo)

  - P6: RAN4 shall define the minimum required time for MR synchronisation and related side conditions (Nokia)

After UE entry the Case 1 (fully offloading case),  MR will go into ultra-deep sleep mode, so MR can’t receive paging immediately after LR receiving LP-WUS with paging indication.

RAN1 also discussed this issue, how does UE report the wake up delay capability, and how does network configure the offset value(s) between an LO and a reference PO/PF, following agreements have been achieved:

Agreement For the offset value(s) between an LO and a reference PO/PF, consider the following options: - Definition: The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay. - Option 1: gNB configures a single offset value. - If the gap between an LO and the PO associated with the offset is no less than the wake-up delay a UE supports, the UE monitors the PO associated with the offset after receiving a wake-up indication in a LP-WUS. - Otherwise, - Option 1-1: the UE follows the legacy paging monitoring procedure. - Option 1-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - Option 2: gNB configures one or multiple offset values. - For the same PO, each offset corresponds to a LO. - This does not preclude the possibility that the same LO may correspond to different POs with different offset values. - Option 2A: A UE does not expect that the gap between the LO associated with the largest offset and the corresponding PO is less than the wake-up delay the UE supports. The UE monitors the LO associated with one offset that has a gap between the LO and the corresponding PO no less than the wake-up delay. - This implies that the gNB needs to configure at least one offset value that is no less than the largest wake-up delay supported by the UEs. - FFS exactly how to choose the offset - Option 2B: - If the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay a UE supports, the UE monitors the LO associated with one offset that has a gap between the LO and the PO associated with the offset no less than the wake-up delay. - FFS exactly how to choose the offset - Otherwise, - Option 2B-1: the UE follows the legacy paging monitoring procedure. - Option 2B-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - FFS exactly how to choose the offset - FFS the UE shall monitor the LO associated with additional offset(s) Note: The PO mentioned above refers to legacy PO configured for the UE. Agreement Support 3 candidate values for the wake-up delay capability a UE reports, two values for ultra-deep sleep state and one value for deep sleep state. - FFS the values - Note: Ultra-deep sleep state and deep sleep state are mentioned above solely for the purpose of RAN1 discussions and it is not intended to be captured in specifications from RAN1 perspective Agreement from RAN1#120 For the offset value(s) between an LO and a reference PO/PF, at least a frame-level offset is provided. - The reference point (reference PO/PF) for the frame-level offset is the start of the PF, or the first PF of the PF(s) (if mapping of POs from multiple PFs to one LO is supported), associated with the LO. - FFS other offset value(s) to determine the MOs of the LO Agreement For UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - FFS: different sets of 3 candidate value for different SSB periodicities - Send an LS to RAN4 after the down-selection to confirm the number of SSBs. Final LS in R1-2501624. Agreement For the offset value(s) between an LO and a reference PO/PF, adopt Option 2B-1. - gNB can configure 1 or 2 offset values. - FFS whether gNB can configure 3 offset values - If multiple offset values are configured and if the gap between the LO associated with the largest offset value and the corresponding PO is no less than the wake-up delay a UE reports, the UE monitors the LO associated with the smallest offset value that has a gap between the LO and the PO no less than the wake-up delay. - Note: if a single offset value is configured, UE behaviour is according to Option 1-1. - All the UEs supporting LP-WUS for idle/inactive mode supports the configuration of 2 offset values (FFS: 3 values).

A LS also sent by RAN1 regarding the MR synchronization. We copy the information as follow:

In RAN1#118bis, it was agreed to support 3 candidate values for the wake-up delay capability a UE reports for LP-WUS operation in IDLE/INACTIVE mode. In addition, the following agreement was made in RAN1#120: Agreement For UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[70ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [70ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - FFS: different sets of 3 candidate value for different SSB periodicities Note that the definition of wake-up delay is the minimum gap time between LP-WUS reception and MR to start PDCCH monitoring. RAN1 respectfully asks RAN4 to check and confirm the number of SSBs currently assumed by RAN1 for MR synchronization for SSB periodicity of 20ms. RAN1 also respectfully asks RAN4 to confirm if it is reasonable to assume the same number of SSBs for MR synchronization for different SSB periodicities.

RAN1 has specified the candidate offset values and related UE behaviors, RAN4 doesn’t need to have the duplicated discussion.

- Observation 2: The wake up delay candidate offset values and related UE behaviors has been specified in RAN1.

As tasked by RAN1, RAN4 should further confirm the number of SSBs for MR synchronization. Currently, 3 samples are for deep sleep state, and 5 samples are for ultra-deep sleep state, with the assumption of 20ms SSB periodicity.

Based on our understanding, for deep sleep state, after UE woke up, 1 sample for coarse sync and 1-2 samples for fine sync is enough.  3 samples is reasonable and can be confirmed.

For ultra-deep sleep state, after UE woke up, 3 samples for coarse sync and 1-2 samples for fine sync is enough. Hence, 5 samples is reasonable and can be confirmed.

With existing SSB periodicity, which no more than 160ms, we think the above assumption is still valid, since we always assume that the sync can be maintained during 160ms.

- Proposal 8: For existing SSB periodicity, which no more than 160ms, 3 samples for sync after UE woke up from deep sleep state and 5 samples for sync after UE woke up from ultra-deep sleep state can be confirmed.

Issue 1-2-7: On LP-WUR cell selection evaluation requirements for case 1

- Proposals

  - P1: No need to define the LR-WUR based cell selection criterion evaluation in case 1. (Apple CMCC vivo Huawei MTK)

  - P2: Define LP-WUR based serving cell selection criterion S evaluation requirement in case 1. (CATT Ericsson)

    - P2-1：The existing MR based evaluation requirements can be the baseline (CATT)

UE derive the Srxlev and Squal in cell selection criterion evaluation, the results are used for following case:

- For initial cell selection, after a UE has switched on and a PLMN has been selected

- For R criterion checking, UE shall perform ranking of all cells that fulfil the cell selection criterion S

- For neighbour cell measurement checking, UE may not perform intra/inter-frequency measurements when Srxlev > Snon/IntraSearchP and Squal > Snon/IntraSearchQ

- In Case 1 (fully offloading case, no neighbour cell measurement), UE has already camped on a suitable cell, and network should guarantee that UE enters the Case 1 after Srxlev > SIntraSearchP and Squal > SIntraSearchQ. With such assumption, in case 1, UE don’t need to evaluate the S criterion anymore.

- Proposal 9: No need to define the LR-WUR based cell selection criterion evaluation in case 1

Issue 1-2-1: LR Accuracy requirements

- Proposals

  - P1: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement. (ZTE Apple xiaomi Ericsson Huawei CT vivo MTK)

  - P2: Option 1: define dedicated accuracy requirement in the performance section but no test case; Option 2: don’t define the accuracy requirement for LR-WUR based RRM measurement in Idle/inactive states, but use it as the simulation target when evaluate the measurement delay requirement simulation. (CMCC)

  - P3: To enable reliable return to MR from LR in a timely manner, it is necessary to have absolute measurement accuracy requirements defined for LR; RAN4 should develop LP-SS based and SSB based measurement accuracy requirements (Nokia)

Issue 1-2-8-1: Accuracy for normal or relaxed MR serving cell measurement

- Proposals

  - P1: No new accuracy requirements will be introduced for MR either normal or relaxed serving cell measurement,

    - no margin will be introduced for MR serving cell measurement in specs. (vivo)

    - same margins as existing MR neighbour cell measurement apply (Huawei)

  - P2: To enable reliable offloading from MR it is necessary to have absolute measurement accuracy requirements defined for MR. The absolute measurement accuracy already defined for idle mode can be re-used when defining idle mode measurements for LR offloading purposes (Nokia)

In legacy, for IDLE mode measurement without measurement report, no dedicated accuracy is defined since it can’t be tested anyway. The accuracy requirement is reflected as a margin when comparing with the R value among the target cells, however, when comparing with the network configured thresholds, such margin is not considered.

After introducing the LP-WUR, the MR and LR measurement result will also be used to determine whether to trigger the measurement offloading/relaxation, the accuracy is rather important among these procedure to guarantee reliable offloading/relaxation.

We are open to margin approach to reflect the measurement accuracy, one advantage is that the measurement accuracy will not impact the conclusion of whether related criteria is fulfilled or not, network could ignore the measurement accuracy factor when configure the threshold of offloading/relaxation criteria.

- Observation 1: In legacy, the measurement accuracy was not considered when comparing the measurement results with the network configured thresholds.

- Proposal 10: Open to further consider whether margin approach can be used to reflect the LR and MR serving cell measurement accuracy.

Issue 1-2-8-2: Accuracy for relaxed MR neighbour cell measurement

- Proposals

  - P1: For relaxed MR measurement, the legacy accuracy requirements for MR are re-used. (CMCC Huawei vivo)

Although the measurement delay requirement is relaxed, the accuracy shall be guaranteed for mobility performance. Therefore, the legacy accuracy requirements shall be used.

- Proposal 11: The legacy accuracy for relaxed MR neighbour cell measurement should be reused for Case#3 and Case#2.

  - Others

Issue 1-5-1: LR based RRM with EMR in IDLE/inactive mode

- Proposals

  - P1: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not. (Apple Nokia)

  - P2: Postpone the discussion on the EMR issue (CMCC vivo)

  - P3: RAN4 to discuss the UE’s behaviour of MR relaxation with EMR impact (Ericsson)

This issue is similar as the issue 1-1-10, that network configures one measurement which can only be performed by MR. Then when UE enters Case#1, whether the measurement need to be done by MR or not should be further investigated.

Similar approach can be followed as high priority measurement. As long as network configures the EMR measurement, UE shall perform the related measurement.

In the existing requirement, if Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, which condition can be fulfilled after UE entered Case#1, the UE shall search for inter-frequency layers configured for idle mode CA/DC measurements at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7. We think this legacy requirement can be the starting point.

- Proposal 12: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, take the legacy requirement as the starting point, that UE shall search for inter-frequency layers configured for idle mode CA/DC measurements at least every Thigher_priority_search.

- Conclusion

Based on the discussion above, the following observation and proposals are concluded.

- Proposal 1: The legacy cell re-selection measurement rules should also be valid for LP-WUR capable UE, including:

- If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may not perform intra-frequency measurements.

- If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ,  the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority.

- Proposal 2: Case#2 is a possible case, its existence depends on network configuration. No separate discussion is needed for Case#2.

- Proposal 3: In order to guarantee the power saving gain, the LP-WUS monitoring entry/exit threshold shall be equal to or higher than the threshold for RRM measurement relaxation (Case 3).

- Proposal 4: No RAN4 impact from Solution 3, close the issue.

- Proposal 5: As long as network configure higher priority layer, UE should monitor it for possible cell-re selection. The relaxed higher priority layer measurement requirement can be applied, i.e., per higher priority layer within 1h.

- Proposal 6: The UE shall filter the LP-SS-RSRP and LP-SS-RSRQ and LP-SSS-RSRP and LP-SSS-RSRQ measurements of the serving cell using at least 2 measurements.

- For LP-SS-RSRP and LP-SS-RSRQ measurement, no need to define filtering space.

- For LP-SSS-RSRP and LP-SSS-RSRQ measurement, within the set of measurements used for the filtering, at least two measurements shall be spaced by LO periodicity /2.

- Proposal 7: For MR evaluation requirement for LP-WUR monitoring/measurement, when UE is in legacy state, reuse Nserv; when UE is in Case#2 or Case#3, consider Nserv*16/2 as the starting point.

- Observation 2: The wake up delay candidate offset values and related UE behaviors has been specified in RAN1.

- Proposal 8: For existing SSB periodicity, which no more than 160ms, 3 samples for sync after UE woke up from deep sleep state and 5 samples for sync after UE woke up from ultra-deep sleep state can be confirmed.

- Proposal 9: No need to define the LR-WUR based cell selection criterion evaluation in case 1

- Observation 1: In legacy, the measurement accuracy was not considered when comparing the measurement results with the network configured thresholds.

- Proposal 10: Open to further consider whether margin approach can be used to reflect the LR and MR serving cell measurement accuracy.

- Proposal 11: The legacy accuracy for relaxed MR neighbour cell measurement should be reused for Case#3 and Case#2.

- Proposal 12: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, take the legacy requirement as the starting point, that UE shall search for inter-frequency layers configured for idle mode CA/DC measurements at least every Thigher_priority_search.

- Reference

- R4-2502600, WF on RRM requirements for NR_LPWUS, vivo

- R4-2500539, Topic summary for [113][226] NR_LPWUS, vivo