3GPP TSG-RAN WG4 Meeting # 114-bis                                                           R4-2503395

Wuhan, China, 7th –11th April, 2025

Agenda item:	7.12.1.2

Source: 	CMCC

Title: 	Discussion on UE RF requirements for ATG with UL-MIMO

Document for:	Discussion

# 1. Introduction

RAN#106 meeting approved revised WID on Enhancements for Air-to-ground network for NR in Rel-19 [1]. After discussions of several meetings, we have made good progress on ATG with UL-MIMO. This paper will focus on the remaining issues.

# 2. Discussion

2.1. ULFPTx mode for UL-MIMO

Issue 4-2: ULFPTx mode for UL-MIMO WF: - FFS

Uplink full power transmission is a method for UE transmission enhancement. Limited by power control, UE cannot reach full power transmission in some cases, and this will influence the uplink coverage and transmission rate. To ensure that the PA can reach the maximum output power for the power class, different ULFPTx modes are designed for UEs with different capabilities. Given the varying terminal implementations and differing requirements across regions, we believe that this feature should be retained. And also since ULFPTx mode has been supported for legacy systems, we think ULFPTx mode should be supported for ATG UL-MIMO.

Proposal 1: Supporting ULFPTx mode for ATG UL-MIMO will be beneficial for the implementation of different ATG UE.

The subsequent challenge is to determine how to incorporate this mode within our specification. From our perspective, Mode 1, Mode 2, and Mode Full correspond to UE capability levels 2 (non-full rated PAs), UE capability level 3 (partially full rated PAs), and UE capability level 4 (all full rated PAs), respectively. The specification does not currently define the output power per PA or per Tx port. When ATG UL-MIMO operates in the ULFPTx mode, it is essential to specify the maximum output power for ATG UEs in the section "UE Maximum Output Power for UL MIMO".

Proposal 2: ATG UE maximum output power should be clarified in the section “UE Maximum Output Power for UL MIMO".

2.2. Single antenna port requirement for UL-MIMO

Issue 4-1: whether preclude the single antenna port related requirement Agreement: - FFS

In a similar situation regarding whether to support ULFPTx mode for UL-MIMO, we have not yet reached a consensus on supporting a single antenna port. Some companies believe that the benefits of implementing these capabilities are minimal and that they are not widely used in legacy UEs. However, in our view, these concerns should not be used as reasons to exclude these capabilities, as they can make the ATG use scenarios more flexible. Moreover, no additional special requirements need to be specified for ATG UEs, and the existing requirements for TN UEs can be reused.

Proposal 3: Apply the single antenna port-related requirements for ATG UEs, and the legacy requirements for TN UEs could be reused.

2.3. Tx Diversity for ATG

Tx Diversity primary function is to enhance the reliability and coverage of the signal by transmitting it simultaneously through multiple antennas. This technique of transmit diversity can effectively counteract the fading phenomena in wireless channels and bolster the signal's resistance to interference. It could also significantly boost the system's performance and user experience.

Proposal 4: Tx Diversity should be supported for ATG.

# 3. Conclusions

In this contribution, UE RF requirements for ATG with UL-MIMO are discussed with the following proposals:

Proposal 1: Supporting ULFPTx mode for ATG UL-MIMO will be beneficial for the implementation of different ATG UE.

Proposal 2: ATG UE maximum output power should be clarified in the section “UE Maximum Output Power for UL MIMO".

Proposal 3: Apply the single antenna port-related requirements for ATG UEs, and the legacy requirements for TN UEs could be reused.

Proposal 4: Tx Diversity should be supported for ATG.

# 4. References

[1] RP-242385, Revised WID on Enhancements for Air-to-ground network for NR