3GPP TSG-RAN WG4 Meeting #114-bis	R4-2503423

# Wuhan , CN, 7th – 11st Apr, 2025

Title: 	Further discussion on less than 5MHz for NTN RRM requirement

Source: 	CATT

Agenda item:	7.10.3.4

Document for:	Discussion

- Introduction

In the last meeting, the WF on RRM requirements for NR_IoT_NTN_req_test_enh was approved and there still are a few issues left [1]. This document will further discuss the remaining RRM issues about less than 5MHz for NTN, and give our understanding and proposals.

- Discussion

CHO requirements

In RAN4#112-bis meeting, RAN4 discussed CHO delay and agreed to extend to [3 Trs] in CHO without L3 measurement requirement. For CHO delay with L3 measurement requirement, since it is different from without L3 measurement, and CHO delay with L3 measurement delay include an additional component Tmeasure compared with CHO delay without L3 measurement [2], so it need to be further discussed. In the last meeting, RAN4 further discussed CHO with L3 measurement requirement, and the agreement is copied as follows [1]:

RAN4#112-bis: Agreements: - For CHO with L3 measurement, when Tmeasure is extended with [3] for intra-frequency case, T∆ in Tinterrupt can be extended to [3] Trs.

For CHO with L3 measurement, since L3 measurement time needs to be considered in CHO, so we think that T∆ is related to Tmeasure. From the network perspective, our intention is just not to prolong the entire CHO delay for too long, as it is unnecessary. Therefore, what we don't expect is that the extensions of both T∆ and Tmeasure all follow legacy.

The agreement on measurement (SSB index detection requirements) is copied as follows [1]:

Agreements: The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to by [3 or 4] and [3] for intra-frequency and inter-frequency cases respectively. Notes: the final value can be confirmed in the next meeting and the corresponding value will be updated in the draftCR.

Therefore, according to the above, if the number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to by 4 and 3 for intra-frequency and inter-frequency cases respectively, the total CHO delay will be too long if the extension of T∆ for less than 5MHz is the same with TN HO, so we support that T∆ can be extended to 2 Trs. If the extension number of SSB samples of Tidentify_xxx_with_index can be reduced compared to the legacy, we can compromise to extend T∆ to 3 Trs.

Observation 1: For CHO with L3 measurement, since L3 measurement time needs to be considered in CHO, so we think that T∆ is related to Tmeasure.

Observation 2: From the network perspective, our intention is just not to prolong the entire CHO delay for too long, as it is unnecessary. What we don't expect is that the extensions of both T∆ and Tmeasure all follow legacy.

Proposal 1: If the number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to by 4 and 3 for intra-f and inter-f cases respectively, we support that T∆ can be extended to 2 Trs.

Proposal 2: If the extension number of SSB samples of Tidentify_xxx_with_index can be reduced compared to the legacy, we can compromise to extend T∆ to 3 Trs.

- Conclusion

This document discussed the issues for less than 5MHz for NR NTN RRM requirements and presented following observations and proposals:

Observation 1: For CHO with L3 measurement, since L3 measurement time needs to be considered in CHO, so we think that T∆ is related to Tmeasure.

Observation 2: From the network perspective, our intention is just not to prolong the entire CHO delay for too long, as it is unnecessary. What we don't expect is that the extensions of both T∆ and Tmeasure all follow legacy.

Proposal 1: If the number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) can be extended to by 4 and 3 for intra-f and inter-f cases respectively, we support that T∆ can be extended to 2 Trs.

Proposal 2: If the extension number of SSB samples of Tidentify_xxx_with_index can be reduced compared to the legacy, we can compromise to extend T∆ to 3 Trs.

- Reference

[1]     R4-2502589, WF on RRM requirements for [114][214] NR_IoT_NTN_req_test_enh, Moderator (Xiaomi)

[2]     3GPP TS 38.133, NR Requirements for support of radio resource management